Agenda item: 	10.5

Source: 	Qualcomm Incorporated, Fraunhofer HHI, InterDigital

Title: 	[8K_TV_5G] 8K Test Sequences

Document for	Discussion and Agreement

# Introduction

This document provides a summary of available 8K Test sequences as provided by Fraunhofer HHI. A concrete proposal for the 8K_TV_5G work item is provided in clause 4.

# HHI 8K Test Sequences

## 2.1	Generation

| Camera: | RED DSMC2 |
| --- | --- |
| Sensor: | Helium 8K S35 35.4 Megapixel CMOS Sensor 29.90 mm x 15.77 mm (Diagonal: 33.80 mm) |
| Lense: | Fujinon XK6x20 (20-120mm) |
| Recording resolution: | 7680x4320 @ 60fps |
| Recording format: | R3D raw (REDCODE 12:1) with RED Log3G10 log curve and REDWideGamutRGB color space |

## Sequences

| BodeMuseum | NeptuneFountain2 |
| --- | --- |
| NeptuneFountain3 | OberbaumSpree |
| QuadrigaTree | SubwayTree |
| TiergartenParkway |  |

## SDR Versions

| Resolution: |  |
| --- | --- |
| 8K | 7680x3840 |
| others | can be generated by downsampling |
| Framerate: | 60 fps |
| Number of Frames: | 600 (10s) |
| Bitdepth: | 10bit |
| Color format: | YCbCr |
| Chroma subsampling: | 4:2:0 |
| Color gamut: | BT.2020 |
| Transfer function: | BT.2020 |
| Sample range: | Standard/Limited/Video |

## HDR Versions

InterDigital is currently producing the HDR versions of the above sequences, i.e the versions will be available as well for 3GPP.

| Resolution: |  |
| --- | --- |
| 8K | 7680x3840 |
| others | can be generated by downsampling |
| Framerate: | 60 fps |
| Number of Frames: | 600 (10s) |
| Bitdepth: | 10bit |
| Color format: | YCbCr |
| Chroma subsampling: | 4:2:0 |
| Color gamut: | BT.2020 |
| Transfer function: | BT.2100 PQ or HLG |
| Sample range: | Standard/Limited/Video |

## License

Digital Video Sequence "QuadrigaTree"

Copyright (C) 2019 Fraunhofer HHI

Author: Björn Kowalewsky

7680x4320p, YCbCr 4:2:0, BT.2020, 10 bit, narrow range, 60 fps, 600frames.

COPYRIGHT AND LICENSE INFORMATION

November, 2021

Fraunhofer Heinrich Hertz Institute (HHI)

Video Coding & Analytics Department

Einsteinufer 37

10587 Berlin, Germany

The video sequences provided above and all intellectual property rights therein remain the property of Fraunhofer HHI. This video sequence is licensed under the Creative Commons Attribution-NonCommercial-NoDerivatives 4.0 International License. To view a copy of this license, visit http://creativecommons.org/licenses/by-nc-nd/4.0/

# Impact to TR 26.955

Based on the above the following is needed to be done for TR 26.955

- Add the reference sequences to https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/

- Annotate the sequences accordingly with the json

- Generate mp4 and png previews

- Add the 14 reference sequences to Annex C of TR 26.955

- Select the 14 reference sequences for generation of 8K HEVC Anchors

# Proposal

It is proposed to

- Adopt the 7 SDR sequences and the corresponding 7 HDR sequences in principle unless any specific issues are identified for the 8K Scenario

- Once the sequences are available, proceed with steps 1 -3 in clause 3.

Once agreed the corresponding pCR to TR 26.955 to address steps 4 and 5 will be generated even during SA4#116e.

It is further proposed to not wait for any additional 8K sequences for work item completion, the set is considered sufficient. However, if additional test material would be made available until the end of 2021, this would still be welcome.