3GPP TSG SA WG4#116e	S4-211475

E-meeting, 10th – 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.998 | CR |  | rev |  | Current version: | 1.0.3 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_5GSTAR] pCR on Spatial computing/cognition scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia Corporation, Samsung Electronics Co. Ltd., B<>Com, PEREY Research & Consulting, Orange, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GSTAR |  |  |  |  |  | Date: |  |  | 2021-11-04 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of required definitions and call flows to cover spatial computing and cognitive services scenario |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section 4.7 on Spatial mapping related data is added and section 6.4 on Cognitive Services is updated to include spatial computing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

====== 1st CHANGE ======

# 2	References

.

.

[Y1] Apple ARKit, ARWorldMap, https://developer.apple.com/documentation/arkit/arworldmap

[Y2] Apple ARKit, ARTrackable, https://developer.apple.com/documentation/arkit/artrackable

[Y3] Google ARCore, Trackable, https://developers.google.com/ar/reference/java/com/google/ar/core/Trackable

[Y4] Microsoft, Spatial Mapping, https://docs.microsoft.com/en-us/windows/mixed-reality/design/spatial-mapping

[Y5] ETSI GS ISG ARF 004-2 v1.1.1 (2021-08): “Augmented Reality Framework (ARF) Interoperability Requirements for AR components, systems and services Part 2: World Storage and AR Authoring functions”

====== 2nd CHANGE ======

====== 3rd CHANGE ======

### 4.4.7 Spatial Mapping

### Define why spatial mapping is needed

4.4.7.1 Visual Features and 3D Maps

Visual features are characteristics of a real world element that can be searched, recognized or tracked in keyframes captured from an AR device visual sensor as it moves in a real environment, using Simultaneous Localization And Mapping approach (SLAM). They are the overlapping points that are recognizable in multiple images of the real environment.  Visual features are extracted from keyframes from a single moving camera or multiple cameras in SLAM systems.

A 3D Map, sparse or dense point cloud, of the real world can be generated from key frames and their matched visual features. The keyframes must be attached to camera information defined in 4.4.3.2 to triangulate 3D points correctly from multiple cameras. This mapping process can be performed either at runtime or offline. A 3D map is then used at runtime to relocalize or register the AR device by matching the features extracted from the current image with the ones stored in the previously built 3D map.

4.4.7.2 Trackables

Trackables are elements of the real world of which features (visual or non-visual) are available and/or could be extracted. A 3D Map trackable, for instance, may define a floor or a wall in the real world consisting of several points with visual features. However, there are other types of trackables as well. For example:

- A controller with LEDs that can be tracked by an AR headset’s vision sensor. The feature in this case is the constellation of LEDs.

- A fiducial marker that is detected as a black and white pattern by an AR device vision sensor. The feature in this case is the black and white pattern.

- Hands visible through an AR headset’s vision sensor. The feature is a learnt model for hands.

All of the above examples give a position of the trackable in reference to the position of the AR headset. A list of different types of Trackables as defined by ETSI ARF can be found in [Y5]. Trackables are defined also for ARKit and ARCore [Y2][Y3].

4.4.7.3 Spatial Anchor

AR objects can be positioned in reference to the real world (e.g., placing a vase on a table) using spatial anchors. A spatial anchor provides a fixed position in the real world based on a common frame of reference that can be used by multiple AR devices. Spatial anchors can refer to trackables for accurate positioning relative to the physical space. Spatial anchors can also be used alone (not referring to trackable) if global coordinates are used. In this case, the anchors are treated as global anchors without trackable as they have global coordinates which positions can be determined..

A spatial anchor may have one or more AR objects linked to it.

4,4.7.4 XR Spatial Description

XR spatial description is a data structure describing the spatial organisation of the real world using anchors, trackables, camera parameters and visual features. It is a meta specification of the real environment including the information for the estimation of the position and orientation (pose estimation) of AR devices for the purpose of registration, tracking and positioning, and provides a coordinate reference system in relation to the real world.

ETSI ARF uses the term World Graph for XR spatial description. It defines the relative position of AR Assets, Trackables and World Anchors by 3D Transforms. A world graph is a kind of scene graph including trackables (embedding their features), and anchors representing the real world. Thus, the AR device can detect, recognize, and estimate its pose in relation to trackables thanks to their features, and based on the 3D transforms defined in the World graph, it can estimate its pose in relation to anchors (to which AR assets will be attached). [21].  Requirements for XR spatial description are also under consideration in MPEG for using a scene graph for real-world mapping possibly as a glTF extension.

XR spatial description data is downloaded and updated periodically from a World Map Server, which may be an AR/MR Application provider. An XR spatial description update can be done by sending updated visual features or keyframes attached to camera parameters from an AR device.

The size of the XR spatial description depends on several parameters, such as, size of the area covered by the XR application, number of supported viewpoints in the area, etc. The size may be from 10MB for a small room to several hundred MBs for a building. The frequency of updates may depend on the application..

The XR spatial description can be exchanged OTT or as a 5G service. For mission and time critical use cases it may be essential to define 5QI to ensure timely updates. This is left FFS.

ETSI ARF defines World Map as a scene graph where anchors are the nodes and trackables are leaves [21].  Requirements for XR spatial description are also under consideration in MPEG for using a scene graph for real-world mapping possibly as a glTF extension.

====== 4th CHANGE ======

## 6.4	5G  spatial computing/cognition service

### 6.4.1	Introduction

This clause introduces the case of cognitive immersive service. In this case, media and other interactions captured by the AR device are sent uplink to a spatial computing/cognitive server, which provides network based AR Function for spatial computing and semantical perception, respectively.

•	Spatial Computing functions: AR functions which process sensor data to generate information about the world 3D space surrounding the AR user. It includes functions such as AR device/object/user’s limb 6DoF localization, sparse or dense 3D mapping, semantic perception. Spatial computing functions require XR spatial description and may result in updates to the XR spatial description.

•	Semantic perception functions: Spatial Computing functions which process sensor data and/or spatial description of the real world (e.g. a 3D map) to generate semantical information about the world 3D space surrounding the AR user. It includes functions such as object/user’s limb or activity detection,  object/user’s limb or activity recognition, object/user’s limb segmentation.

The data sent by the AR device to the AR Functions may include video, visual features, depth maps, sensor data etc. The data is processed by the server and perception results or spatial mapping data is provided to the UE. The data sent by the AR devices is also used for updating the global XR spatial description of the real world maintained by the World Map server. .

### 6.4.2	Relevant use cases

The following use cases are relevant to this scenario.

-	UC#4: AR guided assistant at remote location (industrial services)

-	UE#5: Police Critical Mission with AR

-	UC#9: XR meeting

-	UC#10: AR animated avatar calls

-	UC#12: AR avatar multi-party calls

-	UE#14: AR Streaming with Localization Registry

-	UC#15: 5G Spatial Shared Data

-	UE#16: AR remote cooperation

-	UC#18: Streaming of volumetric video for glass-type MR devices

-	UC#19: AR Conferencing

-	UC#20: AR IoT control

-	UC#21: AR gaming

### 6.4.3	Architectures

#### 6.4.3.1	STAR-based

Figure 6.4.3.1-1 provides an architecture for immersive interactive media distribution using a STAR UE.

Figure 6.4.3.1-1: STAR-based 5G cognitive immersive service architecture

#### 6.4.3.2	EDGAR-based

Figure 6.4.3.2-1 provides an architecture for immersive cognitive media distribution using a EDGAR UE. In this case, similar as before, most of the rendering needs to be accomplished on the server.

Figure 6.4.3.2-1: EDGAR-based 5G cognitive immersive service architecture

### 6.4.4	Procedures and call flows

Figure 6.4.4.1-1 illustrates the procedure diagram for cognitive immersive services using a STAR-based and EDGAR-based UE.

Editor’s Note

- Similar to interactive case:

  - Bigger loop now includes cognitive data (UE sensor&media data)

  - Interactive processing changes to cognitive processing (it may not necessarily result in scene updates but also things like semantic perception results e.g. labels)

- Further work:

  - Match figure to interactive case and add descriptions

  - Possibly add EDGAR case

Figure 6.4.4.1-1 Example call flow for Spatial computing for STAR and EDGAR device types

The example call flow in Figure 6.4.4.1-1 applies to the streaming scenario. Spatial computing including semantic perception (shown in step 2, 3, 5, 6, 14 and 15) can apply also to use cases with XR conversational requirements.

Step 2 &3: An EDGAR type device requests edge computing, which would include spatial computing.

Steps 5&6 : From the anchor references defined in the scene graph, the UE will request an XR spatial description, which can be provided and stored by a World Map server, not shown in the call flow.

Steps 7-10: An AR Session is created and necessary media pipelines are created.

Step 11: Uplink transport sessions are created for sending sensor data from AR Runtime to the edge. Downlink sessions are established for transporting cognitive media and possibly XR spatial description.

Step 13: AR Runtime will capture images with its embedded vision sensors and will transmit images, or visual features extracted from images instead for the sake of privacy, or other sensor data to the Spatial Computing Server where they are processed. Images should be sent ideally at 60Hz for processing. For STAR type devices, the images are processed by the device AR Function.

Step 14-19:  When the AR function provides spatial computing it estimates from data received from the vision sensor the pose of the AR device. It uses the features associated with trackables available in the XR spatial description. The spatial computing AR functions may need XR spatial description provided and stored by the World Map server hosted by an AR/MR application provider and not shown in the call flow. The 3D transformations of anchors defined in the spatial description allow to position the XR Media Objects freely in a common reference coordinate system of the real world. The AR function can provide the AR device pose to the renderer functions (visual and audio) which will render the AR assets from a viewpoint corresponding to the real pose of the AR device. The resulting images and audio will be sent to the AR device display and speakers with a possible adaptation (ex: time warping, distortion to adapt to the lens system of the display). The process may generate updates to the spatial description which can be sent to the World Map Server.

Spatial computing functions providing semantic perception may result in cognitive immersive media (e.g. labels associated for a real world element) which is provided to the Media AS and delivered to the media client either as immersive media or pre-rendered media based on the device type.

### 6.4.5	Content formats and codecs

Based on the use cases, the following formats, codecs and packaging formats are of relevance for Media Streaming of AR:

- Scene Graph and Scene Description

- 2D Video Formats

- 3D Formats such as static and dynamic point clouds or meshes

- 2D Video Formats with depth

- Regular audio formats

- Several video decoding instances

- Decoding tools for such formats

- Encoding tools for 2D formats

- Low-latency downlink and uplink real-time streaming of the above media

- Uplink streaming of pose information

- Uplink streaming of media

### 6.4.6	Standardization areas

The list of potential standardization area that has been collected is provided in the following:

- XR spatial description format and transport