3GPP TSG-SA4 Meeting #116e	S4-211507

# Electronic meeting, Telco, Nov 10-19, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.114 | CR | xxxx | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add MMtel Call Setup Time |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM, Deutsche Telekom AG |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | EQoE_MTSI, TEI17 |  |  |  |  |  | Date: |  |  | 2021-11-04 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | With MMtel functionality like VoLTE/VoNR soon becoming the prevalent method for voice communication, the need for relevant MMtel service quality measurements and service quality assurance increases. There are already possibilities to do some call-quality-related measurements by using the existing QoE metrics specified in TS 26.114, but these currently only describe the quality during an established call. A critical metric which is missing is the call setup time, as seen from the viewpoint of the user who originated the call. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a Call Setup Time metric for mobile-originated calls. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MMtel service assurance is missing a critical metric. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.2.9 (new), 16.4.1, 16.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### ===================== Start of first change =====================

### 16.2.9	Call setup time

The call setup time (CST) is measured on SIP level for originating calls. It is defined as the time between the transmitted INVITE, and the reception of either "200 OK" or "180 RINGING" (whichever comes first).

The syntax for the metric "Call_Setup_Time" is defined in sub-clause 16.3.2.

The measured call setup time shall be stored in the variable CallSetupTime. The unit of this metrics is expressed in milliseconds. The variable is reported by the MTSI client as part of the QoE report (sub-clause 16.4).

NOTE:	The reason for also using "180 RINGING" as an end-of-call-setup criteria is to compensate for the physical answering delay caused by the called user. Thus the metric measures the fastest possible call setup time, i.e. as if the called user had answered directly.

### ===================== Start of next change =====================

### 16.4.1	XML schema for QoE report message

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="urn:3gpp:metadata:2008:MTSI:qoereport"

xmlns="urn:3gpp:metadata:2008:MTSI:qoereport"

elementFormDefault="qualified">

<xs:element name="QoeReport" type="QoeReportType"/>

<xs:complexType name="QoeReportType">

<xs:sequence>

<xs:element name="statisticalReport" type="starType" minOccurs="0"

maxOccurs="unbounded"/>

<xs:any namespace="##other" processContents="skip" minOccurs="0"

maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute processContents="skip"/>

</xs:complexType>

<xs:complexType name="starType">

<xs:sequence>

<xs:element name="mediaLevelQoeMetrics" type="mediaLevelQoeMetricsType" minOccurs="1"

maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="startTime" type="xs:unsignedLong" use="required"/>

<xs:attribute name="stopTime" type="xs:unsignedLong" use="required"/>

<xs:attribute name="callId" type="xs:string" use="required"/>

<xs:attribute name="clientId" type="xs:string" use="required"/>

<xs:attribute name="qoeReferenceId" type="xs:hexBinary" use="optional"/>

<xs:attribute name="recordingSessionId" type="xs:hexBinary" use="optional"/>

<xs:anyAttribute processContents="skip"/>

</xs:complexType>

<xs:complexType name="mediaLevelQoeMetricsType">

<xs:sequence>

<xs:any namespace="##other" processContents="skip" minOccurs="0"

maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="mediaId" type="xs:integer" use="required"/>

<xs:attribute name="totalCorruptionDuration" type="unsignedLongVectorType"
       	use="optional"/>

<xs:attribute name="numberOfCorruptionEvents" type="unsignedLongVectorType"
       	use="optional"/>

<xs:attribute name="corruptionAlternative" type="xs:string" use="optional"/>

<xs:attribute name="totalNumberofSuccessivePacketLoss" type="unsignedLongVectorType"

use="optional"/>

<xs:attribute name="numberOfSuccessiveLossEvents" type="unsignedLongVectorType" 
       	use="optional"/>

<xs:attribute name="numberOfReceivedPackets" type="unsignedLongVectorType" 
       	use="optional"/>

<xs:attribute name="framerate" type="doubleVectorType" use="optional"/>

<xs:attribute name="totalJitterDuration" type="doubleVectorType" use="optional"/>

<xs:attribute name="numberOfJitterEvents" type="unsignedLongVectorType"

use="optional"/>

<xs:attribute name="totalSyncLossDuration" type="doubleVectorType" use="optional"/>

<xs:attribute name="numberOfSyncLossEvents" type="unsignedLongVectorType"

use="optional"/>

<xs:attribute name="networkRTT" type="unsignedLongVectorType" use="optional"/>

<xs:attribute name="internalRTT" type="unsignedLongVectorType" use="optional"/>

<xs:attribute name="codecInfo" type="stringVectorType" use="optional"/>

<xs:attribute name="codecProfileLevel" type="stringVectorType" use="optional"/>

<xs:attribute name="codecImageSize" type="stringVectorType" use="optional"/>

<xs:attribute name="averageCodecBitrate" type="doubleVectorType" use="optional"/>

<xs:attribute name="callSetupTime" type="xs:unsignedLong" use="optional"/>

<xs:anyAttribute processContents="skip"/>

</xs:complexType>

<xs:simpleType name="doubleVectorType">

<xs:list itemType="xs:double"/>

</xs:simpleType>

<xs:simpleType name="stringVectorType">

<xs:list itemType="xs:string"/>

</xs:simpleType>

<xs:simpleType name="unsignedLongVectorType">

<xs:list itemType="xs:unsignedLong"/>

</xs:simpleType>

</xs:schema>

### 16.4.2	Example XML for QoE report message

Below is one example of QoE report message, in this example the measurement interval is 20 seconds, the reporting interval is 5 minutes, but the call ends after 55 seconds.

<?xml version="1.0" encoding="UTF-8"?>

<QoeReport xmlns="urn:3gpp:metadata:2008:MTSI:qoereport"

xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance"

xsi:schemaLocation="urn:3gpp:metadata:2008:MTSI:qoereport qoereport.xsd">

<statisticalReport

startTime="1219322514"

stopTime="1219322569"

clientId="clientID"

callId="callID">

qoeReferenceId="240F512A"

recordingSessionId="0001"

<mediaLevelQoeMetrics

mediaId="1234"

totalCorruptionDuration="480 0 120"

numberOfCorruptionEvents="5 0 2"

corruptionAlternative="a"

totalNumberofSuccessivePacketLoss="24 0 6"

numberOfSuccessiveLossEvents="5 0 2"

numberOfReceivedPackets="535 645 300"

framerate="50.0 49.2 50.0"

numberOfJitterEvents="0 1 0"

totalJitterDuration="0 0.346 0"

networkRTT="120 132 125"

internalRTT="20 24 20"

codecInfo="AMR-WB/16000/1 = ="

averageCodecBitRate="12.4 12.65 12.7"

callSetupTime="345"/>

<mediaLevelQoeMetrics

mediaId="1236"

totalCorruptionDuration="83 0 0"

numberOfCorruptionEvents="1 0 0"

corruptionAlternative="b"

totalNumberofSuccessivePacketLoss="3 0 0"

numberOfSuccessiveLossEvents="2 0 0"

numberOfReceivedPackets="297 300 225"

framerate="14.7 15.0 14.9"

numberOfJitterEvents="0 0 0"

totalJitterDuration="0 0 0"

numberOfSyncLossEvents="0 1 0"

totalSyncLossDuration="0 0.789 0"

networkRTT="220 232 215"

internalRTT="27 20 25"

codecInfo="H263-2000/90000 = ="

codecProfileLevel="profile=0;level=45 = ="

codecImageSize="176x144 = ="

averageCodecBitRate="124.5 128.0 115.1"

callSetupTime="345"/>

</statisticalReport>

</QoeReport>