3GPP TSG SA WG4#116e	S4-211356

E-meeting, 10th – 19th November 2021	revision of S4aV210770

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.116 | CR | draft | rev | 4 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | HEVC 8K Operation Point |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 8K_TV_5G |  |  |  |  |  | Date: |  |  | 03/11/2021 |
| Category: | B |  |  |  |  |  | Release: |  |  | 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | See work item |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Define new 8K TV operation point(s) for TV Video profiles with conforming bitstream requirement based on H.265/HEVC Main-10 Profile Main Tier Profile with the following constraints: - In addition to the luminance resolutions already included in TS 26.116 for UHD services, the specification is expected to support 16:9 aspect ratio and luminance resolution up to 7680x4320 pixels, including 5120 x 2880 pixels. - The specification is expected to enable an 8K operation point that is within the profile level constraints of H.265/HEVC Main-10 Profile, Main Tier and Level 6.1 decoding capabilities. - The specifications is expected to support conformance points with 10-bit BT.2020 non-constant luminance colorimetry with SDR, HDR PQ and HDR HLG. - Define the relevant ISO BMFF encapsulation, CMAF media profile and DASH signaling for the new 8K TV operation point. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work Item objectives not complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 2, 4.2, 4.3, 4.5.9 (new), 5.1.3, 5.11 (new), A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This revision takes into account the revisions done in CR |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

# 1	Scope

The present document specifies requirements and guidelines on video source formats (frame rate, resolution, aspect ratio, colorimetry, bit depth…) and encoding parameters (codec format, random access point period, SEI messages…) for different types of TV services, including linear TV, catch-up TV or on-demand services. A limited set of Operation Points (e.g. SDTV, HDTV, UHD, 8K UHD, …) are defined to provide confidence to content providers/broadcasters on the quality of experience offered by 3GPP services when used for TV-like distribution. Operation Points define format and encoding restrictions but may also be viewed as compatibility points for UEs.

In particular, the Operation Points defined in the present document may serve as the primary tested configurations for TV centric video distribution. The Operation Points are defined based on the analysis and findings in the technical report TR 26.949 [2].

In addition, in the context of DASH operations, not only the main distribution formats are defined, but also a subset of spatial and temporal resolutions. In order to minimize testing for seamless switching experience, suitable lower resolutions of distribution formats are defined. Furthermore, to compensate congestion situations, a minimum service quality is defined in order to provide service continuity.

=====  CHANGE  =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 26.949: "Video formats for 3GPP services".

[3]	Recommendation ITU-R BT.709-6 (06/2015): "Parameter values for the HDTV standards for production and international programme exchange".

[4]	Recommendation ITU-R BT.2020-2 (10/2015): "Parameter values for ultra-high definition television systems for production and international programme exchange".

[5]	Recommendation ITU-T H.264 (04/2017): "Advanced video coding for generic audiovisual services" | ISO/IEC 14496-10:2014: "Information technology – Coding of audio-visual objects – Part 10: Advanced Video Coding".

[6]	Recommendation ITU-T H.265 (11/2019): "High efficiency video coding" | ISO/IEC 23008-2:2020: "High Efficiency Coding and Media Delivery in Heterogeneous Environments – Part 2: High Efficiency Video Coding".

[7]	3GPP TS 26.244: "Transparent end-to-end packet switched streaming service (PSS); 3GPP file format (3GP)".

[8]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[9]	ISO/IEC 14496-15: 2019: "Information technology - Coding of audio-visual objects - Part 15: Carriage of network abstraction layer (NAL) unit structured video in ISO base media file format".

[10]	ISO/IEC 23091-2:2019, "Information technology — Coding-independent code points — Part 2: Video".

[11]	Recommendation ITU-R BT.2100-2 (07/2018): "Image parameter values for high dynamic range television for use in production and international programme exchange".

[12]	3GPP TS 26.511: "5G Media Streaming (5GMS); Profiles, codecs and formats".

[13]	ISO/IEC 23000-19:2021: " Information technology — Multimedia application format (MPEG-A) — Part 19: Common media application format (CMAF) for segmented media".

=====  CHANGE  =====

## 4.2	General requirements on video profile Operation Points

The following requirements apply to video profile Operation Points:

-	16:9 picture aspect ratio shall be used. 3GPP UEs with display aspect ratio different from 16:9 and supporting the TV services over 3GPP are, by default, assumed to display the video in letter-box or pillarbox modes, depending on the screen size and orientation.

-	Y'CbCr (non-constant luminance) as the Chroma Format should be used.

-	4:2:0 chroma sub-sampling shall be used.

-	The following spatial resolutions should be used for:

-	Operation Points (for video intended to be viewed in full-screen mode): 7680 × 4320, 5120 × 2880, 3840 × 2160, 1920 × 1080 and 1280 × 720.

-	Distribution formats: 7680 × 4320, 5120 × 2880, 3840 × 2160, 3200 × 1800, 2560 × 1440, 1920 × 1080, 1600 × 900, 1280 × 720, 960 × 540, 854 × 480, 640 × 360, 426 × 240.

NOTE 1:	Distribution formats within an Operation Point do not exceed the native resolution of the Operation Point, but they may be subsampled in order to optimize distribution or adapt to the viewing conditions.

-	The following frame rates should be used depending on the Operation Point: 24; 25; 30; 50 and 60Hz. The following fractional frame rates may be used: 24/1.001, 30/1.001, 60/1.001 (Hz). Frame rates are not associated to any particular spatial resolution.

-	The following colour space formats may be used depending on the Operation Point: ITU-R BT.709 [3] and ITU-R BT.2020 [4]. If no signal is provided for the colour space, BT.709 [3] should be assumed as default colour space. Receiving devices should support BT.2020 [4] signaling and provide an appropriate mapping of the signal to the supported colour space of the device. Colour spaces are not associated to any particular spatial resolution.

-	The following transfer characteristics may be used depending on the Operation Point: ITU-R BT.709 [3] and ITU-R BT.2020 [4] non-constant luminance transfer characteristics or the electro-optical transfer function as defined in Recommendation ITU-R BT.2100 [11], either for the Perceptual Quantization (PQ) system, or for the Hybrid Log Gamma (HLG) system.

NOTE 2:	Although ITU-R BT.2020 is originally only recommended for 2160p/4320p resolution, this 3GPP specification recommends that BT.2020 be supported irrespective of the resolution to keep the colour space consistent across resolutions.

-	The Random Access Point period shall be less than or equal to 5 seconds, should be less than or equal to 2 seconds and may be less than or equal to 0.5 second for H.264/AVC [5] and 1 second for H.265/HEVC [6] for specific service requirements such as fast channel change or fast access to the bitstream.

-	Bit depth:	Either 8 or 10 bits shall be used depending on the Operation Point.

Table 4.2-1 provides an overview of the Operation Points defined in the present document.

Table 4.2-1: TV over 3GPP services Video Profile Operation Points

| Operation Point name | Resolution format | Picture aspect ratio | Scan | Max. frame rate |  | Chroma format | Chroma sub-sampling | Bit depth | Colour space format | Transfer Characteristics |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H.264/AVC 720p HD | 1280 × 720 | 16:9 | Progressive | 30 | Y'CbCr |  | 4:2:0 | 8 | BT.709 [3] | BT.709 [3] |
| H.265/HEVC 720p HD | 1280 × 720 | 16:9 | Progressive | 30 | Y'CbCr |  | 4:2:0 | 8 | BT.709 [3] | BT.709 [3] |
| H.264/AVC Full HD | 1920 × 1080 | 16:9 | Progressive | 60 | Y'CbCr |  | 4:2:0 | 8 | BT.709 [3] | BT.709 [3] |
| H.265/HEVC Full HD | 1920 × 1080 | 16:9 | Progressive | 60 | Y'CbCr |  | 4:2:0 | 8; 10 | BT.709 [3]; BT.2020 [4] | BT.709 [3]; BT.2020 [4] |
| H.265/HEVC UHD | 3840 × 2160 | 16:9 | Progressive | 60 | Y'CbCr |  | 4:2:0 | 10 | BT.2020 [4] | BT.2020 [4] |
| H.265/HEVC Full HD HDR | 1920 x 1080 | 16:9 | Progressive | 60 | Y'CbCr |  | 4:2:0 | 10 | BT.2020 [4] | BT.2100 [11] PQ |
| H.265/HEVC UHD HDR | 3840 x 2160 | 16:9 | Progressive | 60 | Y'CbCr |  | 4:2:0 | 10 | BT.2020 [4] | BT.2100 [11] PQ |
| H.265/HEVC Full HD HDR HLG | 1920 x 1080 | 16:9 | Progressive | 60 | Y'CbCr |  | 4:2:0 | 10 | BT.2020 [4] | BT.2100 [11] HLG |
| H.265/HEVC UHD HDR HLG | 3840 x 2160 | 16:9 | Progressive | 60 | Y'CbCr |  | 4:2:0 | 10 | BT.2020 [4] | BT.2100 [11] HLG |
| H.265/HEVC 8K UHD | 7680 x 4320 | 16:9 | Progressive | 60 | Y'CbCr |  | 4:2:0 | 10 | BT.2020 [4] | BT.2020 [4] BT.2100 [11] PQ BT.2100 [11] HLG |

Operation Points are defined including the video codec format.

=====  CHANGE  =====

## 4.3	General Video codec requirements

The following video codecs and associated Profiles and Levels should be used:

-	H.264/AVC Progressive High Profile Level 3.1 [5] for 720p HD services

-	H.264/AVC Progressive High Profile Level 4.2 [5] for Full HD services

-	H.265/HEVC Main Profile Main Tier Level 3.1 [6] for 720p HD services

-	H.265/HEVC Main-10 Profile Main Tier Level 4.1 [6] for Full HD services

-	H.265/HEVC Main-10 Profile Main Tier Profile Level 5.1 [6] for UHD services

-	H.265/HEVC Main-10 Profile Main Tier Profile Level 4.1 [6] for Full HD HDR services

-	H.265/HEVC Main-10 Profile Main Tier Profile Level 5.1 [6] for UHD HDR services

-	H.265/HEVC Main-10 Profile Main Tier Profile Level 6.1 [6] for 8K UHD services

The Table 4.3-1 presents the mapping of the operation points with the codec type, profile and level.

Table 4.3-1: Video codec parameters

| Operation Point name | Resolution Format | Codec type, profile and level |
| --- | --- | --- |
| H.264/AVC 720p HD | 1280 × 720 | AVC/H.264 Progressive High Profile Level 3.1 |
| HEVC/H.265 720p HD | 1280 × 720 | HEVC/H.265 Main Profile Main Tier Level 3.1 |
| H.264/AVC Full HD | 1920 × 1080 | AVC/H.264 Progressive High Profile Level 4.2 |
| HEVC/H.265 Full HD | 1920 × 1080 | HEVC/H.265 Main-10 Profile Main Tier Level 4.1 |
| HEVC/H.265 UHD | 3840 × 2160 | HEVC/H.265 Main-10 Profile Main Tier Level 5.1 |
| HEVC/H.265 Full HD HDR | 1920 x 1080 | HEVC/H.265 Main-10 Profile Main Tier Level 4.1 |
| HEVC/H.265 UHD HDR | 3840 x 2160 | HEVC/H.265 Main-10 Profile Main Tier Level 5.1 |
| HEVC/H.265 Full HD HDR HLG | 1920 x 1080 | HEVC/H.265 Main-10 Profile Main Tier Level 4.1 |
| HEVC/H.265 UHD HDR HLG | 3840 x 2160 | HEVC/H.265 Main-10 Profile Main Tier Level 5.1 |
| HEVC/H.265 8K UHD | 7680 x 4320 | HEVC/H.265 Main-10 Profile Main Tier Level 6.1 |

=====  CHANGE  =====

### 4.5.9	H.265/HEVC 8K UHD Operation Point

#### 4.5.9.1	Introduction

An 8K UHD H.265/HEVC Operation Point is introduced that includes three options for the transfer characterics: SDR, HDR PQ and HDR HLG. It is considered that receivers that support 10-bit decoding are all capable of handling any of the three transfer characteristics.

The general requirements and recommendations for Bitstreams and Receivers for H.265/HEVC Operation Points in clause 4.5.1 as well as the additional restrictions documented in this clause 4.5.9 shall apply for the H.265/HEVC 8K UHD Operation Point.

#### 4.5.9.2	Profile, tier and level

A Bitstream conforming to the H.265/HEVC 8K UHD Operation Point shall comply with the following restrictions:

-	The general_profile_idc shall be set to 2 indicating the Main-10 profile.

-	The general_tier_flag shall be set to 0 indicating the Main tier.

-	The value of level_idc shall not be greater than 183 (corresponding to the Level 6.1) and should indicate the lowest level to which the Bitstream conforms.

#### 4.5.9.3	Bit depth

A Bitstream conforming to the H.265/HEVC 8K UHD Operation Point shall be encoded with 10 bits precision, i.e. the fields shall be set as follows

-	bit_depth_luma_minus8 = 2

-	bit_depth_chroma_minus8 = bit_depth_luma_minus8

Receivers conforming to the H.265/HEVC 8K UHD Operation Point shall support 10 bit precision.

#### 4.5.9.4	Spatial resolution

The spatial resolution of the distribution format shall be one of the following:

-	7680 × 4320,

-	5120 × 2880,

-	3840 × 2160,

-	3200 × 1800,

-	2560 × 1440,

-	1920 × 1080,

-	1600 × 900,

-	1280 × 720,

-	960 × 540,

-	854 × 480.

#### 4.5.9.5	Colour information and HDR transfer characteristics

A Bitstream conforming to the H.265/HEVC 8K UHD Operation Point shall comply with the following restrictions in the VUI:

-	colour_primaries shall be set to the value 9,

-	transfer_characteristics shall be set to one of the following values

-	14 for SDR,

-	16 for HDR PQ,

-	18 for HDR HLG,

-	matrix_coeffs shall be set to the value 9,

-	the chroma_loc_info_present_flag shall be equal to 1, and if set the chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field shall both be equal to 2.

For transfer_characteristics set to 16, it implies that Recommendation BT.2020 [4] colorimetry in non-constant luminance and Perceptual Quantization (PQ) electro-optical transfer function (EOTF) as defined in Recommendation ITU-R BT.2100 [11] are in use.

For transfer_characteristics set to 18, it implies that Recommendation BT.2020 [4] colorimetry in non-constant luminance and Hybrid Log Gamma (HLG) opto-electronic transfer function (OETF) as defined in Recommendation ITU-R BT.2100 [11] are in use.

#### 4.5.9.6	Frame rates

A Bitstream conforming to the H.265/HEVC 8K UHD Operation Point shall have one of the following frame rates: 24; 25; 30; 50; 60; 24/1.001; 30/1.001; 60/1.001 Hz.

The frame rate may be indicated in the VUI by setting vui_time_scale and vui_num_units_in_tick.

#### 4.5.9.7	SEI messages for metadata signalling

If HDR PQ is in use, then the same requirements and recommendations on SEI messages as defined in clause 4.5.6.7 apply.

#### 4.5.9.8	Receiver compatibility

Receivers conforming to the H.265/HEVC 8K UHD Operation Point shall support decoding and processing

- H.265/HEVC 720p HD,

- H.265/HEVC Full HD,

- H.265/HEVC UHD,

- H.265/HEVC Full HD HDR,

- H.265/HEVC UHD HDR,

- HEVC/H.265 Full HD HDR HLG,

- H.265/HEVC UHD HDR HLG, and

- H.265/HEVC 8K UHD Bitstreams.

Receivers conforming to the H.265/HEVC 8K UHD Operation Point should support processing the optional SEI messages defined in clause 4.5.9.7.

=====  CHANGE  =====

## 5.11	H.265/HEVC 8K UHD Operation Point

### 5.11.1	Operation Point Identifier

If all Representations in an Adaptation Set conform to the elementary stream constraints for the H.265/HEVC 8K UHD Operation Point as defined in clause 4.9.6 and the Adaptation Set conforms to the MPD signalling according to clause 5.11.2 and 5.11.4, and the Representations conform to the file format constraints in clause 5.8.3, then the @profiles parameter in the Adaptation Set may signal conformance to this Operation Point by using "urn:3GPP:video:op:h265-8K-UHD".

### 5.11.2	MPD Signalling

The requirements as defined in clause 5.1.1 shall apply. In addition, the conditions in 5.11.4 shall apply.

### 5.11.3	File Format Constraints

For Representations conforming to the H.265/HEVC 8K UHD Operation Point the following requirements apply.

The requirements as defined in clause 5.4.3 shall apply. The video track shall be encoded using the requirements and recommendations for H.265/HEVC 8K UHD Operation Point as defined in clause 4.9.6.

In addition, the Representation shall conform to an MPEG CMAF Track that conforms to the ‘c8k0' media profile as defined ISO/IEC 23000-19 [13].

If sample entry hvc1 is in use, then any possibly present Mastering display colour volume SEI message or any possibly present Content light level information SEI message shall be provided in the decoder configuration record and shall be constant for the entire file.

### 5.11.4	Adaptation Set Constraint

The requirements as defined in clause 5.1.3 shall apply. In addition, the following shall apply:

-	@maxWidth shall be set to 7680 and @maxHeight shall be set to 4320.

-	The @codecs parameter shall be set to either hvc1.2.4.L183.B0 or hev1.2.4.L183.B0.

-	@width and @height for Representations shall be set to one of the following pairs: (7680, 4320), (5120, 2880), (3840, 2160), (3200, 1800), (2560, 1440), (1920, 1080), (1600, 900), (1280, 720), (960, 540), or (854, 480).

-	@frameRate shall be set to one of the following values: "24", "25", "30", "50", "60", "24000/1001", "30000/1001" or "60000/1001".

-	The Colour Primaries, Transfer Characteristics and Matrix Coefficients shall be signalled to indicate ITU-R BT.2020, and BT.2100 PQ and BT.2100 HLG as defined in clause 5.1.3. In particular, the Essential Descriptors shall be present to signal BT.2020, and BT.2100 PQ and BT.2100 HLG as follows:

-	an Essential Descriptor shall be used to signal the value by setting the @schemeIdUri attribute to urn:mpeg:mpegB:cicp:MatrixCoefficients as defined ISO/IEC 23091-2 [10] and the @value attribute according to ISO/IEC 23091-2 [10]. The values shall match the values set in the VUI, i.e. the value is set to 9.

-	Essential Descriptors shall be used to signal the value by setting the @schemeIdUri attribute to urn:mpeg:mpegB:cicp:ColourPrimaries and urn:mpeg:mpegB:cicp:TransferCharacteristics, respectively, as defined ISO/IEC 23091-2 [10] and the @value attribute according to the "Colour primaries" Table and the "Transfer characteristics" Table of ISO/IEC 23091-2 [10], respectively. The values shall match the values set in the VUI, i.e.

-	urn:mpeg:mpegB:cicp:ColourPrimaries with value set to 9.

-	urn:mpeg:mpegB:cicp:TransferCharacteristics with value set to

-	14 if SDR is in use,

-	16 if HDR PQ is in use, or

-	18 if HDR HLG is in use.

-	The Essential Descriptors, and if applicable the Supplementary Descriptor, shall be on Adaptation Set level only, i.e all Representations in one Adaptation Set are required to have the same Matrix Coefficients, Color Primaries and Transfer Function.

-	If any Representation contains a mastering display colour volume SEI message or a content light level information SEI message, the same SEI message shall be present in all Representations in the Adaptation Set.

-	For hvc1 this implies that the SEI messages shall be provided in the decoder configuration record of every Representation.

-	For hev1, if any of such SEI message is carried inband within a segment/subsegment of any Representation of the Adaptation Set, it shall be carried with the first picture of that segment/subsegment in decode order in all Representations of this Adaptation Set.

=====  CHANGE  =====

# A.1	3GPP Registered URIs

The clause documents the registered URIs in this specification following the process in http://www.3gpp.org/specifications-groups/34-uniform-resource-name-urn-list

Table A-1 lists all registered URN values as well as

-	a brief description of its functionality;

-	a reference to the specification or other publicly available document (if any) containing the definition;

-	the name and email address of the person making the application; and

-	any supplementary information considered necessary to support the application.

Table A-1: 3GPP Registered URNs

| URN | Description | Reference | Contact | Remarks |
| --- | --- | --- | --- | --- |
| urn:3GPP:video:op:h264-720p-HD | DASH profile identifier for H.264/AVC 720p HD Operation Point | TS 26.116, clause 5.2.1 | Thomas Stockhammer tsto@qti.qualcomm.com | none |
| urn:3GPP:video:op:h264-Full-HD | DASH profile identifier for H.264/AVC Full HD Operation Point | TS 26.116, clause 5.3.1 | Thomas Stockhammer tsto@qti.qualcomm.com | none |
| urn:3GPP:video:op:h265-720p-HD | DASH profile identifier for H.265/HEVC 720p HD Operation Point | TS 26.116, clause 5.4.1 | Thomas Stockhammer tsto@qti.qualcomm.com | none |
| urn:3GPP:video:op:h265-Full-HD | DASH profile identifier for H.265/HEVC Full HD Operation Point | TS 26.116, clause 5.5.1 | Thomas Stockhammer tsto@qti.qualcomm.com | none |
| urn:3GPP:video:op:h265-UHD | DASH profile identifier for H.265/HEVC UHD Operation Point | TS 26.116, clause 5.6.1 | Thomas Stockhammer tsto@qti.qualcomm.com | none |
| urn:3GPP:video:op:h265-Full-HD-HDR | DASH profile identifier for H.265/HEVC Full HD HDR Operation Point | TS 26.116, clause 5.7.1 | Thomas Stockhammer tsto@qti.qualcomm.com | none |
| urn:3GPP:video:op:h265-UHD-HDR | DASH profile identifier for H.265/HEVC UHD HDR Operation Point | TS 26.116, clause 5.8.1 | Thomas Stockhammer tsto@qti.qualcomm.com | none |
| urn:3GPP:video:op:h265-Full-HD-HDR-HLG | DASH profile identifier for H.265/HEVC Full HD HDR HLG Operation Point | TS 26.116, clause 5.9.1 | Thomas Stockhammer tsto@qti.qualcomm.com | none |
| urn:3GPP:video:op:h265-UHD-HDR-HLG | DASH profile identifier for H.265/HEVC UHD HDR HLG Operation Point | TS 26.116, clause 5.10.1 | Thomas Stockhammer tsto@qti.qualcomm.com | none |
| urn:3GPP:video:op:h265-8K-UHD | DASH profile identifier for H.265/HEVC 8K UHD Operation Point | TS 26.116, clause 5.11.1 | Thomas Stockhammer tsto@qti.qualcomm.com | none |