Source:	MBS SWG Chairman

Title:	Report of SA4 MBS SWG AH Telco (10th March 2022)

Document for:	Approval

Agenda Item:	5.1

3GPP SA4 MBS SWG Telco (March 10, 2022)

# 1 Opening of the meeting and Approval of Agenda

Mr. Frederic Gabin (SA4 and MBS SWG chair, Dolby) opens the session on March 10th, 2022 at 15:00 CET.

Thorsten Lohmar and Julien Lemotheux are assigned as scribes.

Details of this meeting can be found here: https://portal.3gpp.org/Home.aspx#/meeting?MtgId=60312

The minutes are shared online: 3GPP SA4 MBS SWG Telco (March 10, 2022)

The following documents were registered:

| TDoc | Title | Source | Contact | Agenda item |
| --- | --- | --- | --- | --- |
| S4aI221312 | MBS SWG telco report 3rd March 2022 | MBS SWG acting Chair (Tencent) | Gilles Teniou | 3 |
| S4aI221317 | [EVEX] Data Access Profile update | BBC | Richard Bradbury | 4.4 |
| S4aI221318 | [EVEX] Additional Stage 2.5 and Stage 3 text for TS 26.532 | Qualcomm Incorporated, BBC | Charles Lo | 4.4 |
| S4aI221320 | Minor update of Data Collection and Reporting Configuration API (section 7.2) | Ericsson LM, BBC, Qualcomm Incorporated | Stefan Håkansson | 4.4 |
| S4aI221325 | [EVEX] Minor update of Data Collection and Reporting Configuration API | Ericsson LM, BBC, Qualcomm Incorporated | Stefan Håkansson | 4.4 |
| S4aI221323 | [5MBP3]: Stage 3 Proposal for Clause 5 (Service Announcement) | Ericsson LM | Thorsten Lohmar | 4.5 |
| S4aI221321 | [FS_NPN4AVProd]: Description of KI#4 (Standby and Program Cameras), incl solutions | Ericsson LM | Thorsten Lohmar | 4.8 |
| S4aI221319 | [FS_5G_MSE] From Ideas to a TR | Qualcomm CDMA Technologies | Thomas Stockhammer | 4.9 |
| S4aI221315 | 5G Media Streaming architecture domain model updates | BBC | Richard Bradbury | 4.12 |
| S4aI221311 | [5MBUSA] Additional stage 2 detail | BBC | Richard Bradbury | 4.14 |
| S4aI221313 | [5MBUSA] MBS User Service procedures | BBC | Richard Bradbury | 4.14 |
| S4aI221322 | [5MBUSA]: Various corrections around File Repair, Consumption Reporting and Reception Reporting | Ericsson LM | Thorsten Lohmar | 4.14 |

The following work items were assigned to this telco

| Edge Extensions to the 5G Media Streaming Architecture | 5GMS_EDGE | 920009 | S4-220327 |  |  | Mar 25, 2022, 15:00 – 17:00 CET, Host Qualcomm) Submission Deadline: Mar 24, 11:59 CET |
| --- | --- | --- | --- | --- | --- | --- |
| 5GMS AF Event Exposure | EVEX |  | S4-220244 | Date: March 3, 2022, 15:00 – 17:00 CET Host: Qualcomm Document Submission deadline: March 2, 2022, 12:00pm CET | Date: March 10, 2022, 15:00 – 17:00 CET Host: Qualcomm Document Submission deadline: March 9, 2022, 12:00pm CET | March 25, 2022 14:00 – 15:00 CET: joint telco with CT3 and SA2 on EVEX Submission Deadline: March 22, 2022 12:00 CET March 25, 2022 15:00 – 17:00 CET Submission Deadline: March 24, 2022 12:00 CET |
| 5G Multicast-Broadcast Protocols | 5MBP3 | 940008 | S4-220022 |  | Date: March 10, 2022, 15:00 – 17:00 CET Host: Qualcomm Document Submission deadline: March 9, 2022, 12:00pm CET | Mar 25, 2022, 15:00 – 17:00 CET, Host Qualcomm) Submission Deadline: Mar 24, 11:59 CET |
| Feasibility Study on Media Production over 5G NPN | FS_NPN4AVProd |  | S4-220280 | Date: March 3, 2022, 15:00 – 17:00 CET Host: Qualcomm Document Submission deadline: March 2, 2022, 12:00pm CET | Date: March 10, 2022, 15:00 – 17:00 CET Host: Qualcomm Document Submission deadline: March 9, 2022, 12:00pm CET | Date: March 25, 2022, 15:00 – 17:00 CET Host: Qualcomm Document Submission deadline: March 24, 2022, 12:00pm CET |
| Feasibility Study on 5G Media Service Enablers | FS_5G_MSE | 940010 | S4-220030 |  | Date: March 10, 2022, 15:00 – 17:00 CET Host: Qualcomm Document Submission deadline: March 9, 2022, 12:00pm CET | Date: March 25, 2022, 15:00 – 17:00 CET Host: Qualcomm Document Submission deadline: March 24, 2022, 12:00pm CET |

The following agenda was shared online

| MBS SWG Telco (10 March 2022) proposed Tdoc allocation | Gabin, Frederic | Thu, 10 Mar 2022 12:29:22 +0000 |
| --- | --- | --- |

3              Reports /Liaisons if any, postponed from the formal preceding SA4 meeting

1312 (for information)

4.4          5GMS AF Event Exposure (EVEX)

1314->1316->1318, 1317, 1320->1324->1325

4.5      	5G Multicast-Broadcast Protocols (5MBP3)

1323(r01)

4.8          Study on Media Production over 5G NPN (FS_NPN4AVProd)

1321(_BBC)

4.12    	TEI17 and any other Rel-17 matter

EVEX/5GMS_EDGE: 1315

4.14    	TEI17 and any other Rel-17 matter

5MBUSA: 1310->1313, 1311, 1322

4.9          Study on 5G Media Service Enablers (FS_5G_MSE)

1319

The agenda was approved.

# 2 	IPR and Anti-trust Reminder

Available in :  S4-201473

# 3	Reports/Liaisons

| S4aI221312 | MBS SWG telco report 3rd March 2022 | MBS SWG acting Chair (Tencent) | Gilles Teniou |
| --- | --- | --- | --- |

was noted.

# 4 List of Work Items for submission of Contributions in the current meeting

## 4.0	General

Agenda items 4.1 - 4.3, 4.7, 4.9 -  4.11 and 4.13 are not subject of this telco.

## 4.4 5GMS AF Event Exposure (EVEX)

| S4aI221317 | [EVEX] Data Access Profile update | BBC | Richard Bradbury |
| --- | --- | --- | --- |

Revisions

- none

Presenter: Richard Bradbury (BBC)

Discussion:

- This is adding existing text from the document into a figure.

- Is this a pCR or dCR? The cover page says dCR, but doc is still under working group control.

Decision:

- Agreed with an updated cover page.

S4aI221317 is agreed.

| S4aI221318 | [EVEX] Additional Stage 2.5 and Stage 3 text for TS 26.532 | Qualcomm Incorporated, BBC | Charles Lo |
| --- | --- | --- | --- |

Revisions

- none

Presenter: Charles Lo (Qualcomm)

Discussion:

- Richard: The applicationId may not be updated during the session. The exposure filter should be updatable. When something should be modified, then better create a new provisioning session.

- Richard: same logic should also apply for allowedExposureFilter and reportedParameters

- Charles: would tend to agree, will think about it some more and likely revise document accordingly

- Richard: this also applies to your propose change to clause 4.2.3.2.4 Update Data Reporting Provisioning Session properties - it should be disallowed.

Decision:

- Will be revised.

S4aI221318 is noted.

| S4aI221325 | [EVEX] Minor update of Data Collection and Reporting Configuration API | Ericsson LM, BBC, Qualcomm Incorporated | Stefan Håkansson |
| --- | --- | --- | --- |

Revisions

- none

Presenter: Gunnar Heikkilä (Ericsson)

Discussion:

- Gunnar: First row of change (removal of clause 7.2.2.3.3) was a mistake, it should still be there.

- Richard - as the client, we don’t know what the sessionId will be in Table 7.2.3.2.1. Is this read-only?

- The server should set this sessionId value. Need to check whether we have a convention about a field that is writable until it’s set, and THEN is read-only.

- Thorsten: C:RO convention is also a bit weird since Read operation is by definition

Decision:

- Agreed

S4aI221325 is agreed.

## 4.5 5G Multicast-Broadcast Protocols (5MBP3)

| S4aI221323 | [5MBP3]: Stage 3 Proposal for Clause 5 (Service Announcement) | Ericsson LM | Thorsten Lohmar |
| --- | --- | --- | --- |

Revisions

- https://www.3gpp.org/ftp/tsg_sa/WG4_CODEC/3GPP_SA4_AHOC_MTGs/SA4_MBS/Inbox/Drafts/S4aI221323r01.docx

Presenter: Thorsten Lohmar (Ericsson)

Discussion:

- Richard - you’re trying to describe things in a technology-neutral way, but some technologies distinguish between what’s in separate documents, and others don’t. How much consistency are you trying to maintain? If you are willing to change a name, you have already broken the existing and outdated schema, and you can simplify things a lot.

- Thorstein - maybe ignore how the information is structured on the client for now?

- Thomas - all of this is good discussion. We should also understand where the information is exposed, and on which interfaces. Just put “static resources” in for now. The URL defines what that is, and you don’t have to say that in this specification. Take information, put it in the cache, the application then has access to it.

- Richard - is that the Media Entry Point? The examples I’m thinking of all have one root entry point. What has more than one?

- Thomas - master and supplementary, as one example.

- Charles - we’re unnecessarily restricting our focus - streaming, or media, or …

- Thorstein - is there an array of possibilities?

- Richard - app service element points at the URL, maybe packaged with the bundle or separate, but you could say what the content type is before you retrieve the resource.

Decision:

- Will revise (Thorstein will provide number here)

- Revised to S4aI221328

S4aI221323 is revised to S4aI221328

## 4.8 Study on Media Production over 5G NPN (FS_NPN4AVProd)

| S4aI221321 | [FS_NPN4AVProd]: Description of KI#4 (Standby and Program Cameras), incl solutions | Ericsson LM | Thorsten Lohmar |
| --- | --- | --- | --- |

Revisions

- https://www.3gpp.org/ftp/tsg_sa/WG4_CODEC/3GPP_SA4_AHOC_MTGs/SA4_MBS/Inbox/Drafts/S4aI221321_BBC.docx

Presenter: Richard Bradbury (BBC)

Discussion:

- Thorstein has an r1, but hasn’t uploaded it yet.

- Trying to complete key issues in the document.

- Showing mixer between multiple cameras and program feed.

- Have two solutions, need to add details to solutions.

- Modify QoS rule or install multiple QoS rules - camera would change the traffic marking, and that would match a different filter set.

- Will work to get additional input

- Ed - how do you depict cameras with different bitrates? 1K or 4K would have the same marking but different bitrates.

- Thorstein - was thinking about switching between active and standby cameras, have not been thinking about this.

- Richards - high-speed cameras for slow-motion playback are post-processing - we just need to make sure that we can get the stream into post-processing. You may want inactive cameras to still be high quality, because the active camera might not be able to record the goal. Worried that the first solution might be too slow - want to switch on a single frame.

Decision:

- Will produce a revision.

- Revised to S4aI221329

S4aI221321 is revised to S4aI221329

## 4.9 Study on 5G Media Service Enablers (FS_5G_MSE)

| S4aI221319 | [FS_5G_MSE] From Ideas to a TR | Qualcomm CDMA Technologies | Thomas Stockhammer |
| --- | --- | --- | --- |

Revisions

- none

Presenter: Thomas Stockhammer (Qualcomm)

Discussion:

- Agreed on outline at last meeting,

- (We lost Thomas, with three minutes to go)

- No comments on the call.

Decision:

- Postponed to next call.

S4aI221319 is postponed.

## 4.12 TEI17 and any other Rel-17 matter

### 4.12.1 EVEX/5GMS_EDGE:

| S4aI221315 | 5G Media Streaming architecture domain model updates | BBC | Richard Bradbury |
| --- | --- | --- | --- |

Revisions

- none

Presenter: Richard Bradbury (BBC)

Discussion:

- (Imed isn’t available this week - Richard will present, but recognize that he’s not available)

- We have items in Rel-17 that affect 26.501, and 5GMS_EDGE is one of those items (along with EVEX).

- This is a discussion paper for now.

- Fredeic will send comments.

Decision:

- Will postpone until the call on March 25th.

S4aI221315 is postponed.

### 4.12.2	5MBUSA

| S4aI221311 | [5MBUSA] Additional stage 2 detail | BBC | Richard Bradbury |
| --- | --- | --- | --- |

Revisions

- none

Presenter: Richard Bradbury (BBC)

Discussion:

- Adding detail to help the CT group understand what to do with the spec.

- How many distinct distribution methods do we have? Is four modes, one for each bullet, correct?

- Thoms - I might ingest a software update once and distribute it many times. We still have a mode where ingest is completely unrelated to distribution - don’t need to tie them together.

- Richard - what are the things that need to be known at provisioning time? If you have a private ingest, don't you need to be able to configure that? We need to tell the system what to do, even if this never reaches the client. Could be a carousel, even with one object and no versioning.

- Richard - with the pull mode, you can check whether a file has changed, but with the push mode, that doesn’t happen.

- Whatever we do, we have to tell CT enough for them to understand what to do.

- Charles - need to check how much we tie ingest to distribution.

- Richard - the existing text may be why I was assuming a tight linkage - may need to treat ingest and distribution as orthogonal, but need to explain ingest someplace.

Decision:

- postponed

S4aI221311 is postponed.

| S4aI221313 | [5MBUSA] MBS User Service procedures | BBC | Richard Bradbury |
| --- | --- | --- | --- |

Revisions

- none

Presenter: Richard Bradbury (BBC)

Discussion:

- Is the MBSF client the MBSTF client? Yes, but it’s not in the diagram.

- But it looks like the diagram doesn’t separate MBSF and MBSFT clients. Would be good to separate the two.

- Thorstein - merging everything into one data model hides that the data model is used to create the service announcement file. Would be good to check with Stage 3 and see what’s needed to activate the session.

- Richard - we need to return the service announcement in one use case - that’s what I’m trying to show here.

- We aren’t doing the OpenAPI for this, but I hope the OpenAPI does show this - maybe we can persuade the CT group to use our data structure.

- Thorstein - would be good to have one group doing the service model.

- Richard - but that’s not going to happen by itself.

- Frederic - specify it here and tell the CT group that historically, things like this have been specified in SA4.

- Richard - splitting the monolithic announcement into two layers (no matter which box it’s in).

- Richard - Got excellent feedback from Charles, Thorstein, and Qi, and have made changes to reflect feedback.

- Thorstein needs to think about this more.

Decision:

- Agree as a baseline for further work? We can’t send anything to to SA before SA4#119e anyway.

- Will note this and expect a revision.as 1316.

S4aI221313 is noted.

| S4aI221322 | [5MBUSA]: Various corrections around File Repair, Consumption Reporting and Reception Reporting | Ericsson LM | Thorsten Lohmar |
| --- | --- | --- | --- |

Revisions

- none

Presenter: Thorsten Lohmar (Ericsson)

Discussion:

- We found inconsistencies that needed to be fixed.

- This is additional detail, beyond what SA2 needs in the architecture.

- Need to make boxes in diagrams and clause headings consistent, so people can tie the two together.

- Charles - We’re adding a new term (“MBS User Services”) on the fly here - not sure that’s appropriate.

- Thorsten - need to add a new definition

- Thomas - I think the User Service only exists in the MBS AF

- Richard - support the proposal to change the term “file repair” to “object repair”

- Thomas - would like to make sure we have entities for every clause we have in text.

- May need distinguish between consumption reporting and metrics reporting

- Thomas - is “metrics” something that happens during the session in real time, like an RTCP message? No, this is about providing higher-level metrics used in post-processing.

Decision:

- Will provide a revision, just wanted comments for now

- Revised to S4aI221327

S4aI221322 is revised to S4aI221327.

# 5   	Review of the future work plan

| Edge Extensions to the 5G Media Streaming Architecture | 5GMS_EDGE | 920009 | S4-220327 |  |  | Mar 25, 2022, 15:00 – 17:00 CET, Host Qualcomm) Submission Deadline: Mar 24, 11:59 CET |
| --- | --- | --- | --- | --- | --- | --- |
| 5GMS AF Event Exposure | EVEX |  | S4-220244 | Date: March 3, 2022, 15:00 – 17:00 CET Host: Qualcomm Document Submission deadline: March 2, 2022, 12:00pm CET | Date: March 10, 2022, 15:00 – 17:00 CET Host: Qualcomm Document Submission deadline: March 9, 2022, 12:00pm CET | March 25, 2022 14:00 – 15:00 CET: joint telco with CT3 and SA2 on EVEX Submission Deadline: March 23, 2022 12:00 CET March 24, 2022 15:00 – 17:00 CET Submission Deadline: March 24, 2022 12:00 CET |
| 5G Multicast-Broadcast Protocols | 5MBP3 | 940008 | S4-220022 |  | Date: March 10, 2022, 15:00 – 17:00 CET Host: Qualcomm Document Submission deadline: March 9, 2022, 12:00pm CET | Mar 25, 2022, 15:00 – 17:00 CET, Host Qualcomm) Submission Deadline: Mar 24, 11:59 CET |
| Feasibility Study on Media Production over 5G NPN | FS_NPN4AVProd |  | S4-220280 | Date: March 3, 2022, 15:00 – 17:00 CET Host: Qualcomm Document Submission deadline: March 2, 2022, 12:00pm CET | Date: March 10, 2022, 15:00 – 17:00 CET Host: Qualcomm Document Submission deadline: March 9, 2022, 12:00pm CET | Date: March 25, 2022, 15:00 – 17:00 CET Host: Qualcomm Document Submission deadline: March 24, 2022, 12:00pm CET |
| Feasibility Study on 5G Media Service Enablers | FS_5G_MSE | 940010 | S4-220030 |  | Date: March 10, 2022, 15:00 – 17:00 CET Host: Qualcomm Document Submission deadline: March 9, 2022, 12:00pm CET | Date: March 25, 2022, 15:00 – 17:00 CET Host: Qualcomm Document Submission deadline: March 24, 2022, 12:00pm CET |

# 6	Close of the session

The meeting was closed at 16:13 CET.

# 7	Attendees

China Unicom-Shuai Gao (来宾)

Thomas Stockhammer

Thomas Stockhammer

Gunnar Heikkilä

LGE - Woosuk Kwon (게스트)

Szucs, Paul

Gibellino Diego

Ed O'Leary

Sungryeul Rhyu - Samsung (Guest)

He, Shane (Nokia - FR/Paris-Saclay)

Gabin, Frederic (Dolby, SA4 MBS SWG chair)

Thorsten Lohmar

Charles Lo

LEMOTHEUX Julien INNOV/IT-S

Qi Pan Huawei (来宾)

Loic Fontaine

Emmanuel Thomas

Richard Bradbury

Stephane Onno

Dawkins Spencer

Prakash Reddy Kolan

Xin Wang

TELUS - Peng Tan

Qi Pan Huawei (来宾)

LGE - Woosuk Kwon (게스트)

Huan-yu Su (Huawei)

Gilles

Nokia - Saba

M. Frédéric Gabin 	frederic.gabin@dolby.com	+33 6 78 44 85 75