3GPP TSG-WG SA4 Meeting #118E e-meeting 	S4-220425

Elbonia, April 6th – 14th, 2022

Source:	Huawei, HiSilicon

Title:	discussion on use cases of new event type for exposure in 5GMS

Document for:	Discussion and Approval

Agenda Item:	8.6

Work Item / Release:	EVEX / Rel-17

Abstract: Discussion on use cases for exposure of new 5GMS event types, such as invocation of dynamic policy invocation and network assistance.

# 1. Introduction

In the previous discussion, the invocations of downlink dynamic policies, the invocations the AF-based UL/DL Network Assistance and the DL media streaming access activity in 5GMSd AS are regarded as the new exposed events. However, the use cases are not clear that which entity does need such events exposed by DC-AF. This paper intends to discussion the use cases for those newly defined events from SA4 perspective.

# 2. Discussion

In TS 26531, there is an EN in clause 4.2 that the additional event types may also be deemed suitable for NWDAF, based on further discussion with SA2.

Editor’s Note: Currently-defined 3GPP event types are specified in TS 23.502 and TS 23.288. Additional types of UE application level information available at the AF – for example content hosting logs, QoS and charging policy modification, network assistance and consumption reports as defined in TS 26.501 and TS 26.512 for 5G Media Streaming may also be deemed suitable as event exposure services for subscription by the NWDAF in performing related data analytics, based on further discussion with SA2.

As we know, in SA2 TS 23.288, before determining the potential inputs to NWDAF, the use case that how NWDAF can obtain and make use of the input need to be firstly studied. However, the SA2 R17 eNA topic is already completed and there are no related objectives in SA2 R18 eNA_Ph3 study item. There are no time window or topic to ask SA2 for new event types evaluation.

Besides, the LS S2-2201528 from SA2 also mentions that:

the 5GMS events beyond what specified so far in TS 23.288 (Rel-17 eNA) is not in the eNA scope of Rel-17 and requires potentially alignment CR(s) from another study (e.g. EVEX) for any stage 2 Spec changes at SA WG2 level.

Therefore, it is essential to study the use case about how to make use of such these new added event types from SA4 perspective.

For DL media streaming access activity in 5GMSd AS, this event type has been discussed before, which can be exposed to the Application Provider in S4-210964.

# 3. Use cases for newly added 5GMS event type

-	New exposure event type for invocation of downlink dynamic policies.

The Media Session Handler may invoke the dynamic policy API to 5GMS AF via M5d to request a specific policy and charging treatment to be applied to a particular application data flow of the download streaming. Afterwards, the 5GMSd AF invokes appropriate APIs to 5GC, e.g. Npcf_PolicyAuthorization and Nnef_AFSessionWithQoS, for the dynamic policy change. Then 5GMSd AF can also obtain the status information (policy accepted, rejected, etc) and information on policy enforcement such as the enforcement method and enforcement bit rate.

After recording the invocation of the dynamic policies, the 5GMSd AF may report the records to its subordinate Data Collection for further exposure. The new event for invocation of dynamic polices may include the time when MSH invokes the dynamic function in 5GMSd AF, the requested policy template, the status information, etc. The 5GMS Application Provider may subscribe this event to the Data Collection AF. Based on the details about the invocations for dynamic policies, the 5GMS Application Provider may better analyse the network quality provided by the MNO, the dynamic network requirements for this media streaming service. Then the 5GMS Application Provider may do the corresponding optimizations, e.g. provisioning more appropriate policy templates, negotiating more suitable Service Level Agreement (SLA) with the MNO.

-	New exposure event type for invocation of AF-based UL/DL network assistance.

The Network Assistance (NA) feature enables a UE to receive a bit rate recommendation from the 5GMS AF that provides the NA server function. The 5GMS AF may use the Npcf_PolicyAuthorization notification or Nnef_Monitoring Event procedure to receive QoS changes, e.g. estimation of throughput, recommendation of a bit rate. The 5GMS AF receives these policy change notifications asynchronously.

After recording the invocation of AF-based UL/DL network assistance, the 5GMS AF can report it to its subordinate Data Collection AF. And the Data Collection AF may expose this to the Event Consumer AF within the 5GMS Application Provider for network status aware optimization, e.g. fast congestion window adjustment.

-	New exposure event type for invocation of DL media streaming access activity.

In 5G Media Streaming, the 5GMS AS acts as a CDN server (e.g., edge server) in the hosting and delivery of streaming media content (i.e., of ingested/egested content in downlink/uplink streaming). The corresponding media streaming access activities, i.e., CDN access logs, available at the 5GMS AS can be forwarded to the 5GMS AF for subsequent event exposure to consumer entities such as the Application (Service) Provider.

For example, with the streaming activities exposed from Data Collection AF, e.g. the visited number of users and the access history for different contents, the 5GMS Application Provider may make use of such information for better CDN content distribution.

# 4. Conclusion and proposal

It is proposed to take the discussion into account and agree clause 3 as the informative Annex of TS 26.501.