Source:	Ericsson LM

Title:	IVAS-7a – Updated audio format designators and configurations

Document for:	Discussion and Agreement

Agenda Item:	7.5

# Introduction

At SA4#117-e, IVAS-7a [1] was updated to include draft notations and definitions for the audio formats currently being required to support by the IVAS design constraints in IVAS-4 [2].

This contribution contains an updated proposal based on the discussion at the previous meeting.

# Proposal

The notations proposed are being inspired by corresponding notations used earlier in VRStream, back then for naming files of individual channels [3]. The proposal for IVAS is however to use sample-wise interleaved audio files. Nevertheless, notations for the individual channels are needed, at least for definition of channel configurations, including channel ordering.

Further, even though the reasoning for the proposal in [4] was to clearly distinguish left/right, up/down directions, there are also benefits in using the mathematical notations with a sign representing the direction, especially if angles are used in processing. Not specifying the positive direction might make the processing more prone to bugs in the translation from L/R, U/D to signed angles. Therefore, it is proposed to specify the positive directions of the spherical coordinate system and adapt the notations correspondingly.

In addition, [4] did not include any proposal for the nominal angles to use for the various multi-channel configurations for IVAS. In relation to such nominal angles, there are typically tolerances for the loudspeaker positions. [5] specifies such tolerances, and it is proposed to adopt these for IVAS, to be documented either in the processing plan or the test plan [6]. Nominal loudspeaker positions are proposed based on the channel configurations that, e.g., have been used for MPEG-H 3D Audio [7], which also includes the channel ordering.

A detailed proposal is outlined below, as an updated to the current draft IVAS-7a [1].

# References

- Pdoc IVAS-7a: Processing plan for selection phase, v.0.3.0

- Pdoc IVAS-4: IVAS Design Constraints, v.0.3.0

- S4-AHVIC-139: Revision of DRAFT VRStream-2 - Submission Process for VRStream Audio Profiles, v.0.3

- S4-220164: IVAS-7a – Audio format and designators, Ericsson LM

- Recommendation ITU-R BS.2051-2

- Pdoc IVAS-8a: Test Plan for Selection Phase, v.0.3.0

- ISO/IEC 23008-3:2019

## Audio format

The format of input/output audio files is headerless PCM with samples stored in 16-bit 2’s complement little endian format. For multi-track audio, the audio tracks are sample-wise interleaved in the order specified by the corresponding configuration in Table 5.

## Audio track designators

Audio tracks are designated according to Table 3 with further specification according to Table 4.

Table 3: Track designators

| Track designator | Definition |
| --- | --- |
| M | Mono channel |
| L | Left channel |
| R | Right channel |
| CH_A<a>_E<e> | Channel at nominal azimuth angle <a> and nominal elevation angle <e> |
| LFE | LFE channel |
| A_<n> | Ambisonics components with ACNindex <n>-1 |
| O_<n> | Object ID <n> |

Table 4: Track specification

| Specification variable | Definition |
| --- | --- |
| <a> | Nominal azimuth angle in range: ±180 degrees Represented by three-digit number always including a sign (padded from the left with zeros if necessary). Positive values indicate positions left of the frontal direction. The sign assigned to azimuth value zero shall be +. |
| <e> | Nominal elevation angle in range: ± 90 degrees Represented by two-digit number always including a sign (padded from the left with zeros if necessary). Positive values indicate positions above the horizontal plane. The sign assigned to elevation value zero shall be +. |
| <n> | Index: 1, 2, … |

## Audio track configurations

Input/output audio shall follow configurations as specified in Table 5. Ambisonics components follow the ACN ordering where ACNindex = n2+n+m for real-valued spherical harmonics components of order n and degree m=0, 1, … , where m=[-n,…,n].

Table 5: Audio track configurations

| Audio format | Number of tracks | Index | Configuration (incl. ordering) | Azimuth Range | Elevation Range |
| --- | --- | --- | --- | --- | --- |
| Mono | 1 |  | M | - | - |
| Stereo/Binaural | 2 |  | L, R | - | - |
| Multi-channel 5.1 | 6 | 1 | CH_A+030_E+00 | +30 | 0 |
|  |  | 2 | CH_A-030_E+00 | -30 | 0 |
|  |  | 3 | CH_A+000_E+00 | 0 | 0 |
|  |  | 4 | LFE | - | - |
|  |  | 5 | CH_A+110_E+00 | +100 … +120 | 0 … +15 |
|  |  | 6 | CH_A-110_E+00 | -100 … -120 | 0 ... +15 |
| Multi-channel 7.1 | 8 | 1 | CH_A+030_E+00 | +30 ... +45 | 0 |
|  |  | 2 | CH_A-030_E+00 | -30 … -45 | 0 |
|  |  | 3 | CH_A+000_E+00 | 0 | 0 |
|  |  | 4 | LFE | - | - |
|  |  | 5 | CH_A+110_E+00 | +85 … +110 | 0 |
|  |  | 6 | CH_A-110_E+00 | -85 … -110 | 0 |
|  |  | 7 | CH_A+135_E+00 | +120 … +150 | 0 |
|  |  | 8 | CH_A-135_E+00 | -120 … -150 | 0 |
| Multi-channel 5.1+4 | 10 | 1 | CH_A+030_E+00 | +30 | 0 |
|  |  | 2 | CH_A-030_E+00 | -30 | 0 |
|  |  | 3 | CH_A+000_E+00 | 0 | 0 |
|  |  | 4 | LFE | - | - |
|  |  | 5 | CH_A+110_E+00 | +100 … +120 | 0 … +15 |
|  |  | 6 | CH_A-110_E+00 | -100 … -120 | 0 … +15 |
|  |  | 7 | CH_A+030_E+35 | +30 … +45 | +30 … +55 |
|  |  | 8 | CH_A-030_E+35 | -30 … -45 | +30 … +55 |
|  |  | 9 | CH_A+110_E+35 | +100 … +135 | +30 … +55 |
|  |  | 10 | CH_A-110_E+35 | -100 … -135 | +30 … +55 |
| Multi-channel 7.1+4 | 12 | 1 | CH_A+030_E+00 | +30 … +45 | 0 |
|  |  | 2 | CH_A-030_E+00 | -30 … -45 | 0 |
|  |  | 3 | CH_A+000_E+00 | 0 | 0 |
|  |  | 4 | LFE | - | - |
|  |  | 5 | CH_A+135_E+00 | +120 … +150 | 0 |
|  |  | 6 | CH_A-135_E+00 | -120 … -150 | 0 |
|  |  | 7 | CH_A+090_E+00 | +85 … +110 | 0 |
|  |  | 8 | CH_A-090_E+00 | -85 … -110 | 0 |
|  |  | 9 | CH_A+030_E+35 | +30 … +45 | +30 … +55 |
|  |  | 10 | CH_A-030_E+35 | -30 … -45 | +30 … +55 |
|  |  | 11 | CH_A+135_E+35 | +100 … +150 | +30 … +55 |
|  |  | 12 | CH_A-135_E+35 | -100 … -150 | +30 … +55 |
| FOA | 4 |  | A_1, A_2, A_3, A_4 | - | - |
| HOA_O* | OAO+12 |  | A_1, A_2, …, A_<OAO+12-1> | - | - |
| [Mono objects] | 1, …, TBD |  | O_1, …, O_<TBD> | - | - |

*O = Ambisonics order