Source:	Nokia Corporation, Orange

Title:	MASA format updates

Document for:	Discussion & Agreement

Agenda Item:	7.5 – IVAS_Codec

# 1. Introduction

IVAS support for spatial audio based on smartphone capture has been actively discussed since the Fukuoka meeting (SA4#97) leading to proposal of the Metadata-assisted spatial audio (MASA) as one IVAS encoder format. The MASA spatial metadata parameter set was later agreed at the Kochi meeting (SA4#100). Specifically, a proposal on MASA definition of spatial metadata (Table 1 of S4-181216) was agreed in SA4#100 to be later inserted as annex to IVAS-4 (once MASA is accepted) and documented in [1].

In addition to spatial metadata, the MASA format proposal includes descriptive metadata required for correct data ingest of variable size metadata chunks and for providing additional information potentially useful for encoding, decoding, or rendering. Example creation and rendering of the MASA format is possible using the IVAS MASA C Reference Software, which was originally made available at SA4#106 Busan meeting. The most recent version of the software package is available in [2].

This input provides proposed updates to the MASA format. The updates relate to corrections, improvements, and simplification of the format and the related software. The accompanying zip archive provides the IVAS MASA C Reference Software package that implements all the changes.

We propose to agree on these updates to the MASA format.

# 2. Proposed MASA format updates

The proposed updates relate to both the spatial metadata that describes the 3D sound scene and the descriptive metadata that provides additional information relating to, e.g., the capture setup used for creation of the audio and metadata. We discuss these separately in the following. The changes are relative to previous versions of the descriptions [3], [4], [5] and the latest IVAS MASA C Reference Software [2].

## 2.1 Descriptive metadata updates

The descriptive metadata is provided in each frame, although it is generally not frequently updated. One exception for this is ‘Number of directions’ parameter that may change frame-to-frame depending on the practical format generation device or application. This same parameter also defines the size of a specific metadata frame.

Table 1 presents the updated MASA descriptive metadata parameters. The updates are discussed in the following. The detailed description of the fields can be found in Annex A of this document.

Table 1. MASA format descriptive common metadata parameters

| Field | Bits | Description |
| --- | --- | --- |
| Format descriptor | 64 | Defines the MASA format for IVAS. Eight 8-bit ASCII characters: 01001001, 01010110, 01000001, 01010011, 01001101, 01000001, 01010011, 01000001 Values stored as 8 consecutive 8-bit unsigned integers. |
| Channel audio format | 16 | Combined following fields stored in two bytes. Value stored as a single 16-bit unsigned integer. |
| Number of directions | (1) | Number of directions described by the spatial metadata. Each direction is associated with a set of direction dependent spatial metadata. Range of values: [1, 2] |
| Number of channels | (1) | Number of transport channels in the format. Range of values: [1, 2] |
| Source format | (2) | Describes the original format from which MASA was created. |
| (Variable description) | (12) | Further description fields based on the values of ‘Number of channels’ and ‘Source format’ fields. When all bits are not used, zero padding is applied. |

‘Format descriptor’ is a 64-bit chunk consisting of eight 8-bit characters: I, V, A, S, M, A, S, A. This field replaces the previously used ‘Version’ field. The idea is that there can be one unique MASA format for the IVAS codec, and it is therefore unnecessary, e.g., to be able to update the version number.

‘Channel audio format’ is a 16-bit chunk that further defines the ‘Number of directions’, ‘Number of channels’, ‘Source format’, and additional description depending on the ‘Number of channels’ and ‘Source format’.

‘Number of channels’ is updated to support 1 or 2 channels only. By removing previously discussed support for 3 or 4 channels, the MASA format is significantly simplified. This limitation avoids duplication of the First-order Ambisonics (FOA) input format and removes advanced built-in input combinations that have been previously proposed but lacked wide support.

Previously defined [3] fields ‘Transport channel definition’ and ‘Source format description’ are removed and the pertinent information associated with these fields is integrated to new fields that form a variable 12-bit description (including any zero padding):

- Transport definition field

- Channel angle field

- Channel distance field

- Channel layout field

Details of the new configuration are found in Annex A of the present document.

These proposed updates simplify the overall format description and remove superfluous field combinations. Some finetuning of details, e.g., microphone patterns is also done. The most significant simplification comes from the limitation of the ‘Number of channels’ parameter.

## 2.2 Spatial metadata updates

The MASA spatial metadata was agreed at the Kochi meeting (SA4#100) and documented in the EVS SWG report [1]. There is one proposed update to this spatial metadata set and some algorithmic improvements.

### 2.2.1 Proposed update of the spatial metadata

Tables 2a and 2b present the updated MASA spatial metadata parameters. Table 2a contains the spatial metadata parameters that are dependent of number of directions, i.e., these parameters are provided for each of the directions present in the input. Table 2b contains the spatial metadata parameters that are independent of number of directions, i.e., these parameters are always provided once per each frame.

The proposed update here concerns Table 2a, where the ‘Distance’ parameter is now removed. This feature has been discussed in context of 6-DoF support in IVAS. Currently, the draft IVAS design constraints [6] do not require any 6-DoF support from the codec, and the feature thus goes beyond a reasonable requirement for capture-based IVAS encoder input format. On the other hand, the IVAS MASA C Reference Software [2] has no means to determine the ‘Distance’ information for spatial audio, which limits, e.g., test item creation.

Parameters of Table 2b are not updated.

Table 2a. MASA format spatial metadata parameters (dependent of number of directions)

| Field | Bits | Description |
| --- | --- | --- |
| Direction index | 16 | Direction of arrival of the sound at a time-frequency parameter interval. Spherical representation at about 1-degree accuracy. Range of values: “covers all directions at about 1° accuracy” Values stored as 16-bit unsigned integers. |
| Direct-to-total energy ratio | 8 | Energy ratio for the direction index (i.e., time-frequency subframe). Calculated as energy in direction / total energy. Range of values: [0.0, 1.0] Values stored as 8-bit unsigned integers with uniform spacing of mapped values. |
| Spread coherence | 8 | Spread of energy for the direction index (i.e., time-frequency subframe). Defines the direction to be reproduced as a point source or coherently around the direction. Range of values: [0.0, 1.0] Values stored as 8-bit unsigned integers with uniform spacing of mapped values. |

Table 2b. MASA format spatial metadata parameters (independent of number of directions)

| Field | Bits | Description |
| --- | --- | --- |
| Diffuse-to-total energy ratio | 8 | Energy ratio of non-directional sound over surrounding directions. Calculated as energy of non-directional sound / total energy. Range of values: [0.0, 1.0] (Parameter is independent of number of directions provided.) Values stored as 8-bit unsigned integers with uniform spacing of mapped values. |
| Surround coherence | 8 | Coherence of the non-directional sound over the surrounding directions. Range of values: [0.0, 1.0] (Parameter is independent of number of directions provided.) Values stored as 8-bit unsigned integers with uniform spacing of mapped values. |
| Remainder-to-total energy ratio | 8 | Energy ratio of the remainder (such as microphone noise) sound energy to fulfil requirement that sum of energy ratios is 1. Calculated as energy of remainder sound / total energy. Range of values: [0.0, 1.0] (Parameter is independent of number of directions provided.) Values stored as 8-bit unsigned integers with uniform spacing of mapped values. |

### 2.2.2 Improvements to direction indexing

The direction indexing corresponds to the function for transforming the audio direction angular values (azimuth ϕ and elevation θ) into an index, and the inverse function for transforming the index into the audio direction angular values.

In the Annex of document [5] these functions are described relying on a spherical grid of points uniformly spread on the unitary radius sphere. The spherical grid is defined as a succession of horizontal circles of points distributed on the sphere and corresponding to several elevation values. The indexing functions make the connection between the angles corresponding to each of these points on the grid and the 16-bit index. The proposed improvements retain all these characteristics and contain the following updates:

- fixing a bug by correcting the index representation of the points situated near the sphere poles

- the number of points ni,i=1,…,Nθ, where Nθ=122, is modified as follows to use exactly 216 points and enable fast indexing based on analytical cumulative cardinality determination:

  - on the equator of the spherical grid (θ=0) it is set to 
n1=430

  - at the poles (θ=±90 degrees) it is set to 
nNθ=1

  - the function calculating the number of points ni on the grid for other elevation indices, i=2,…,Nθ-1, is replaced using the following definition: 
ni=cumNi-cumNi-12

with cumN1=0 and

cumNi=2  roundi216-4322sin(i-12)δ-sinδ2sin(Nθ-32)δ-sinδ2

where δ is the uniform quantization step for i=1,…,Nθ-1,  2 roundix/2 is a rounding function to the nearest even integer (above x for  i=2, closest for i>2). The term cumNi gives the cumulative cardinality (i.e., cumulative number of points in the spherical grid) in a spherical zone going from the first non-zero elevation value to the i-th elevation value. This cumulative cardinality is derived from the relative area on the spherical surface, assuming a (near)uniform point distribution of the remaining number of points 216-432 (let alone the equator and poles).

- the indexing function relies on a direct uniform scalar quantization of elevation (instead of a codebook search) for i<Nθ-2 and computes the 16-bit index based on an analytical determination of cumulative cardinality for i=2,…,Nθ-1.

- the deindexing function uses a second order polynomial approximation for finding the first estimation of the elevation value corresponding to a given 16-bit index, instead of testing the successive index offsets for each circle of points.

As consequence the resulting method ensures the grid has exactly 216 points, the encoding distortion for a source of spherically uniformly distributed points is slightly reduced, and the indexing and deindexing functions have 10 times lower maximum complexity compared to the implementation in [2].

# 3. Summary and proposal

In this document, we provide several updates to the MASA format. The updates primarily simplify the format by limiting support to 1 or 2 channels with metadata and by removing superfluous (descriptive) metadata parameter combinations. In addition, the spatial metadata is simplified by removing the ‘Distance’ information, which has so far not been in IVAS scope, and which is not supported in available software. Furthermore, we provide algorithmic corrections and improvements in the software.

Following the updates, the MASA format is more concise, and the open points from previous discussions have been addressed. The IVAS MASA C Reference Software is also improved in terms of computational complexity and by ensuring full coverage of the 216 points on the spherical grid.

We propose to agree on the proposed updates to the MASA format. Thus, the updated software would also replace the previous version in [2].

# References

[1]	S4-181223: Draft report from SA4#100 EVS SWG

[2]		S4-210840: Updates to IVAS MASA C Reference Software, Nokia Corporation

[3]	S4-181216: On MASA spatial metadata energy ratio parameters, Nokia Corporation, Fraunhofer IIS

[4]	S4-190395: Proposal for IVAS MASA Channel audio format parameter, Nokia Corporation

[5]	S4-191167: Description of the IVAS MASA C Reference Software, Nokia Corporation

[6]	S4-211642: Permanent document: IVAS Design Constraints (IVAS-4), v0.3.0

# Annex A: Descriptive metadata for Metadata-assisted spatial audio inputs

Format descriptor (64 bits)

The unique format descriptor code is provided at the beginning of every MASA format metadata frame. It specifies MASA format for the IVAS codec.

| Required bit value | Decoded value | Additional description |
| --- | --- | --- |
| 01001001, 01010110, 01000001, 01010011, 01001101, 01000001, 01010011, 01000001 | “IVASMASA” | Unique format descriptor |

Channel audio format (16 bits as specified below)

Two bytes providing the following individual fields:

- Number of directions

- Number of channels

- Source format

and a variable 12-bit description configured based on ‘Number of channels’ and ‘Source format’.

Number of directions (1 bit)

This parameter field indicates how many directions are described in current MASA format frame. Size of the metadata chunk associated with the current frame depends on the number of directions.

| Bit value | Decoded value | Additional description |
| --- | --- | --- |
| 0 | 1 direction |  |
| 1 | 2 directions |  |

Number of channels (1 bit)

This parameter field indicates how many transport channels are used for the MASA format. This parameter is required in some form to read the correct number of channels. Some additional channel format descriptors depend on the number of channels.

| Bit value | Decoded value | Additional description |
| --- | --- | --- |
| 0 | 1 channel |  |
| 1 | 2 channels |  |

Source format (2 bits)

This parameter field describes the format of source signals that were used to form the MASA format input file/stream. This parameter provides additional information that can benefit encoding, decoding, and/or rendering. First bit value (00) is the default value.

| Bit value | Decoded value | Additional description |
| --- | --- | --- |
| 00 | Default/Other | Audio originates from unknown format(s) including mixed sources |
| 01 | Microphone grid | Audio originates from various (irregular) microphone grids (e.g., smartphones or other UEs) |
| 10 | Channel-based | Audio originates from premixed channel-based audio (e.g., 5.1) |
| 11 | Ambisonics | Audio originates from Ambisonics format |

Variable description (12 bits including zero padding)

Based on the values of the ‘Number of channels’ bit and ‘Source format’ bits, the variable description is configured to provide up to three additional fields to further describe the source format or transport channels. This information can guide, e.g., metadata encoding and rendering. The following presents the possible field combinations and their definitions.

Source format == 00 (Default/Other)

If number of channels is 1 (bit value 0), no additional metadata is specified. Instead, 12-bit zero padding is applied.

If number of channels is 2 (bit value 1), following additional fields are configured in order:

- Transport definition field (3 bits). This field describes the configuration of the two transport channels. The possible bit values and corresponding configurations are provided in Table 3.

- Channel angle field (3 bits). This field describes symmetric angle positions for transport signals with directivity patterns. In this notation, 0° corresponds to the front. The bit values and corresponding configuration are defined in Table 4.

- Channel distance field (6 bits). The bit values and corresponding configuration are defined in Table 5.

Table 3. Transport definition field for Source formats: Default/Other and Microphone grid

| Bit value | Decoded value | Additional description |
| --- | --- | --- |
| 000 | Unknown/Other |  |
| 001 | Omni |  |
| 010 | Subcardioid |  |
| 011 | Cardioid |  |
| 100 | Supercardioid |  |
| 101 | Hypercardioid |  |
| 110 | Dipole |  |
| 111 | Binaural |  |

Table 4. Channel angles for directive patterns for Source formats: Default/Other and Microphone grid

| Bit value | Decoded value | Additional description |
| --- | --- | --- |
| 000 | Unspecified |  |
| 001 | ±90 deg. |  |
| 010 | ±70 deg. | XY |
| 011 | ±55 deg. | XY, ORTF |
| 100 | ±45 deg. | NOS, XY, Blumlein |
| 101 | ±30 deg. |  |
| 110 | ±0 deg. | AB. Needs spacing for any stereo image |
| 111 | Reserved |  |

Note: If Transport definition value is “Unknown”, “Omni”, or “Binaural”, value 000 is assumed.

The channel distance parameter is defined with a few predefined values and the distance values between 0.01 m and 1 m are calculated as an equal multiplicative interval such that there are 60 values from 0.01 m to 1 m. The equation for this is given as:

ddec=59100B-3100

where ddec is the decoded distance value and B is the bit value as an integer value, i.e., B=3, …, 62. The result is in meters.

Table 5. Channel distance for Source formats: Default/Other and Microphone grid

| Bit value | Decoded value | Additional description |
| --- | --- | --- |
| 000000 | Unspecified | Distance is not specified, or it is unknown |
| 000001 | 0 m / coincident | No distance between microphones, i.e., they are coincident |
| 000010 | < 0.01 m | Distances smaller than 0.01 m |
| 000011 | 0.01 m | (Distances formed with equation above) |
| … | … | (Distances formed with equation above) |
| 111110 | 1 m | (Distances formed with equation above) |
| 111111 | > 1 m | Distances larger than 1 m |

Source format == 01 (Microphone grid)

If number of channels is 1 (bit value 0), no additional metadata is specified. Instead, 12-bit zero padding is applied.

If number of channels is 2 (bit value 1), following additional fields are configured in order:

- Transport definition field (3 bits). This field describes the configuration of the two transport channels. The possible bit values and corresponding configurations are provided in Table 3.

- Channel angle field (3 bits). This field describes symmetric angle positions for transport signals with directivity patterns. In this notation, 0° corresponds to the front. The bit values and corresponding configuration are defined in Table 4.

- Channel distance field (6 bits). The bit values and corresponding configuration are defined in Table 5.

The field definitions used for Microphone grid source format and Default/Other source format are the same. Differentiation is based on Source format parameter itself.

Source format == 10 (Channel-based)

For premixed content, the original layout can be provided. In addition to common CICP layouts relevant for IVAS, two generic options (3D and 2D) are available. The description of the bit values is provided in Table 6. The transport signals with this source format are assumed to be a mono (1 channels) or left-right stereo (2 channels) downmix of the multi-channel signals, and thus the number of channels can be 1 or 2 (bit values 0 or 1).

In addition to the 3-bit Channel layout field, 9 bits of zero padding is applied to complete the 12-bit variable description.

Table 6. Channel layout field for the channel-based source format

| Bit value | Decoded value | Additional description |
| --- | --- | --- |
| 000 | Unknown/Other | Unknown layout or other (3D) layout. Default option. |
| 001 | Other planar | Other 2D layout |
| 010 | 2.0 | CICP2 positions, ITU order |
| 011 | 5.1 | CICP6 positions, ITU order |
| 100 | 5.1+2 | CICP14 positions azimuth, 35° elevation, ITU order |
| 101 | 5.1+4 | CICP16 positions azimuth, 35° elevation, ITU order |
| 110 | 7.1 | CICP12 positions, ITU order |
| 111 | 7.1+4 | CICP19 positions azimuth, 35° elevation, ITU order |

Note 1: ITU channel order is given in ISO/IEC 23008-3:2015, Table 95.

Note 2: Azimuth positions are given in ISO/IEC 23091-3:2018, Table 3.

Source format == 11 (Ambisonics)

If number of channels is 1 (bit value 0), no additional metadata is specified. Instead, 12-bit zero padding is applied.

If number of channels is 2 (bit value 1), following two additional fields are configured in order:

- Transport definition field (3 bits). This describes the configuration of the two transport channels. The possible bit values and corresponding configurations are provided in Table 3. However, bit values 001 (omni) and 111 (binaural) are not allowed and should be interpreted as bit value 000.

- Channel angle field (3 bits). Describes symmetric angle positions for transports signals with directive patterns. 0° is assumed to point directly to the front. This is defined in Table 4.

- In addition, 6 bits of zero padding is applied to complete the 12-bit variable description.

For Ambisonics-based transport signals, transport channels are assumed to be coincident, and there is therefore no ‘Channel distance’ field specified.