Source:	Audio SWG

Title:	Update of Backward Interoperability and Bit Rate in the design constraint IVAS-4

## Document for:	Agreement

## Agenda Item:	7.5

Introduction

The following proposal is the agreed outcome of the on-line editing during the first Audio SWG session.

Proposal

- Source proposes to replace the following sentence in the backward interoperability box of [1]:

The IVAS Codec shall support certain stereo modes of operation which include an embedded bit-exact EVS mono downmix bitstream at the bit-rates from 9.6 kbit/s to 24.4 kbit/s SWB (9.6/13.2/16.4/24.4 kbit/s).

with

The IVAS Codec shall support certain stereo modes of operation which include an EVS-SWB bitstream representing a mono downmix.

2) Source proposed to replace the following in the Bit Rate Box,

When EVS bit-exact operation is used (see Backward interoperability), the IVAS codec shall operate at bit rates of EVS (including all EVS Primary and AMR-WB IO modes). In other cases than EVS bit-exact operation: the IVAS codec…

with

When input is a monaural signal without spatial metadata, the IVAS codec shall operate at bit rates of EVS (including all EVS Primary and AMR-WB IO modes). When input is a stereo signal and EVS interoperable operation is supported (see Backward interoperability), the bit-rates of the EVS bitstream representing a mono downmix shall be from 9.6 kbit/s to 24.4 kbit/s. In other cases: the IVAS codec…

References

[1] S4aA220002 Proposed edits to IVAS Design Constraints (IVAS-4) 0.3.0