Source:	VoiceAge Corporation

Title:	Proposal of Draft Overview of Experiments for IVAS Selection Testing

Document for:	Discussion

Agenda Item:	7.5

# Introduction

In [1], a high-level discussion on the configurations most relevant to be evaluated during the IVAS Selection process. In this contribution we present a first draft of possible organization of Selection Test experiments.

# Selection test experiments - overview

The table below presents a draft example proposal for Selection test experiments. It is expected to run each experiment twice, with two different languages. The goal of the table is to have a rough idea of what experiments the Selection test can cover. The table can also serve as a basis for External Listening Laboratories to estimate the costs per experiment. Finally, it can help to decide whether the whole Selection test can fit within the targeted budget. If it is felt that the test should be more comprehensive, or if it turns out that the cost per experiment is significantly higher than for EVS Selection, we can consider to run each experiment only once in External Laboratories, and to run the x-check tests in-house by candidate codec proponents, on their own expenses, as proposed in [1].

| Exp | Input Format | Source Material | Output Format | Bitrates | FER/Jitter | Headtracking | Methodology | Nb of References | Nb of test Conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Stereo | Clean speech | Stereo |  | ≤ x% | No |  |  |  |
| 2 | Stereo | Speech+Background | Stereo |  | ≤ x% | No |  |  |  |
| 3 | Stereo | Mixed & Music | Stereo |  | ≤ x% | No |  |  |  |
| 4 | 5.1 | Clean speech | Binaural |  | ≤ x% | No |  |  |  |
| 5 | 5.1 | Speech+Background | Binaural |  | ≤ x% | Yes |  |  |  |
| 6 | 5.1 | Mixed & Music | Binaural |  | ≤ x% | No |  |  |  |
| 7 | 7.1.4 | Clean speech | Binaural |  | ≤ x% | Yes |  |  |  |
| 8 | 7.1.4 | Speech+Background | 7.1.4 |  | ≤ x% | No |  |  |  |
| 9 | 7.1.4 | Mixed & Music | 7.1.4 |  | ≤ x% | No |  |  |  |
| 10 | FOA | Clean speech | Binaural |  | ≤ x% | Yes |  |  |  |
| 11 | FOA | Speech+Background | 7.1.4 |  | ≤ x% | No |  |  |  |
| 12 | FOA | Mixed & Music | Binaural |  | ≤ x% | No |  |  |  |
| 13 | HOA3 | Clean speech | Binaural |  | ≤ x% | No |  |  |  |
| 14 | HOA3 | Speech+Background | Binaural |  | ≤ x% | Yes |  |  |  |
| 15 | HOA3 | Mixed & Music | 7.1.4 |  | ≤ x% | No |  |  |  |
| 16 | Objects | Clean speech | Binaural |  | ≤ x% | Yes |  |  |  |
| 17 | Objects | Speech+Background | 7.1.4 |  | ≤ x% | No |  |  |  |
| 18 | Objects | Mixed & Music | Binaural |  | ≤ x% | No |  |  |  |
| 19 | MASA | Clean speech | Binaural |  | ≤ x% | No |  |  |  |
| 20 | MASA | Speech+Background | Binaural |  | ≤ x% | Yes |  |  |  |
| 21 | MASA | Mixed & Music | 7.1.4 |  | ≤ x% | No |  |  |  |

Notes:

- Currently considered methologies are P.800 DCR and MUSHRA

- Bitrates will be specified later, depending on the methodologies etc.

- Stereo may include binauralized samples (without head tracking)

- Stereo and Binaural Output is listened via headphones

- For inputs 7.1.4, FOA, HOA3, Objects & MASA vertical dimension is assumed in the samples

- Headtracking is assumed simulated

- Maximum Frame Error Rate (FER) x% depends on whether channel error conditions are mixed with clean channel conditions in the same experiment (as assumed in the above table), or whether separate experiments are designed specifically for testing channel errors. In the former case, x should not be too high to prevent compressing results for clean channel conditions, e.g. x=3.

- DTX on/off is assumed within the same experiment, where DTX on is used for relevant conditions

- ESDRUs are used only for listening via headphones

- FOA can possibly be merged with HOA

- Tandem is not tested in Selection

- Rate switching is not tested in Selection

- Operating points not tested in Selection will be addressed in the Characterization test

# Proposals

- To include the above table as a placeholder into IVAS-8a (Test Plan for Selection Phase) [2], under brackets.

- For candidate External Listening Laboratories to indicate approximate cost estimates per experiment for different listening scenarios and test methodologies. More specifically, providing approximate cost estimates for the following scenarios:

  - P.800 DCR [3] using headphones

  - P.800 DCR using 7.1.4 loudspeaker setup

  - BS.1534 MUSHRA [4] using headphones

  - BS.1534 MUSHRA using 7.1.4 loudspeaker setup

# References

S4aA220004: High-Level Organization of IVAS Selection Test

S4-220301: IVAS Permanent Document IVAS-8a: Test Plan for Selection Phase, v.0.3.0

Recommendation ITU-T P.800 (08/1996): Methods for subjective determination of transmission quality

Recommendation ITU-R BS.1534-3 (10/2015): Method for the subjective assessment of intermediate quality level of audio systems