3GPP TSG SA WG4#119e	S4-220607r02

E-meeting, 11th – 20th May 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.955 | CR | pseudo | rev | - | Current version: | 1.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_5G_Video] Verification Updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Ericsson, Interdigital, HHI. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GVideo |  |  |  |  |  | Date: |  |  | 04/05/2022 |
| Category: | B |  |  |  |  |  | Release: |  |  | 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Verification missing and adjusting text to account for the HDRTools bugs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Verification for HM, VTM, ETM. Proposed update to the TR text related to the wPSNR metric computation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | All changes are attached including updates to attachments. These cross-checks are plugged into a verification report and attached to each anchor. Two issues with HDRTools quality metrics computation utility are reported. The first issue is unstable behaviour of HDRTools metric computation utility in the case of fractional frame rate specified in parameters. Corresponding bug-report has been filied : Metrics calculation for fractional frame rate (#20) · Issues · standards / HDRTools · GitLab. The second issue is ambiguous configuration of the wPSNR HDR metric calculation in HDRTools. It was identified that peak value 1023 is being used in wPSNR HDR metric, which contradicts the TR specification and the provided configuration files. Text change addressing the identified issues is proposed. Based on offline Revision 1 available: Issue 1 For HDR Tools instead of using fractional, we use integer frame rate for the metrics computation and add a note to the report that this was done because of an existing bug in the tool. (Thomas) will create the note For bitrates, we use EFS. EVC has no SEI message included. Issue 2: Bug is HDRTools which sets to HDR Tools includes 1023 for wPSNR => HM, EVC, AV1 are already correct. VVC is included in this contribution. HDR Tools includes 1020 VTM with 1020 Thomas creates a revision 2 and upload Nils: please run HDRTools with integer for verifying the results for VVC, EVC and HM. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

===== CHANGE 1 =====

### 5.5.7	Reference computation of SDR metrics

Clause 5.5.3 specifies that all metrics are computed with 10-bit precision. 8-bit to 10 bit up-conversion is performed with HDRConvert tool [67] using the attached script HDRConvert_8bto10b.cfg. Note that ScaleOnly is set to 1 to address sequences with pixel values that are not within standard video range.

Computation of PSNR(Y), PSNR(U) and PSNR(V) metrics as defined in clause 5.5.3 is performed with HDRMetrics tool as included in HDRTools version 0.23 [67]  with the following parameters specified:

- EnablePSNR=1

- EnableJVETPSNR=1

- MaxSampleValue=1020.0

Computation of MS-SSIM metric as defined in clause 5.5.4 is performed with HDRMetrics tool with the following parameters specified:

- EnableJVETMSSSIM=1

- MaxSampleValue=1020.0

NOTE: An issue was discovered for the HDRMetrics tool with fractional frame rate. The bug is reported and expected to be fixed in a future version of HDRMetrics tool. For the purpose of this report, integer frame rates have been used for HDR metrics computation as metrics are independent of the frame rate. The bitrate is still measured based on the fractional frame rate.

For the computation of VMAF, the C++ executable "vmafossexec" [59], open source provided by Netflix is used (https://github.com/Netflix/vmaf/releases/tag/v2.1.1).

Here is the command line:

vmafossexec $VMAF_FMT $WIDTH $HEIGHT ref.yuv test.yuv $VMAFMODEL --thread 1 --log metrics.vmaf

$VMAF_FMT: describe yuv subsampling (yuv420p10le as all metrics are computed in 10 bit domain)

$VMAFMODEL: vmaf_v0.6.1.pkl

thread: 1 (no threads used)

Additional libvmaf parameters phone_model and enable_transform are set to 0.

===== End of CHANGE 1=====

===== CHANGE 2 =====

### 5.5.8	Reference computation of HDR metrics

Computation of wPSNR(Y), wPSNR(U) and wPSNR(V) metrics as defined in clause 5.5.5 is performed with HDRMetrics tool version 0.23 [67] with with reference config file HDRMetrics_wtPSNR.cfg as attached and where hdrTable.txt is attached to this report.

The following HDRMetrics parameters are specified:

- EnablePSNR=1

- EnableJVETPSNR=0

- MaxSampleValue=1023.0

Computation of DeltaE100 and PSNRL100 metrics requires conversion from YUV to linear light RGB data which is performed with the use of HDRConvert tool [67] and reference config file HDRConvertYCbCr420ToEXR2020.cfg as attached. After the conversion for reference and decoded video clips is done, computation of DeltaE100 and PSNRL100 metric as defined in clause 5.5.5 is performed with HDRMetrics tool version 0.23 [67] with reference config file HDRMetrics_deltaE100.cfg as attached

===== End of CHANGE 2=====

===== CHANGE 3 =====

### 5.9.1	Principles

Submissions of anchors, tests and associated metrics is based on 3GPP member input. However, such input to be fully credible is expected of to be verified. Verification in the context of this document includes the following according to Figure 5.9.1-1:

- Anchor bitstream verification: Anchor bitstreams are correct. By using a defined reference sequence, an anchor configuration as well as a reference encoder, two different executions of this process results in the same anchor bitstream.

- Anchor reconstruction is correct. By using a verified anchor bitstream, a reference decoder in two different implementations results

  - in the same anchor sequence,

  - in the same quality metrics.

The equivalent process is applied for tests.

Figure 5.9.1-1: Verification and cross-check processes

Verification is primarily supported by cross-checks from independent members who carry out the execution of the same process and done by the initial submitter of the anchors/tests. Whereas process 2 is deemed to be of manageable complexity, process 1 is of significantly higher complexity. Based on this it is expected that anchor reconstruction (process 2a) and anchor metrics (process 2b) are cross-checked and verified for every anchor, whereas anchor bitstreams are only cross-checked and verified for one anchor of the entire anchor tuple.

A successful cross-check is defined as follows:

- Anchor bitstream verification: Initial run and cross-check result in the same size and md5 for the anchor bitstream

- Anchor reconstruction verification. By using a verified anchor bitstream, a reference decoder in two different implementations results in the same md5 for the anchor sequence.

- Anchor metrics verification. By using a verified anchor bitstream, a reference decoder in two different implementations results in the same quality metrics with a maximum deviation of 0.05 for all metrics that are required for characterization.

The TR includes verified metrics in the main body of the text (i.e. successful completion of stage 3). Verified metrics are agreed to be correct and have been sufficiently cross-checked.

The TR may also include non-verified anchors, tests and metrics in the Annex G. In this case, the reason for the lacking verification is explained.

===== CHANGE 4: UPDATE ALL HM, ETM AND VTM =====

#### 6.2.9.2	H.265/HEVC Anchors

HEVC anchor streams are provided according to the key system here:

- https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-1-FHD/265/

HEVC anchor results are provided with the appropriate keys as defined in Table 6.2.8.3.1-1

- in the attached csv files

- https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-1-FHD/265/Metrics/

Cross-checks have been conducted and are documented here: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-1-FHD/265/cross-check.csv.

Based on these cross-check results, the verification status for the H.265/HEVC test stream is provided in https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-1-FHD/265/verification.csv and Table 6.2.9.2-1 with S for successful and F for failed. Empty cells indicate missing verification.

Table 6.2.9.2-1 Verification status of H.265/HEVC anchors for Full HD Scenario

| Anchor + Key | Summary |  |  | 22 |  |  | 27 |  |  | 32 |  |  | 37 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Verification Type | B | R | M | B | R | M | B | R | M | B | R | M | B | R | M |

| S1-A01-265 |  |  | S |  |  | S |  |  | S |  |  | S |  |  | S |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S1-A02-265 |  |  | S |  |  | S |  |  | S |  |  | S |  |  | S |
| S1-A03-265 |  |  | S |  |  | S |  |  | S |  |  | S |  |  | S |
| S1-A04-265 |  |  | S |  |  | S |  |  | S |  |  | S |  |  | S |
| S1-A05-265 |  |  | S |  |  | S |  |  | S |  |  | S |  |  | S |
| S1-A06-265 |  |  | S |  |  | S |  |  | S |  |  | S |  |  | S |
| S1-A07-265 |  |  | S |  |  | S |  |  | S |  |  | S |  |  | S |
| S1-A08-265 |  |  | S |  |  | S |  |  | S |  |  | S |  |  | S |
| S1-A11-265 |  |  | S |  |  | S |  |  | S |  |  | S |  |  | S |
| S1-A12-265 |  |  | S |  |  | S |  |  | S |  |  | S |  |  | S |
| S1-A13-265 |  |  | S |  |  | S |  |  | S |  |  | S |  |  | S |
| S1-A14-265 |  |  | S |  |  | S |  |  | S |  |  | S |  |  | S |
| S1-A15-265 |  |  | S |  |  | S |  |  | S |  |  | S |  |  | S |
| S1-A16-265 |  |  | S |  |  | S |  |  | S |  |  | S |  |  | S |
| S1-A17-265 |  |  | S |  |  | S |  |  | S |  |  | S |  |  | S |

===== CHANGE 5: UPDATE VTM HDR RESULTS  WITH ATTACHED =====

The data is uploaded.