3GPP TSG SA WG4 #119-e	S4-220752

E-meeting, 11-20 Mary, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.955 | CR | <CR#> | rev |  | Current version: | 1.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_5GVideo] Proposed editorial changes to the TR text and text for PSNR curves overlap. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM, Fraunhofer HHI, InterDigital and Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GVideo |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | B |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This document provides update to the text of TR26.955. Proposed editorial changes reflect earlier reached agreement documented in meeting notes which were missed for implementation in the current text, and text to document the PSNR curve overlap is also provided. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated TR text |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency of the TR |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8, 8.2.3, 8.3.3 and 8.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Introduction This document targets the text of the TR26.955, presents changes to the TR text .

Currently utilized terminology “no intra” and “intra” for scenario identifications was found potentially misleading and it was agreed to change it during the 118th SA4 meeting. This decision was document in the meeting notes and in the email communication over the reflector.

Change 1

It is proposed to replace across the document the terms “no intra” and “intra” for scenario identifications with terms “No RA” and “RA, fixed QP”.

End of the Change 1

It was agreed earlier that TR shall not report on a codec performance until results verification is confirmed. Current version of the TR reports on the performance results of the test scenarios for which verification results are not provided, and also reports the results which are known to be produced with a missconfiguration.

Change 2

It is proposed to remove across the documents the tables reporting BD rate gains for codecs or scenarios when coding results are not agreed and/or verification was not completed. The table of results to be banked out in the following subclauses until coding results are agreed and verified:

- Clause 8.2.3 VVC Characterization against H.264/AVC

- Clause 8.4.4 AV1 Characterization against H.265/HEVC HM

- Clause 8.4.5 AV1 against H.265/HEVC SCC

- Clause 8.4.6 AV1SCC against H.265/HEVC

- Clause 8.4.7 AV1SCC against H.265/HEVC SCC

End of the Change 2

It was discussed and agreed earlier (S4aV200569), that bd-rate comparison of new video codecs against the AVC may be inaccurate due to very large performance gap (around  60%-70% bd-rate gain on average) between AVC anchor and new codecs and significant misalignment (as little as as  20% overlap )  between rate distortion curves which lead to significant inaccuracy of the metric computation as stated in reference [44] (Technical Paper ITU-T HSTP-VID-WPOM (07/2020): "Working practices using objective metrics for evaluation of video coding efficiency experiments".)

Change 3a

### 5.8	Characterization

…

As described in [44], the PSNR ranges of two curves generally do not overlap completely, and the current practice is to avoid measuring the part of the area between curves that are extrapolated rather than interpolated, which can give unpredictable results if the non-overlapping parts are large. Therefore, the area between the two curves is only measured in the region where there is an overlap. If two curves overlap is only in a very small region, the BD-rate will be calculated using only a small (and possibly atypical) part of the available data. Therefore, it is important that the overlap is substantial for the BD-rate value to be meaningful.

The characterisation of the codecs in the framework is documented when the overlap is substantial.

Change 3b

### 8.2.3	VVC Characterization against H.264/AVC

#### 8.2.3.1	Overview

This clause provides information related to the characterization of VTM against H.264/AVC according to clause 7.2.1. The results provided in clauses 6 and 8.2.2 are considered for the characterization.

The results in clause 6 for the anchor are not verified.

Analysis of the PSNR ranges provided in the clause 6 for H.264/AVC and in sub-clauses 8.2.2 of the VVC shows that their rate distortion curves exhibit an average overlap of 61% across scenarios. As a consequence, the aggregation of results for all rate points for all sequences in each scenario is not substantial enough and do not provide an accurate representation for codec characterisation. BD rate gain are not reported in this section.

A way to obtain a substantial overlap for accurate estimation of BD rate gain estimates would be to adjust QP values of AVC for better alignments with the ones of HEVC and VVC.

#### 8.2.3.2	Scenario 1: Full HD

This clause provides information related to the characterization of VTM mode configurations against H.264/AVC for Scenario 1 Full HD.

For this scenario, the PSNR ranges of the curves between VVC and H.264/AVC overlaps at an average of 59% and can be as low as 29%

#### 8.2.3.3	Scenario 3: Screen Content

This clause provides information related to the characterization of VTM mode configurations against H.264/AVC for Scenario 3 Screen Content.

For this scenario, the PSNR ranges of the curves between VVC and H.264/AVC overlaps at an average of 43% and can be as low as 33 % in the non RA configuration and an average of 42 % and can be as low as 24% in the RA configuration.

#### 8.2.3.4	Scenario 4: Messaging and Social Sharing

This clause provides information related to the characterization of VTM mode configurations against H.264/AVC for Scenario 4 Messaging and Social Sharing.

For this scenario, the PSNR ranges of the curve between VVC and H.264/AVC in nooverlaps at an average of 69% and can be as low as 48% in the non RA configuration and an average of 76% and can be as low as 55% in the RA configuration.

#### 8.2.3.5	Scenario 5: Online Gaming

This clause provides information related to the characterization of VTM mode configurations against H.264/AVC for Scenario 5 Online Gaming.

For this scenario, the PSNR ranges of the curves between VVC and H.264/AVC overlaps at an average of 67% and can be as low as 54% in the non RA configuration and an average of 71% and can be as low as 61% in the RA configuration.

#### 8.2.3.6	Summary

[Table 8.2.3.6-1 provides a summary of BD rate gain in psnr and vmaf of VTM against H.264/AVC for different scenarios and configurations.

Table 8.2.3.6-1 Summary of BD rate gain in psnr and vmaf of VTM against H.264/AVC for different scenarios and configurations

| Scenario | average psnr | min psnr | max psnr | average vmaf | min vmaf | max vmaf |
| --- | --- | --- | --- | --- | --- | --- |
| S1 |  |  |  |  |  |  |
| S3 no intraNo RA |  |  |  |  |  |  |
| S3 intraS3 RA, fixed QP |  |  |  |  |  |  |
| S4 no intraNo RA |  |  |  |  |  |  |
| S4 intraS4 RA, fixed QP |  |  |  |  |  |  |
| S5 no intraNo RA |  |  |  |  |  |  |
| S5 intraS5 RA, fixed QP |  |  |  |  |  |  |
| Minimum |  |  |  |  |  |  |
| Maximum |  |  |  |  |  |  |

]

End of the Change 3

Change 4

### 8.3.3	EVC Characterization against H.264/AVC

#### 8.3.3.1	Overview

This clause provides information related to the characterization of ECM against H.264/AVC according to clause 7.2.1. The results provided in clauses 6 and 8.3.2 are considered for the characterization.

The results in clause 6 for the anchor are not verified.

Analysis of the PSNR ranges provided in the clause 6 for H.264/AVC and in sub-clauses 8.3.2 of the EVC shows that their rate distortion curves exhibit an average overlap of 70% across scenarios. As a consequence, the aggregation of results for all rate points for all sequences in each scenario is not substantial enough and do not provide an accurate representation for codec characterisation. BD rate gain are not reported in this section.

A way to obtain a substantial overlap for accurate estimation of BD rate gain estimates would be to adjust QP values of AVC for better alignments with the ones of HEVC and EVC.

#### 8.3.3.2	Scenario 1: Full HD

This clause provides information related to the characterization of EVC mode configurations against H.264/AVC for Scenario 1 Full HD.

For this scenario, the PSNR ranges of the curves between EVC and H.264/AVC overlaps at an average of 69% and can be as low as 3%

#### 8.3.3.3	Scenario 3: Screen Content

This clause provides information related to the characterization of ETM mode configurations against H.264/AVC for Scenario 3 Screen Content.

For this scenario, the the PSNR ranges of the curves between EVC and H.264/AVC overlaps at an average of at 63 % and can be as low as 53 % in the non RA configuration and an average of 62 % and can be as low as 43 % in the RA configuration.

#### 8.3.3.4	Scenario 4: Messaging and Social Sharing

This clause provides information related to the characterization of ETM mode configurations against H.264/AVC for Scenario 4 Messaging and Social Sharing.

For this scenario, the the PSNR ranges of the curves between EVC and H.264/AVC overlaps at an average of at 70% and can be as low as 49% in the non RA configuration and an average of 75% and can be as low as 55% in the RA configuration.

All Rate-Quality curves and BD rate gain plots are provided in the attachment as well as online here https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-4-Sharing/ETM/Characterization/.

#### 8.3.3.5	Scenario 5: Online Gaming

This clause provides information related to the characterization of ETM mode configurations against H.264/AVC for Scenario 5 Online Gaming.

For this scenario, the the PSNR ranges of the curves between EVC and H.264/AVC overlaps at an average of at 73% and can be as low as 62% in the non RA configuration and an average of 79% and can be as low as 69% in the RA configuration.

#### 8.3.3.6	Summary

[Table 8.3.3.6-1 provides a summary of BD rate gain in psnr and vmaf of ETM against H.264/AVC for different scenarios and configurations.

Table 8.3.3.6-1 Summary of BD rate gain in psnr and vmaf of ETM against H.264/AVC for different scenarios and configurations

| Scenario | average psnr | min psnr | max psnr | average vmaf | min vmaf | max vmaf |
| --- | --- | --- | --- | --- | --- | --- |
| S1 |  |  |  |  |  |  |
| S3 no intraNo RA |  |  |  |  |  |  |
| S3 intraS3 RA, fixed QP |  |  |  |  |  |  |
| S4 no intraNo RA |  |  |  |  |  |  |
| S4 intraS4 RA, fixed QP |  |  |  |  |  |  |
| S5 no intraNo RA |  |  |  |  |  |  |
| S5 intraS5 RA, fixed QP |  |  |  |  |  |  |
| Minimum |  |  |  |  |  |  |
| Maximum |  |  |  |  |  |  |

]

End of change 4

Change 5

### 8.4.3	AV1 Characterization against H.264/AVC

#### 8.4.3.1	Overview

This clause provides information related to the characterization of AV1 against H.264/AVC according to clause 7.2.1. The results provided in clauses 6 and 8.4.2 are considered for the characterization.

The results in clause 6 for the anchor are not verified.

Analysis of the PSNR ranges provided in the clause 6 for H.264/AVC and in sub-clauses 8.2.2 of the AV1 shows that their rate distortion curves exhibit an average overlap of 50% across scenarios. As a consequence, the aggregation of results for all rate points for all sequences in each scenario is not substantial enough and do not provide an accurate representation for codec characterisation. BD rate gain are not reported in this section.

A way to obtain a substantial overlap for accurate estimation of BD rate gain estimates would be to adjust QP values of AVC and AV1 for better alignments with the ones of HM.

#### 8.4.3.2	Scenario 1: Full HD

This clause provides information related to the characterization of AV1 mode configurations against H.264/AVC for Scenario 1 Full HD.

For this scenario, the the PSNR ranges of the curves between AV1 and H.264/AVC overlaps at an average of at 53% and can be as low as 21%

#### 8.4.3.3	Scenario 3: Screen Content

This clause provides information related to the characterization of AV1 mode configurations against H.264/AVC for Scenario 3 Screen Content.

For this scenario, the the PSNR ranges of the curves between AV1 and H.264/AVC overlaps at an average of at 21 % and can be as low as 5 % in the non RA configuration and an average of 73 % and can be as low as 62 % in the RA configuration.

#### 8.4.3.4	Scenario 4: Messaging and Social Sharing

This clause provides information related to the characterization of AV1 mode configurations against H.264/AVC for Scenario 4 Messaging and Social Sharing.

For this scenario, the the PSNR ranges of the curves between AV1 and H.264/AVC overlaps at an average of at 47% and can be as low as 30% in the non RA configuration and an average of 57% and can be as low as 49% in the RA configuration.

#### 8.4.3.5	Scenario 5: Online Gaming

This clause provides information related to the characterization of AV1 mode configurations against H.264/AVC for Scenario 5 Online Gaming.

For this scenario, the the PSNR ranges of the curve between AV1 and H.264/AVC overlaps at an average of at 43% and can be as low as 27% in the non RA configuration and an average of 59% and can be as low as 54% in the RA configuration.

#### 8.4.3.6	Summary

[Table 8.4.3.6-1 provides a summary of BD rate gain in psnr and vmaf of AV1 against H.264/AVC for different scenarios and configurations.

Table 8.4.3.6-1 Summary of BD rate gain in psnr and vmaf of AV1 against H.264/AVC for different scenarios and configurations

| Scenario | average psnr | min psnr | max psnr | average vmaf | min vmaf | max vmaf |
| --- | --- | --- | --- | --- | --- | --- |
| S1 |  |  |  |  |  |  |
| S3 no intraS3 no RA |  |  |  |  |  |  |
| S3 intraS3 RA, fixed QP |  |  |  |  |  |  |
| S4 no intraS4 no RA |  |  |  |  |  |  |
| S4 intraS4 RA, fixed QP |  |  |  |  |  |  |
| S5 no intraS5 no RA |  |  |  |  |  |  |
| S5 intraS5 RA, fixed QP |  |  |  |  |  |  |
| Minimum |  |  |  |  |  |  |
| Maximum |  |  |  |  |  |  |

]