Source:	NTT

Title:	FS_eiRTCW Permanent Document

Version: 	3.0.0

Document for:	Agreement

Agenda Item:	10.9

- Scope

The present document extends immersive Real-time Communication for WebRTC (iRTCW) and introduces a new concept called native WebRTC signalling.

In 3GPP, the use of WebRTC technology has been investigated since Rel-12 (around 2014). They are a network-based architecture for WebRTC access to IMS specified in Annex U to TS 23.228 and its stage 3 protocols specified in TS 24.371. They define functional entities including WIC (WebRTC IMS Client) and eP-CSCF (P-CSCF enhanced for WebRTC). The eP-CSCF is assumed to be located in the Home IMS domain and communicates with other IMS entities using the existing interfaces. For the C-plane signalling between WIC and eP-CSCF, those specifications specify an option to use SIP over WebSocket, whose information model can be used for options other than SIP over WebSocket. Although SIP satisfies almost all conversational applications, it is somewhat over-engineered or too strict to extend. Another method which is flexible, extensible, and can be optimized for new XR conversational applications, therefore, should be investigated. These requirements remind us of the original design principle of WebRTC. WebRTC, by its inherent characteristics, does not regulate C-plane signalling and allow a wide range of C-plane signalling. This study looks over this design principle again and investigates a new SIP-decoupled C-plane signalling, called native WebRTC.

Regarding the level of signalling details, TS 24.371 specifies a signalling transport mechanism using SIP over WebSocket, but it is not a mandatory mechanism for eP-SCSF. Even though there are other options such as XMPP or other application protocols over WebSocket, a RESTful based interface, etc., TS 24.371 does not specify any details of C-plane signalling using other options. Each service provider (e.g., operator) develops its own application by following the guidelines in TS 24.371. Its subscriber downloads the application and connects to the service and other subscribers only within the same service. Detailed C-plane signalling is left open to each operator’s design. In contrast, this study tries to identify a new C-plane signalling in detail (as an interface specification) to the extent that client implementations based on it have enough interoperability. This realizes connectivity to any operators or roaming services for new XR real-time communications. Operators can provide the interface common to them according to well-defined C-plane signalling specifications. Clients can connect to any operators via the interface (see Figure1).

Figure 1. Two approaches for defining specifications and their application connectivity

2	Motivations for Native WebRTC Signalling

2.1 	C-plane Signalling comparison

The C-plane signalling can be expressed as follows. Now, there are roughly four possible methods, classified in terms of their protocol stacks (see Figure 2).

Figure 2. Comparison of protocol stacks

The first method is MTSI-based, using SIP and SDP. General C-plane signalling requirements for conversational services can be covered by SIP. Interoperability is fine with the existing 5G core network. It is to be treated in IMS-based AR Conversational Services (IBACS).

The second is the method specified in TS 24.371. It enables the WebRTC clients to communicate over an IMS-based core network; only the interfaces for downloading dedicated applications and the signalling path using WebSocket are specified for C-plane signalling. Ordinary implementations adopt SIP-like protocols over WebSocket. In most cases, it is partially SIP-compliant or tightly coupled with SIP to adapt WebRTC clients in IMS domain.

The third method is an alternative to the second method that uses SIP-like protocol over WebSocket. The third method uses another signalling protocol over WebSocket, but SIP-decoupled approaches are investigated. It can be more lightweight, omitting features that is not used in XR conversational. Some constraints on SDP are necessary for interoperability. Non-browser based implementations are also in the scope. This method is the main subject of this study, FS_eiRTCW.

The other is a general WebRTC protocol stack that is not specified and left open to the users (i.e., service providers). C-plane may be SIP, XMPP, http, etc. A general WebRTC application uses SDP syntax compliant to RFC 4566 for its internal representation, when setting the local and remote descriptions. C-plane protocol may have its own on-the-wire format for SDP, which can be constructed from SDP and be serialized out to SDP.

Editor’s Note: 
- The reason why WebRTC signalling is necessary
- Comparison interworking between WebRTC signalling and existing SIP

3	Use Cases

Editor’s Note: 
- Use cases and communication methods (P2P, SFU, MCU)

4	Gap Analysis for WebRTC Architecture

4.1	General

This clause identifies what functional entities and reference points are needed for WebRTC, and proposes a possible architecture integrated with 5GC (5G Core Network) defined in 3GPP TS 23.501 [TS23.501].

4.2	Functional Entities for WebRTC

4.2.1	General

This clause enumerates functional entities in terms of 1) WebRTC specifications, 2) WebRTC implementations, and 3) providing inter-operator services.

1)	functional entities that are essential for this study and already defined in IETF RFCs or 3GPP specifications concerning WebRTC (see clause 4.2.2).

2)	functional entities that are not directly specified in WebRTC-related specifications in IETF RFCs or 3GPP specifications but are considered to be widely implemented for realizing WebRTC services; they are essential for this study (see clause 4.2.3).

3)	functional entities that may be specifically required for inter-operator or 3rd-party collaboration services if modification of signalling and termination of media on network boundaries are needed (see clause 4.2.4).

4.2.2	Functional Entities defined in WebRTC specifications

4.2.2.1	UE (User Equipment)

User Equipment (UE) contains a user agent function for WebRTC. The user agent function is equivalent to “WebRTC Endpoint” as described below.

For the purposes of the present document, the following terms and definitions given in IETF RFC 8825 [RFC8825] apply:

WebRTC Endpoint: Either a WebRTC browser or a WebRTC non-browser. It conforms to the protocol specification.

WebRTC Browser (also called a "WebRTC User Agent" or "WebRTC UA"): Something that conforms to both the protocol specification and the JavaScript API[W3C.WD-webrtc].

WebRTC Non-Browser: Something that conforms to the protocol specification but does not claim to implement the JavaScript API. This can also be called a "WebRTC device" or "WebRTC native application".

4.2.2.2	WSF (WebRTC Signalling Function)

The WebRTC Signalling Function (WSF) is a functional entity that is responsible for WebRTC signalling mechanism including capability exchange and management of call sessions between UEs and the network. This functional entity is described as “Servers” or “Web Server” in IETF RFC 8825 [RFC8825] clause 3. Each operator or 3rd-party in this study is assumed to have their own WSF in their network.

4.2.3	Functional Entities widely implemented for WebRTC

4.2.3.1	WMCF (WebRTC Media Centre Function)

The WebRTC Media Centre Function (WMCF) is a functional entity that performs media signal processing. WMCF terminates media signals and performs media processing (e.g., mixing, selective forwarding, transcoding) which are required for conferencing applications. It may also perform decryption and encryption of media packets if DTLS, SRTP, or TLS is used for a transport layer. It also has the function of storing contents (including text or other static material as well as audio and video) and providing them to the UE.

In cases, WMCF performs as a simple media relay function. It simply relays media data packets and supports IP packet connectivity. When UE behave as ICE Agents defined in IETF RFC 8445 [RFC8445], WMCF may be either STUN servers defined in IETF RFC 8489 [RFC8489] for connectivity check or TURN servers defined in IETF RFC 8656 [RFC8656] for relaying media data packets. This functional entity facilitates NAT traversal of UE and the connectivity between UE and other network functions.

This functional entity is generally implemented in WebRTC Multipoint Control Unit (MCU) or Selective Forwarding Unit (SFU).

4.2.3.2	CSF (Conference Supporting Function)

The Conference Supporting Function (CSF) provides the following functionality:

-	Conference session management, i.e., “CRUD” operation – create, read, update, delete of conference instances.

-	Providing supplementary files (e.g., icon images of participants, and shared documents) via best-effort transport different from the channels for real-time media.

-	Storage of user subscription data specific to MNO’s WebRTC services.

Note: In this study, it is assumed that a single user (i.e., identity) and its subscription data (associated with the identity) are assigned, owned, and managed by both MNO and service provider independently. The two identities have a link with each other via some technique. User subscription data specific to Service Provider’s services are stored in their networks.

-	Authorization Endpoint and Token Endpoint of OAuth 2.0 described in IETF RFC 6749[RFC6749] for establishing authentication linkage between MNO’s ID and Service Provider’s ID.

Editor’s note: OAuth token will be used to C-Plane authentication at WSF and U-Plane authentication at WMCF in the Service Provider model. STUN/TURN authentication with OAuth token is defined in IETF RFC 7635[RFC7635].

Portal http(s) servers of WebRTC services provide this function in general implementations.

4.2.4	Functional Entities needed for inter-operator services

4.2.4.1	WNSGF (WebRTC NNI Signalling Gateway Function)

The WebRTC NNI Signalling Gateway Function (WNSGF) is located at the boundary of the networks where different operators or third-party network connects.

Each operator or 3rd-party has its own WebRTC Signalling Functions (WSF) so that WSFs are connected to each other with border control functions such as security, policy management, charging, etc. WNSGF is inserted into “Signaling Path” in Figure 2 of RFC8825 and responsible for border control functions and supports session establishment between disparate address realms' networks.

4.2.4.2	WNMGF (WebRTC NNI Media Gateway Function)

The WebRTC NNI Media Gateway Function (WNMGF) is a media relay located at the boundary of the networks where different operators or third-party network connects. It is the function responsible for the border control and transport of media data packets between different networks. It may also transcode media data packets.

4.3	Possible Architecture

4.3.1 Overview

Figure 4.4-1 depicts a possible network architecture of this study. It contains the functional entities described in clause 4.2 and reference points between the entities.

Figure 4.3.1-1. Possible Architecture (from WebRTC’s viewpoint)

WSF and CSF may co-locate in a physical node. WNSGF and WNMGF are also optional when gateway functions are not needed at the network boundary. Terminal authentication needs further study.

4.3.2	Reference Points

The reference points shown in Figure 4.4-1 are described as follows.

Reference points for signalling are called as “Control Plane” or “C-Plane” in this study. Reference points for Media are similarly called as “User Plane” or “U-Plane” in this study.

Editor’s note: The interface names here are tentative. They are used only for the study. The final interface names may be updated based on the outcome of 5G_AREA.

Reference Points for Signalling

Rs-u		Reference Point between a WSF and a UE.

Rs-i		Reference Point between a WSF and another WSF in the same network (DN) or between a WSF and a WNSGF.

Rs-a		Reference Point between a WSF and a CSF.

Rs-n		Reference Point between a WNSGF and another WNSGF in an external network.

Reference Points for Media

Rm-u	Reference Point between a WMCF and a UE.

Rm-i		Reference Point between a WMCF and another WMCF in the same network (DN) or between a WMCF and a WNMGF.

Rm-n	Reference Point between a WNMGF and another WNMGF in an external network.

Reference Points for signalling nodes to control media nodes

Mc-i		Reference Point between a WSF and a WMCF.

Mc-r		Reference Point between a WNSGF and a WNMGF

Other Reference Points

Rh-u		Reference Point between a CSF and UE.

Detailed protocol for each reference point will be discussed in clause 5.

4.4	Possible Architecture with 5GC

4.4.1 Overview

A possible architecture in terms of WebRTC view is described in clause 4.3. This clause shows a solution for integrating the pure WebRTC architecture with 5GC (5G core).

NOTE: “pure WebRTC” means the original WebRTC described in IETF work, which basically does not take into account domain specific functions or features (e.g., mobile networks).

4.4.2 Mapping of Functional Entities

4.4.2.1	General

This clause identifies the mapping of functional entities shown in Figure 4.3.1-1 into 5GC functional entities defined in 3GPP TS 23.501 [TS23.501].

Currently, the mapping of two WebRTC functional entities are discussed. The mapping of other WebRTC functional entities needs further study.

4.4.2.1.1	WSF and AF

WSF is interconnected with UE and is expected to process the following:

1)	authenticate a UE.

2)	setup a WebRTC session required by a UE, which may be in another network.

3) manage QoS for the media path of a WebRTC session.

WSF interacts with 5GC functions and UE to perform 1) and 3) as the following:

1)	WSF retrieves the identity of a UE via EDGEAPP framework, then authenticates the UE.

3) WSF requests PCF to manage QoS for the media path of a WebRTC session retrieved from WebRTC C-Plane signals.

Editor’s note: The detail description of EDGEAPP framework is for further study.

Additionally, these processes are close to the processes of IMS functions such as P-CSCF and S-CSCF defined in 3GPP TS 23.228 [TS23.228]. The process of 1) is performed by S-CSCF and UDM, and 3) is performed by P-CSCF and PCF.

WSF can be mapped into “AF (Application Function)” of 5GC according to the definition of AF in 3GPP TS 23.501 clause 6.2.10 due to the following reasons:

-	WSF interacts with the 3GPP Core Network to provide services.

-	The interaction between WSF and PCF/UDM is close to IMS interactions with 5GC.

NOTE:	Process 2) is discussed in clause 4.2.2.1.2.

Editor’s note: A possible idea is as follows:
User subscription data for MNO specific to WebRTC are stored in CSF’s backend DB. User subscription data for OTT are placed in OTT data network. The linkage between MNO’s subscription data for WebRTC and UE’s identity as a 5G terminal is realized using EDGEAPP features. The linkage between MNO’s subscription data and OTT’s subscription data is realized with OAuth or OIDC (This part needs further study)

4.4.2.1.2	WNSGGF

4.2.2.1.2.1	Overview

This clause identifies the mapping of WNSGF to a 5GC functional entity. There are a couple of possibilities currently identified. The following two 5GC functional entities can be mapped from WNSGF:

-	NEF (see clause 4.2.2.1.2.2)

-	SEPP (see clause 4.2.2.1.2.3)

As another possibility, it may be appropriate that WNSGF is mapped to a brand new functional entity (e.g., Interconnection Border Control Function: IBCF). The exact mapping of WNSGF needs further study.

4.4.2.1.2.2	 WNSGF and NEF

When WSF is mapped into an AF, WNSGF can be mapped into an NEF (Network Exposure Function) due to the following reasons:

-	When WSF processes 2) of clause 4.2.2.1.2 and the session relates to other operator’s network, WSF (mapped to an AF) of operator A is requested to interact with WNSGF on the boundary of operator B to communicate with WSF (mapped into an AF) in operator B due to operator B’s policy. In this model, the relationship between WSF (in A) and WNSGF (in B) is close to the relationship between AF and NEF described in 3GPP TS 23.501 clause 6.2.10.

-	The major function of WNSGF is close to the former three functionalities described in 3GPP TS 23.501 clause 6.2.5.0; WNSGF exposes WSF’s WebRTC signalling capability and events. WNSGF interworks with WebRTC signalling between Rs-i and Rs-n reference points in terms of security and translation of internal-external information.

When WNSGF is mapped into an NEF, the definition of the NEF function may need to be modified as follows:

-	Descriptions for the exposure of WSF’s WebRTC signalling capability and the events by WNSGF are added to 3GPP TS 23.501 clause 7.2.8.

-	Descriptions for the event exposure details are added to 3GPP TS 23.502 clause 4.15.3.

-	Descriptions for the capability exposure details are added to 3GPP TS 23.502 clause 5.2.6.

4.4.2.1.2.3	WNSGF and SEPP

Security Edge Protection Proxy(SEPP) is defined in 3GPP TS 33.501[TS33.501] and TS 23.501. The SEPP is an entity sitting at the perimeter of the PLMN for protecting control plane messages, hiding network topology. The SEPP enforces inter-PLMN security on the N32 interface that is a reference point between a SEPP in one PLMN and a SEPP in another PLMN.

WNSGF is also located at the perimeter of the PLMN and its function is protecting control plane messages and hiding network topology. The function of WNSGF is close to that of SEPP.

The difference is the type of PLMN. WNSGF is located at the edge of inter-HPLMN. On the other hand, SEPP is expected to be used for N32 that lies between HPLMN and VPLMN.

4.4.3 Possible Architecture integrated with 5GC

The functional entities shown in Figure 4.4-1 are connected to 5GC (5G Core) as described in Figure 4.5-1.

Figure 4.4.3-1. Possible Architecture (integrated with 5GC)

WSF is mapped into an AF, and WNSGF is mapped into an NEF.

WSF is interconnected with PCF via N5 interface. WSF manages QoS of real-time media packets and signalling packets via N5 interface. WSF may interact with UDM to authenticate the UE.

Both signalling packets and media packets between UE and the network are transmitted via N6 interface. Signalling packets (C-Plane packets) from UE are transmitted to WSF, and real-time media packets (U-Plane packets) from UE are transmitted to WMCF. C-Plane signals may travel to WNSGF via RS-i, and may travel further to other operator’s WNSGF via RS-n. U-Plane signals may travel to WNMGF via Rm-i, and may travel further to other operator’s WNMGF. (see Figure 4.5-2)

Figure 4.4.3-2. Possible Architecture (from 5GC view, with data flows of C/U-Planes)

4.4.4	Mapping to iRTCW Collaboration Scenarios

The following table shows the mapping of functional entities in this study into iRTCW collaboration scenarios. Each box shows the condition (required or not) for MNO. The target of this study is Scenarios 3 and 4.

Table 4.4.4-1: Mapping to iRTCW collaboration scenarios

| Functional Entity | Collaboration Scenario 3 |  | Collaboration Scenario 4 |
| --- | --- | --- | --- |
|  | 3A / Service Provider provides WebRTC services and MNO assists the services. | 3B / MNO provides WebRTC services only in the MNO’s network | MNO’s WebRTC service interconnects with other MNO’s or Service Provider’s service |
| WSF | Required | Required | Required |
| WMCF | Required (Note 1) | Required | Required |
| CSF | Required | Required | Required |
| WNSGF | N/A (Note 2) | N/A | Required |
| WNMGF | N/A (Note 2) | N/A | Required |
| NOTE 1: Scenario 3A in this table assumes WMCF acts as a TURN relay server at the network boundary between the MNO and the service provider. Further Operator-Assistance models may be introduced. NOTE 2: Scenario 3A in this table assumes WSF does not communicate with Service Provider’s WebRTC functions and WMCF works as a gateway by itself. Further Operator-Assistance models may be introduced. |  |  |  |

4.5	Possible Architecture in Software View

4.5.1	Overview

There are two types of WebRTC Endpoint as described in 4.1.1; One is “WebRTC Browser”, and the other is “WebRTC Non-Browser”. This clause shows possible architectures for each type of endpoints in terms of software development.

4.5.2	WebRTC Browser

A JavaScript (JS) application runs on “WebRTC Browser” type UE. The application runs on a web browser that has capabilities of JS APIs including	WebRTC API defined by W3C. (see Figure 4.5.2-1)

Figure 4.5.2-1. Software view of “WebRTC Browser” type endpoint

According to the concept of WebRTC described in IETF RFC 8829 [5], the procedures and protocols stated in this study are expected to be fully writable only with JS.

All of the functions needed for realizing immersive RTC must be provided via JS API because the only way for JS applications to access devices or networks is utilizing JS API. Functions provided by enablers for immersive RTC must be accessible via web browser’s JS API.

4.5.3	WebRTC Non-Browser

An application written in a programming language specific to the UE platform runs on “WebRTC Non-Browser” type UE. (see Figure 4.6.2-1)

Figure 4.5.3-1. Software view of “WebRTC Non-Browser” type endpoint

NOTE:	The programming language and programming APIs used to write applications depend on the UE platform. For example, Java and Android API (SDK) will be selected for Android platform UEs, Swift and its libraries will be selected for iOS platform UEs, and C++ and Win64 API will be selected for Windows platform UEs.

The application is realized in a way other than JS running on a web browser. This study does not state details of the application’s implementation; this study mainly discusses the network interface. The network interface is the same for both “Browser” and “Non-Browser” type UEs.

4.6	Media connection model

In the original WebRTC design, the communication between UEs is thought to be peer-to-peer (P2P). In most of the existing WebRTC implementations, however, the media connection is not P2P. An intermediate server (or servers) between UEs is used. In the multi-party call, the intermediate server which performs media processing is helpful for a UE because, for a UE, decoding all media from other UEs is a heavy load. Direct full-mesh connections among multiple UEs consumes a lot of network resources. Additionally, such an intermediate server is useful even for a one-to-one communication for offloading immersive media processing which needs more computation power than conventional media. This leads to the discussion about split rendering.

This study mainly focuses on the media connection model with intermediate servers.

P2P connection has some benefit for one-to-one communication (i.e., no need for an intermediate server and less server-relayed delay). For that reason, P2P connection is also considered for some special cases.

4.7	IP Addressing

4.7.1	General

IP addressing for UE has some options: assigning IPv4 address only, IPv6 address only, or both.

In the operator deployment, the number of available IPv4 addresses would be insufficient for its subscribers. Generally, operators use IPv4 private address (and ISP shared address defined in IETF RFC 6598[RFC6598]) with network address translation (NAT).

In Clause 4.7, appropriate IP addressing is identified, discussing NAT-traversal in the WebRTC user plane and network verified ID retrieval.

4.7.2	NAT

4.7.2.1	Overview

NAT, including port translation as NAPT (Network Address and Port Translation), is a method of mapping an IP address space into another, which is mainly used to translate a private IP address into a global IP address, and vice versa, for communicating with external networks.

Generally, UE can be assigned with an IP address through a PDU session in operator networks. When an IPv4 address is allocated, as mentioned in 4.7.1, a private IP address or an ISP shared address is used. On the contrary, when an IPv6 address is allocated, a global unicast address is assigned.

NAT is essential for carrier-grade network deployment. Subscribers can be much more than usually available IPv4 global address space, and they are treated by using IPv4 private address and NAT. The same private address can be reused in each different domain behind NAT. Although NAT deployments have a wide variety, NAT is generally installed in a DN (data network) and often put in the middle between the UPF and other functional entities (see Figure 4.7.2-1).

On the other hand, IPv6 global unicast addresses basically do not require NAT, except for special security reasons or some transition method between IPv6 and IPv4 domains.

Figure 4.7.2-1 Possible NAT location

4.7.2.2	NAT Variation

NAT is classified into some types by its address translation and packet filtering behavior.

The first version of STUN in IETF RFC 3489[RFC3489] defines three types:

-	Full Cone NAT,

-	Restricted NAT (Restricted Cone NAT or Restricted Port Cone NAT), and

-	Symmetric NAT.

Full cone NAT does not limit access to an internal UE from external network entities, which have not communicated with the internal UE. Any external entities can re-use the external IP address and port number mapped to a specific internal UE and can access to it (Figure 4.7.2.2-1). Full cone NAT is less restrictive than other NATs. Restricted NAT only permits external entities to access the internal UE if the NAT have received any packets from the internal UE directed to the external UE (Figure 4.7.2.2-2). Symmetric NAT uses a different pair of an external IP address and port, which are specific to each external entity and only the external entity can access to the internal UE through the IP address and port pair.

Figure 4.7.2.2-1 Full Cone NAT behaviour

Figure 4.7.2.2-2 Restricted or Symmetric NAT behaviour

4.7.2.3	Existing NAT-traversal

4.7.2.3.1	General

An effective NAT-traversal method is different depending on the NAT type described in Clause 4.7.2.2.

In the original WebRTC design, STUN and TURN are listed, included as ICE, for major NAT-traversal methods. In addition, Hosted NAT Traversal (HNT, described in IETF RFC 7326 [RFC7326]) and its similar mechanism are frequently used in real implementations for conversational applications.

4.7.2.3.2	STUN

STUN is the method for UE behind the NAT to discover its external IP address observed by external networks. This method supports P2P communications and only works for full-cone NAT.

This study excludes STUN because the main communication model is not P2P but with intermediate servers (as described in Clause 4.6), and general NATs deployed in operator networks are not limited to full-cone type.

4.7.2.3.3	TURN

TURN is the method for UE behind the NAT to communicate with external nodes via an intermediate server. TURN is a protocol for the session management and requires an intermediate server.

Generally, this method is regarded as the last resort for NAT-traversal for UDP-based conversational services. This method does not require the alignment with other control plane signalling, but is equipped as its own user plane connection management mechanism. This method needs additional message exchanges and has a protocol overhead.

The TURN server has its authentication mechanism for UEs and can be used for the purpose of traffic steering for an inter-operator communication scenario detailed in Clause 4.8.

4.7.2.3.4	HNT

HNT (Hosted NAT Traversal) is the mechanism that a session border controller (SBC) placed at the edge of networks intermediates the communication between UEs behind NAT.

The problem tackled by HNT is that a UE behind a NAT tries to set up a session with its private address and port number for media, which have no clue to the SBC for the real media which comes later.

Regarding the control plane signalling, the signalling part of the SBC modifies media-related information represented by the private IP address and port number set in the SDP offered by an originating node into a global IP address and a new port number. This modification enables a terminating node to target the accessible IP address and port pair provided by the SBC. In the signalling return path, the SBC also modifies the terminating node’s IP address and port number set in the SDP answered by the terminating node into new ones, and forwards it to the originating node. This is to solicit the originating node to send media to the SBC. Once the SBC receives the first media packet from the originating node targeting at the solicitation, the SBC recognizes the real NAT-ed IP address and port pair of the originating node. The SBC captures that information and uses it for relaying packets from the terminating node to the originating node. This is called “latching”.

This method is embedded in the control plane signalling and does not require extra message exchange. For that reason, it has no additional protocol overhead. It is a better feature than TURN in the same condition requiring an intermediate server.

Since this study focuses on the connection model with an intermediate server, the NAT issues can be argued differently. Let’s assume that all communication services are provided by the intermediate server as a conference. UEs can just join the open channel provided by the server and receives media from the server. UEs can also send their media to the intermediate server and the server mixes the media and distributes to other UEs. In this model, the first join packet from a UE to the NAT and the NAT to the server creates an address mapping at the NAT. The server simply sends packets to the source address of the join packet from the UE.

This mechanism does not need the dedicated protocol and there is no additional protocol overhead for NAT-traversal by sending media to the specific IP address and port pair exposed by WMCF. That points are analogous to HNT (Figure 4.7.2.3-1).

Figure 4.7.2.3-1 HNT like NAT-traversal

4.7.2.4	Conclusion of NAT handling

NAT-traversal problems have been discussed and several solutions have been proposed as described above. However, if equipment for NAT-traversal is not required, certainly less server resources would be needed.

In short, it is preferable that only IPv6 global unicast address be assigned to UE and no dedicated NAT-traversal equipment be used. Intermediate servers are used mainly for media processing and for the media relay when there is no direct IP reachability (e.g., across inter-operator connection).

4.7.3	IP Address and Trustable Subscriber Identifier

The operator uses subscription identifiers (e.g., GPSI (Generic Public Subscription Identifier) in 5GC) for managing its customer’s service subscription and charging. In WebRTC support, the operator needs to check customer’s service requests by checking against operator’s subscriber database organized with the subscription identifier. An OTT-specific ID and password may be insufficient even in the collaboration scenario with external service providers because they cannot be securely linked with subscriber information in the viewpoint of the operator. The issue is how the MNO deduces (or retrieves) the trustable subscriber identifier from customer’s requests, which are carried by IP packets.

Trustable subscriber identifiers in the MNO network are required for certain validity check, since a UE’s self-claimed GPSI and source IP address are untrusted.

The EDGEAPP architecture specifies the method how the EAS function block retrieves the GPSI from terminal’s source IP address. The AF regarded as an EAS can retrieve the GPSI bound to the UE by Eees_UEIdentifier API in EDGEAPP. This mechanism and its flow contain authentications conducted at the related network functions (i.e., EES and NEF), which enable the EAS to acquire the valid GPSI in the operator network as a trustable subscriber identifier.

Validity of the terminal’s source IP address needs consideration. UE’s self-claimed IP address, especially presented in an application level, is not trustable. The source IP address presented in an IP header can be relatively trustable when the IP packet is transmitted through a connection with some handshake procedures.

The IP address linkage with a subscriber identifier also has an issue when NAT is deployed. In Release 18, the method with which the AF can identify the trustable subscriber identifier (e.g., GPSI) to invoke the 3GPP network service API for the UE (Application client) remains to be investigated in eEDGEAPP. In VoLTE, this linkage with NAT can be achieved with the help of additional operator-specific information (e.g., PDN session related value). In the AF for WebRTC, it depends on which additional information element can be acquired by the AF. There is no clear answer for the ID linkage between the NAT-ed IP address and the subscriber identifier.

Contrarily, the UE IP address without translated by NAT can be linked with GPSI by Eees_UEIdentifier API (though detailed specification is needed).

In terms of ID linkage, using IPv6 global unicast address for UE is reasonable.

Using IPv4 private address will be studied further when NAT-ed ID linkage issue is solved.

4.7.4	Conclusion of IP Addressing

In terms of the required server resources for NAT-traversal and unclear retrieval of the trustable subscriber identifier, using IPv6 global unicast address for UE is reasonable. NAT deployments have a wide variety of behaviors and cannot be treated straightforward (refer to Clause 4.7.2). Using media relay servers that act as either TURN or HNT covers most cases with NAT-traversal. However, there are still issues using IPv4 private address with NAT, such as ID linkage (refer to Clause 4.7.3). For the sake of simplicity and to concentrate on identifying signalling requirements, this study considers IPv6-only use.

Also, IPv6-only use is acceptable for future services because IPv6 address allocation to UEs is now widely available among operators. Also, IPv6-only deployment (or not using limited IPv4 address resource) leads to efficient system development and equipment utilization.

4.8	QoS Enabled End-to-End Path

This study covers two collaboration scenarions as is described in the previous clause. In the collaboration scenario where the WebRTC functions in an MNO assist an external service provider (OTT or another MNO), setting up a QoS-enabled media path across different networks needs to be studied .

The media path from a UE to the external service provider is roughly divided into three sections:

Section 1) Between a UE and the UPF (Operator’s CN section)

Section 2) Between the UPF and the operator’s network edge (Operator’s DN section)

Section 3) Between the operator’s network edge and the external service provider network edge

Section 4) A network in the external service provider

Section 4) is a matter of a service provier and out of scope of this study.

Regarding Section 1), this section includes the operator’s core network. In this section, QoS is controlled by the PCF. In the collaboration scenario with an external service provider, the main signalling server is placed in the service provider’s domain. While UE exchanges control plane signalling messages with the signalling server placed in the service provider’s domain, UE sends a QoS-related request separately to the WSF placed in the operator network. The WSF receives and interprets the UE’s request and requests the PCF to prioritize the UE’s specific session.

Regarding Section 2), operator’s DN may have sufficient bandwidth and other QoS mechanism may be adopted.

Regarding Section 3), this section’s QoS control needs a bandwidth guaranteed path (i.e., a dedicated line) and forwarding control. Such a forwarding path selection cannot be realized by simple IP routing. A couple of options are possible. The first option is that only media packets to be prioritized are transmitted to WMCF placed in the operator’s network (which acts as TURN server) and the WMCF relays the media to the main media server in the OTT network via guaranteed path (Path 1 in Figure 4.8-1). The second option is that a non-prioritized forwarding path can be selected using a different destination IP address other than WMCF’s transport address within the same PDU session (Path 2 in Figure 4.8-1). The third option is that such a non-prioritized path can be selected through a different PDU session which goes through a different gateway (Path 3 in Figure 4.8-1). Such a PDU session can be selected by Data Network Name (DNN).

Figure 4.8-1. Forwarding Path Selection

Editor’s Note: 
Analyze gaps and identify required enhancements of terminal device and network architectures including additional functional entities (e.g., WebRTC Signalling Server, ICE-STUN Server, IMS Interworking Gateway, NNI Gateway).

- Stage2 work requirements
- Necessary functional blocks
- Architectural comparison (details are in annex)

# 5	Requirements for C-Plane Signalling

5.1	General

This clause identifies requirements for control plane signalling for each reference point. In this study, reference points for control plane are Rs-u, Rs-i, Rs-n, Rs-a, as described in clause 4.

5.2	Rs-u

5.2.1	Base Protocol

HTTP[RFC7230-7235][RFC7540]/HTTPS and WebSocket[RFC6455] are available options for signalling between UE and WSF so that connection setup procedure could be invoked by JavaScript API as described in IETF RFC 8825 [RFC8825] clause 3. Nevertheless, HTTP/HTTPS is less appropriate for two reasons described in RFC 6455 clause 1.1:

-	Server load caused by http transactions (based on request-response)

-	A connection has two sessions each for sending and receiving signalling packets

In addition, when a notification from the network to the UE is required, for such as an incoming call, an HTTP(S) connection is originated from the network side, but this case has some problem. Generally, NAT box is placed between UE and network entities, therefore NAT-traversal problem should be resolved. Besides, in terms of security configuration, UEs often deny incoming TCP[RFC793] connections.

For that reasons mentioned above, only WebSocket is utilized for the base protocol of signalling in this study. WebSocket can solve the three problems, server load, number of sessions and the NAT-traversal.

5.2.2	Upper Layer Protocol over WebSocket

In IETF RFC 8825 [RFC8825], upper layer protocols over the base protocol of control plane are not specified and are thought to be application specific. In the RFC, SIP[RFC3261] and XMPP[RFC6120] are listed as candidate protocols for control plane.

5.2.2.1	SIP

Utilizing SIP for control plane signalling for WebRTC is already described in 3GPP TS 24.371 [TS24.371] clause 5. One of the main advantages of using SIP is the ease of interwork between WebRTC-aware network and IMS network. On the other hand, disadvantages of using SIP are as follows:

-	UE and network must be able to understand both WebRTC and SIP. SIP is not widely used outside of telephony. If SIP must be used in conjunction with WebRTC, the advantage of WebRTC, friendliness to web-based development environments and developers, is to be spoiled.

-	SIP has a strictly managed communication model as SIP dialogue. In principle, the originatedal signalling is transparently relayed through the network and the terminals manage the dialogue with each other. These characteristics are not compatible with the UE-network relation model, which is the scope of this study.

-	UE and network must be able to understand both WebRTC and SIP. SIP is not widely used outside of telephony. If SIP must be used in conjunction with WebRTC, the advantage of WebRTC, friendliness to web-based development environments and developers, is to be spoiled.

-	SIP specifies methods divided by each signalling characteristic (i.e., INVITE, ACK, BYE, CANCEL, PRACK, UPDATE, SUBSCRIBE, NOTIFY, REFER, PUBLISH, INFO). Adding control for a new characteristic may need to start from the method definition.

-	Less affinity with cloud environment which mainly uses HTTP. For example, raw values of the IP addresses related to the SIP dialog (consisting a communication path of SIP trapezoid) are in the protocol header or message body, therefore changing communication elements is difficult once the call session is established.

For those reasons above, SIP is not appropriate except for the applications where the interwork to IMS is expected.

5.2.2.2	XMPP

There is no specification using XMPP for the upper layer protocol of the control plane signalling in 3GPP and no major commercial implementations of WebRTC either. The reason seems that XMPP can be used on its own and does not need to be combined with other protocols. WebSocket encapsulation of XMPP has little benefit except the case that an application using XMPP is implemented using JavaScript.

Therefore, this study will not utilize SIP and XMPP. More optimal (or WebRTC native) signalling protocols for the upper layer of control plane is to be identified in this study.

5.2.2.3	Other Existing Implementations

Among the existing implementations of WebRTC communication, JSON[RFC8259] format is mainly used for the upper layer of control plane. This is because JSON format is easy to handle in JavaScript. In this study, the potential of JSON for the upper layer protocol of control plane signalling is investigated.

In 3GPP specifications, RESTful APIs (such as service-based interface and Northbound APIs) are often defined using OpenAPI 3[OpenAPI] and the message-body of the APIs are based on JSON. However, OpenAPI is mainly suitable for RESTful APIs and not suitable for message-driven APIs such as control plane signalling over WebSocket. For this reason, AsyncAPI[AsyncAPI] as well as OpenAPI are used for identifying API schemas in this study. Those two APIs are managed by Linux Foundation.

5.2.3 Protocol Stack

As described above, JSON based protocol over WebSocket is a solution for the protocol of Rs-u. (see Figure 5.2.3-1)

WebSocket can be deployed over several versions of HTTP.

-	WebSocket with HTTP/1.1 is specified in IETF RFC 6455 [RFC6455] and used in this study. HTTP/1.1 is not, however, shown in the protocol stack because HTTP/1.1 does not remain after upgrading into WebSocket.

-	WebSocket with HTTP/2 is specified in IETF RFC 8441 [RFC8441] and used in this study. HTTP/2 is shown in the protocol stack because HTTP/2 framing remains after a stream in HTTP/2 connection is upgraded into WebSocket.

-	WebSocket with HTTP/3[RFC9114] is specified in IETF RFC 9220 [RFC9220] but not used in the current version of this study. The transport protocol used over HTTP/3 needs to be selected in alignment with IETF/W3C discussions.

The sub layers of each protocol are according to the existing specifications.

-	TLS under HTTP/1.1 and HTTP/2 is specified in IETF RFC 8446 [RFC8466].

-	TCP under TLS is specified in IETF RFC 793[RFC793].

-	IPv4 and v6 under TCP are specified in IETF RFC 791[RFC791](IPv4) and IETF RFC 8200 [RFC8200](IPv6).

Figure 5.2.3-1. Protocol Stack of Rs-u

Editor’s Note: 
Identify signalling protocol details (e.g., based on JSON) for the common WebRTC-based immersive RTC session management.
Identify information elements in the C/U-Plane signal (including NNI) to enhance connectivity of media sessions with carrier assistance for WebRTC-based applications (including OTT applications).
Identify the minimal functional capabilities needed to support the enhancements identified in above.

- Stage3 requirements
- C-plane signalling requirements
- Signalling methods comparison (details are in annex)

6	Requirements for U-plane Signalling

7	Interworking with IMS Network

8	Tethered Cases

Editor’s Note: SmarTAR-related clause;
Identify enhancements for E2E QoS realizations over 5G systems for communications between MNOs and WebRTC clients operating over non-5G links (e.g., Wi-Fi) using WebRTC-based transport.  This also includes communication between WebRTC clients operating on tethering/tethered devices.

9	Security Considerations

Editor’s Note: 
Considerations that the third-party access to the operator network need to be controlled with SLAs and with secure access to protect the underlying network resources.
- Rate limiting
- Abuse protection
- Security measures

10	Related Groups Considerations

Editor’s Note: 
Identify collaboration formation with other WGs in 3GPP and SDOs including IETF and W3C.

11	Conclusions and Recommendations

12	References

[TS23.228]		3GPP TS 23.228: “IP Multimedia Subsystem (IMS); Stage 2”

[TS23.501]		3GPP TS 23.501: “System architecture for the 5G System (5GS); Stage 2”

[TS33.501]		3GPP TS33.501: “Security architecture and procedures for 5G system”

[TS24.371]		3GPP TS 24.371: “Web Real-Time Communications (WebRTC) access to the IP Multimedia (IM) Core Network (CN) subsystem (IMS); Stage 3; Protocol specification”

[W3C.WD-webrtc]	W3C Proposed Recommendation, “WebRTC 1.0: Real-time Communication Between Browsers”, <https://www.w3.org/TR/webrtc/>

[RFC791]		IETF RFC 791: “Internet Protocol”

[RFC793]		IETF RFC 793: “Transmission Control Protocol”

[RFC3261]		IETF RFC 3261: “SIP: Session Initiation Protocol”

[RFC3489]		IETF RFC 3489: “STUN – Simple Traversal of User Datagram Protocol (UDP) Through Network Address Translators (NATs)”

[RFC6120]		IETF RFC 6120: “Extensible Messaging and Presence Protocol (XMPP): Core”

[RFC6455]		IETF RFC 6455: “The WebSocket Protocol”

[RFC6598]		IETF RFC 6598: “IANA-Reserved IPv4 Prefix for Shared Address Space”

[RFC6749]		IETF RFC 6749: “The OAuth 2.0 Authorization Framework”

[RFC7230]		IETF RFC 7230: “Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing”

[RFC7231]		IETF RFC 7231: “Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content”

[RFC7232]		IETF RFC 7232: “Hypertext Transfer Protocol (HTTP/1.1): Conditional Requests”

[RFC7233]		IETF RFC 7233: “Hypertext Transfer Protocol (HTTP/1.1): Range Requests”

[RFC7234]		IETF RFC 7234: “Hypertext Transfer Protocol (HTTP/1.1): Caching”

[RFC7235]		IETF RFC 7235: “Hypertext Transfer Protocol (HTTP/1.1): Authentication”

[RFC7362]		IETF RFC 7362: “Latching: Hosted NAT Traversal (HNT) for Media in Real-Time Communication”

[RFC7540]		IETF RFC 7540: “Hypertext Transfer Protocol Version 2 (HTTP/2)”

[RFC7635]		IETF RFC 7635: “Session Traversal Utilities for NAT (STUN) Extension for Third-Party Authorization”

[RFC8200]		IETF RFC 8200: “Internet Protocol, Version 6 (IPv6) Specification”

[RFC8259]		IETF RFC 8259: “The JavaScript Object Notation (JSON) Data Interchange Format”

[RFC8441]		IETF RFC 8441: “Bootstrapping WebSockets with HTTP/2”

[RFC8446]		IETF RFC 8446: “The Transport Layer Security (TLS) Protocol Version 1.3”

[RFC8825]		IETF RFC 8825: “Overview: Real-Time Protocols for Browser-Based Applications”

[RFC8829]		IETF RFC8829: “JavaScript Session Establishment Protocol (JSEP)”

[RFC8445]		IETF RFC 8445: “Interactive Connectivity Establishment (ICE): A Protocol for Network Address Translator (NAT) Traversal”

[RFC8656]		IETF RFC 8656: “Traversal Using Relays around NAT (TURN): Relay Extensions to Session Traversal Utilities for NAT (STUN)”

[RFC9114]	IETF RFC 9114: “HTTP/3”

[RFC9220]	IETF RFC 9220: “Bootstrapping WebSockets with HTTP/3”

[OpenAPI]		OpenAPI Initiative “OpenAPI Specification v3.0.0” https://spec.openapis.org/oas/v3.0.0

[AsyncAPI]	AsyncAPI Initiative “AsyncAPI Specification v2.4.0” https://asyncapi.com/docs/specifications/v2/4/0

13	Open Issues

13.1	Materials for Further Study

a) The C-plane signalling protocol should support basic WebRTC service operations such as client registration, authentication and authorization; call control; and data channel management that are relevant to the new architecture.

c) Security considerations for interoperable WebRTC services such as authentication, authorization, and key management

d) Deployment options of traditional WebRTC functions in 5G network, and mapping of those functions to 5G media architecture

NOTE: Mapping of WebRTC functions to 5GMS functions to be confirmed in 5GAREA study

e) Feasibility to use existing 5GMS architecture enablers for betterment of WebRTC services.

13.2	Candidate Use Case

Editor’s note: 
Media handling remains unidentified, but the use case of one-to-one communication is to be included in clause 3 by some form. The following description is suggested by CMCC.

3D Telepresence is a real-time bidirectional video communication system that enables two people to communicate at distance as if they were physically together. Using the benefits of WebRTC, we can combine 3D Telepresence with a regular video call service. It offers users a more natural and intuitive way for remote connection.

3D Video Call establishes a one-on-one communication between users to guarantee low latency and high security holographic communication experience. The 3D video stream and the audio stream are being compressed and real-time transmitted over WebRTC protocol. Both ends support incoming and outgoing calls.

For example, Alice sits in a conference room in Midtown Manhattan, there are cameras, microphones, and sensors to capture her image, voice and movements from multiple vantage points. The captured imagery and voice get transmitted in real-time to the remote peer Bob, who is working in his studio in Boston. Both of them can see each other’s imagery in three dimensions and also hear the voice. They can talk naturally with the full range of communication cues, such as eye contact, hand gestures, and body languages. The whole experience makes them feel like they’re actually sitting in the same room.

Annex <A>:
Architectural WebRTC Entity Examples

Editor’s Note: 
Architectural example of integration of WebRTC with 5G network

Annex <B>:
Protocol Stack Examples

Editor’s Note: 
Definite example of C-plane protocol stack
Reference of U-plane (other TS/TRs) and supplemental explanation

Annex <C>:
WebRTC Signalling Protocol Examples

Editor’s Note: 
Expected signalling regulation examples (Async API)

Annex <D>:
WebRTC Signalling Flow Examples

Editor’s Note: 
Sequence and message examples using Annex C

Annex <E>:
Conference Management Protocol Examples

Editor’s Note: 
Examples of conference session management (OpenAPI)

Annex <F>:
Conference Management Flow Examples

Editor’s Note: 
Sequence and message examples using Annex E

Change History:

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2022-04 | SA4#118 |  |  |  | Initial version |  | 1.0.0 |
| 2022-05 | SA4#119 | S4-220516 |  |  | This version contains the changes of agreed SA4#118 |  | 2.0.0 |
| 2022-06-01 | SA4#119 | S4-220777 |  |  | This version contains the changes of agreed SA4#119 (S4-2207585, S4-220778) Concepts included or merged (S4-220780, S4-220781) |  | 2.0.1 |
| 2022-06-01 | SA4#119 post |  |  |  | Editorial change in reference clause |  | 2.0.2 |
| 2022-08-03 | SA4#120 | S4-220925 |  |  | This version contains the changes of agreed during SA4#119 (S4a-220006, S4a-220007, S4a-220019) Updated references to RFC 9114 and 9220 |  | 3.0.0 |