Agenda Item:		10.5

Source:		Facebook Japan K.K. (Rapporteur)

Title:			iRTCW Permanent Document

Version:		0.14

Document for:	Discussion

This document includes texts, figures, or other information that may complement TS 26.113 or be included later in the specification. Each section is related to a corresponding section in TS 26.113.

Introduction

4. System description

5. Functional components

5.1	3D video capture

Resolutions and rates of RGBD frames would influence the overall quality and their minimum required (or recommended) values may be specified for iRTC & other RTC applications. FoVs (horizontal and vertical) of RGBD frames, typically depending on the number and location (distance) of sensors, may also be specified.

5.1.1	Depth formats & point cloud generation

Depth information is typically stored in 16-bit unsigned integer format, which indicates the distance between each point and a vertical plane including the depth sensors, which typically consist of an (IR) transmitter and one or more receivers, in millimeter [1], [2]. Each point in 3D space is mapped onto a 2D image plane via a series of transforms illustrated below.

Mapping of 3D points to 2D image plane [3]

In the figure, [R t]T represents the rotation and translation from a 3D world coordinate system to a 3D camera’s coordinate system, whose parameters can be supplied by UE’s motion sensors. K is the camera intrinsic matrix defined as

where

| Parameter | Unit | Definition | Note |
| --- | --- | --- | --- |
| fx | float | X-axis focal length (in pixel) |  |
| fy | float | Y-axis focal length (in pixel) |  |
| cx | float | X-axis principle point (in pixel) |  |
| cy | float | Y-axis principle point (in pixel) |  |
| s | float | Skew coefficient | Zero if image axes are perpendicular |

Depending on notation, transpose of K is also used as a camera intrinsic matrix. These device-specific information is supplied by typical OSs used in UEs [4], [5], which assume s=0. Therefore a point cloud in e.g., PLY format can be generated from a pair of RGB and D frames by reversing the transforms, with parameters available to applications in UEs. The FoVs of RGB and D cameras will be in general different and how to align them is left to the discretion of the implementation.

5.1.2	Size measurement of 3D Objects

In [6], the needs for scaling 3D objects were illustrated for several scenarios. The conference managing server or receiving iRTC client needs the size information of 3D objects for a proper scaling. How to define or estimate the size of 3D (point cloud) objects, e.g., by recycling some information gathered in the generation of point cloud or using a separate set of sensors, has yet to be discussed.

There are smartphone applications that estimate the length, width and height, or volume of objects seen through the viewfinder. The measurement in general depends on the types of cameras used. These applications typically place a box or cube around an object and show the measured values. If the cube around a 3D object is set to a minimum, then the size of the cube may be used as an estimate of the object’s size, as illustrated below:

Camera-based measurement applications [7], [8]

Considering that typical depth cameras represent the depth information in millimeter [9], the size information may be similarly represented as

| Parameter | Unit | Definition | Note |
| --- | --- | --- | --- |
| w | Int | Width of cube (in mm) |  |
| h | Int | Height of cube (in mm) |  |
| d | int | Depth of cube (in mm) |  |

The size information can be signalled to a far-end iRTC client or conference managing server for scaling the 3D object to other objects or backgrounds. The size information may be transmitted periodically or in an on-demand fashion, depending on applications, and may also be used locally.

The error in the size estimation may be reduced by following common rules for placing boxes or cubes during initial measurements. A cube may not always be an optimal form to contain a point cloud captured with a limited number of RGB or D sensors of UEs, which may also depend on the FoVs of the cameras.

Editor’s Note: This will be discussed also with Video SWG.

5.2	Microphone description

A microphone array description is provided in [10] and further extended below as an example. The sending or receiving iRTC client, or audio infra may identify the direction of each microphone from a set of angles and a type (e.g., pick up pattern). Alternatively they may identify the position of each microphone from a set of cartesian coordinates.

In many spatial audio formats, a microphone processing block converts raw signals from microphones into a device-independent representation.

The angles are expressed in units of 1/10000 radians. For example, 3.1416 () radians is expressed as 31416.

| Parameter | Unit | Definition | Note |
| --- | --- | --- | --- |
| Yaw (𝛹, psi) | int | Direction angle | -31416 < 𝛹 ≦ 31416 |
| Pitch (𝛳, theta) | int | Elevation angle | -31416 < 𝛳 ≦ 31416 |
| Roll (𝛷, phi) | int | Rotation angle | -31416 < 𝛷 ≦ 31416 |
| MicrophoneType | int | A number that uniquely identifies microphone type | May be used for indicating vendor-defined types |

These parameters can describe the orientation of each microphone relative to a camera, and its type. The rotation angle, not defined in [10], can be used to indicate the rotation of a directional microphone whose pick up pattern is not axisymmetric. It is assumed that the RGB or D cameras for capturing 2/3D video are located on a plane parallel to the ZY plane, pointing objects in the direction of X axis.

Although typical UEs (at the time of writing this contribution) have 2-3 microphones, they are rarely located on the same plane. Usually a microphone is located on top of a display while another is located at the bottom of UE. Sometimes a microphone is located on the other side of display (with main cameras). Therefore the Linear or Planar mode [10], designated with the wMicArrayType parameter, would not be applicable.

In some contemporary UEs, e.g., foldable devices, the direction or elevation angle may be different in each session or even vary during a session, and the sending or receiving iRTC client, or audio infra may need to be informed of such changes for proper rendering. In addition to the geometric description, if applicable, other or additional information may be beneficial to the microphone processing, e.g.:

- acoustic overload point (AOP)

- noise floor (may be difficult to signal)

- total harmonic distortion (THD)

- microphone sensitivity.

Editor’s Note: Need for transmitting this information will be discussed and studied further.

6. Session management

7. Packet-loss handling

8. Adaptation

9. Architecture and function

9.1	WebRTC QoS architecture

Collaboration scenarios.

9.1.1	5G support for OTT WebRTC

In this scenario, the application provider offers a WebRTC service to their customers and is responsible for the security and privacy of the data exchanged over their service. The application provider desires to improve the quality of the service for mobile users.

The collaboration scenario is similar to the 5GMS streaming model, where all involved Application Servers (AS) are in non-trusted domain.

The following figure depicts this collaboration scenario:

    - MNO-provided trusted WebRTC functions

In this collaboration scenario, the user utilizes a variety of WebRTC-based conferencing services for personal and work purposes. The user is concerned about their privacy, e.g., through threats from man-in-the-middle attacks that result from usage of untrustworthy ICE functions. the user opts for using a trusted WebRTC configuration, that is provided by their MNO, which offers ICE functions such as STUN, TURN, MCU, etc.

In addition, the MNO will offer the required traffic handling to the WebRTC sessions, from some or all application providers, based on existing SLAs or based on user contractual agreements.

The following figure depicts this collaboration scenario:

    - MNO-facilitated WebRTC services

The MNO may offer several WebRTC-based services such as remote gaming, AR telephony, etc. These services are facilitated and managed by the MNO and are offered to the MNO’s customers exclusively.

All WebRTC functionality is hosted by the MNO, which ensures the quality of the services through appropriate network assistance, such as QoS allocation.

The following figure depicts this collaboration scenario:

    - Inter-operable WebRTC services

As an extension of previous collaboration scenario, a globally inter-operable WebRTC service is provided, where mobile users from different MNOs are able to join the same service and benefit from the 5G system support for better end-to-end quality of service. In this collaboration scenario, the WebRTC functions are hosted by one or more MNOs.

The following figure depicts this collaboration scenario:

  - WebRTC functions in the context of iRTCW

## 9.2.1	General

We have agreed on 4 different collaboration scenarios for iRTCW at the SA4-118e meeting. The collaboration scenarios are listed here for convenience:

- 5G support for OTT WebRTC: in this scenario the WebRTC session runs completely over the top. However, the MNO may offer support in form of QoS allocation, bitrate recommendations, and QoE report collection based on request by the UE.

- MNO-provided trusted WebRTC functions: in this scenario the MNO offers trusted support functions such as ICE servers to the WebRTC application on the UE.

- MNO-facilitated WebRTC services: the MNO may host and facilitate WebRTC sessions by providing a trusted WebRTC signaling server, which may also offer 5G network assistance.

- Inter-operable WebRTC services: collaboration scenario 3 is extended with functions to support MNO to MNO inter-operability.

Based on the documented collaboration scenarios, we identify the following functions and describe their roles.

## 9.2.2	Potential 3GPP-defined functions

9.2.2.1	General

These functions will be discussed and possibly confirmed in the context of the architecture that will be defined by the 5G_AREA WI.

9.2.2.2	Provisioning server

The provisioning server may enable an application provider to perform provisioning of the following functionalities:

- QoS support provisioning for WebRTC sessions

- Charging provisioning for WebRTC sessions

- Collection of consumption and QoE metrics data provisioning related to WebRTC sessions

- Offering ICE functionality provisioning such as STUN and TURN servers

- Offering WebRTC signaling servers provisioning, potentially with interoperability to other signaling servers.

The provisioning server may not be relevant to all collaboration scenarios and some of the 5G support functionality may be offered without application provider provisioning.

### 9.2.2.3	Configuration server

The configuration server stores WebRTC-related configuration information and makes them accessible to the UE. It stores information and recommendations to operate network-assisted WebRTC sessions over 5G.

The configuration information may consist of static information such as the following:

- Recommendations for media configurations

- Configurations of STUN and TURN server locations

- Configuration about consumption and QoE reporting

- Discovery information for WebRTC signaling and data channel servers and their capabilities.

### 9.2.2.4	Media session handler (MSH)

The MSH is an entity running on the UE, which assists with the 5G integration of the WebRTC application. It exchanges, on behalf of the application, information about the WebRTC sessions with the network.

The MSH receives information about a new WebRTC session from the application. It relays the information to the Support Function. It also receives events and other network information about the WebRTC session from the Support Function, which it may relay to the application.

### 9.2.2.5	Network support function

The support functionality includes the following:

- Network Support Function receives information about a WebRTC session and its state

- Network Support Function requests QoS allocation for a starting or modified session

- Network Support Function receives notification about changes to the QoS allocation for the ongoing WebRTC session

- Network Support Function exchanges information about the WebRTC session with the trusted STUN/TURN/Signaling Server, e.g. to identify a WebRTC session and associate it with a QoS template.

## 9.2.3	WebRTC functions

### 9.2.3.1	Trusted ICE functions

The MNO may offer trusted ICE functions to the WebRTC application to be used during the WebRTC ICE gathering phase. These functions may be STUN and TURN servers that facilitate NAT and Firewall traversal.

The MNO-operated trusted ICE functions may assist with the 5G integration of the WebRTC application. This could be done by triggering network assistance to starting or ongoing WebRTC sessions.

## 9.2.4	iRTCW-defined functions

### 9.2.4.1	Trusted WebRTC signaling server

The trusted WebRTC signaling server is used to setup and manage MNO-operated WebRTC applications. They offer a standardized signaling protocol for the session setup to both parties of the WebRTC session. The WebRTC signaling server will handle the offer/answer exchange and will have access to the SDP in both directions.

The WebRTC signaling server may use that knowledge to offer network assistance and other 5G features to the endpoints of the WebRTC session.

9.2.4.2	Inter-working function

This function provides inter-working functionality to enable MNO-facilitated WebRTC sessions that involve end-points across different MNOs. They may for example provide cross-network signaling functionality to allow WebRTC signaling server that are hosted in different networks to communicate, in order to establish and manage the WebRTC sessions.

### 9.2.4.3	Trusted media server

A media server may be offered by the MNO to support WebRTC sessions. It may offer a wide range of functionality such as:

- a content server that serves content to the WebRTC application, e.g. through a data channel

- media processing functionality: may be used by the WebRTC application as a relay that performs some media processing function such as transcoding, recording, 3D reconstruction, etc.

- scene composition functionality: the server may compose a 3D scene and distribute it to several point-to-point WebRTC sessions

- MCU functionality: the server may offer multi-party conferencing functionality to merge a number of point-to-point WebRTC sessions

- SFU (Selective Forwarding Unit) functionality: the server may offer the selection, copy, and forwarding functionality of IP steams produced by multiple WebRTC endpoints (i.e., participants).

# Mapping to collaboration scenarios

The following table provides an initial mapping of the identified functions to the collaboration scenarios:

| Functions/CS | Collaboration scenario 1 | Collaboration scenario 2 | Collaboration scenario 3 | Collaboration scenario 4 |
| --- | --- | --- | --- | --- |
| Provisioning server | Optional | Optional | Optional | Optional |
| Configuration server | Optional | Required | Required | Required |
| MSH | Required | Optional | Optional | Optional |
| Network support function | Required | Required | Optional (maybe fulfilled by WebRTC signaling server) | Optional |
| Trusted ICE function | N/A | Required | Optional | Optional |
| Trusted WebRTC signaling server | N/A | N/A | Required | Required |
| Trusted media server | N/A | Optional | Optional | Optional |

References

[1] Azure Kinect DK hardware specifications, Microsoft

[2] Intel RealSense D400 series product family datasheet, Intel

[3] Camera calibration parameters, MathWorks

[4] intrinsicMatrix, Apple

[5] CameraIntrinsics, Google

[7] Quick measure, Samsung

[8] Measure app, Apple

[9] S4aR220014, 3D video capture description for iRTC client in the terminal, Meta

[10] Windows microphone array geometry descriptor format, Microsoft

# Revision history

| Date | Meeting | Subject / Comment | Old | New |
| --- | --- | --- | --- | --- |
| 2022-05-20 | SA4#119-e |  | 0.10 | 0.11 |
| 2022-08-26 | SA4#120-e |  | 0.11 |  |