3GPP TSG SA WG4 #120-e	                                                                              SA4-220947

E-meeting, 17 – 26 August 2022

Source:	vivo

Title:	[FS_XRTraffic] Discussion on XR Traffic Model

Agenda Item:	9.6

Document for:	Discussion and Decision

- Introduction

In RAN plenary #94e meeting, the WID for Rel-18 XR enhancement was approved [1], and the objectives are shown as follows:

The study is to be based on Release 17 TR 38.838, on corresponding Release 17 work from SA4 (as per SP-210043) and on Release 18 work from SA2 (as per SP-211166). Objectives on XR-awareness in RAN (RAN2): - Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of. - Study how the above information aids XR-specific traffic handling. Objectives on XR-specific Power Saving (RAN1, RAN2): - Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques: - C-DRX enhancement. - PDCCH monitoring enhancement. Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements; - Dynamic scheduling/grant enhancements.

In SA4, XR traffic characteristics are being studied in FS_XRTraffic SI and further conclusions on XR traffic characteristics are captured in TR 26.926[2]. We notice that the XR video traces and configurations files in SA4 study are updated and captured in the latest TR 26.926 agreed in SA4 #117-e [3].

In this contribution, we provide our views on XR traffic model based on the latest TR 26.926 [2].

- Discussion on Packet size

In R17 XR study, a packet models the set of IP packets belong to the same video frame. The size of a packet is determined by the given data rates and frame rates, which is modelled as a random variable following truncated Gaussian distribution, as shown in Table 5.1.1.5-1 in TR 38.838 [4].

Table 5.1.1.5-1: Statistical parameter values for dual eye buffer packet size (TR 38.838)

| Parameter | unit | values for evaluation | Optional values for evaluation |
| --- | --- | --- | --- |
| Mean: M | byte | R×1e6 / (2×F) /8 | R×1e6 / (2×F) / 8 |
| STD | byte | 10.5% of M | 4% of M |
| Max | byte | 150% of M | 112% of M |
| Min | byte | 50% of M | 88% of M |
| R: data rate of the flow in Mbps F: frame generation rate of the flow in fps |  |  |  |

We did some analyses based on the traces and configurations files according to the latest permanent document of TR26.926 [3]. Note that in the analysis a packet denotes the set of IP packets corresponding to a frame.

Figure 1 and Table 1 shows the packet size distribution for the P-trace according to SA4’s input. It can be seen that the packet size distribution based on the latest P-trace from SA4 are similar the packet size model in R17 XR study.

Observation 1: As per the latest permanent document of TR26.926 agreed in SA4 #117e, the packet size distribution is similar to that for R17 XR study. The traffic model with packet size distribution as described in TR 38.838 can be reused for R18 XR study

Figure 1. The statistic of packet size by using the P-trace data from SA4

Table 1. Packet distribution deriving from the latest P-trace files provided by SA4

| Parameter | VR 2-1 | VR 2-2 | VR 2-3 | VR 2-4 | VR 2-5 | VR 2-6 | VR 2-7 | VR 2-8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bitrate (Mbit/s) | 30 | 30 | 30 | 30 | 30 | 30 | 45 | 45 |
| Mean (byte) | 7378 | 7211 | 7640 | 7467 | 6699 | 3689 | 10506 | 10265 |
| STD (byte) | 560 | 543 | 176 | 173 | 847 | 296 | 1574 | 1530 |
| Min (byte) | 3531 | 3488 | 7115 | 6974 | 1522 | 1719 | 3544 | 3480 |
| Max (byte) | 9224 | 9117 | 8164 | 7996 | 13106 | 4836 | 13523 | 13147 |
| STD/Mean | 7.60% | 7.53% | 2.30% | 2.32% | 12.64% | 8.02% | 14.99% | 14.91% |
| Min/Mean | 47.86% | 48.37% | 93.14% | 93.40% | 22.72% | 46.61% | 33.74% | 33.90% |
| Max/Mean | 125% | 126% | 107% | 107% | 196% | 131% | 129% | 128% |

- Discussion on Jitter

The jitter model for R17 XR study was discussed in RAN1 #104-e and RAN1 #104bis-e meetings.  During the discussions, jitter was analysed by companies based on the trace and configuration files provided in the LS from SA4 #111e-meeting [5]. The jitter distribution for R17 XR study derived from P-trace files by SA4 (S4aV200634 [6]) is shown in the Table 2.

Table 2. Jitter distribution for R17 XR study derived from P-trace files by SA4 (S4aV200634)

|  | VR 2-1 | VR 2-3 |
| --- | --- | --- |
| Mean (ms) | 0 | 0 |
| STD (ms) | 2.77 | 2.76 |
| Min (ms) | -5.39 | -5.39 |
| Max (ms) | 5.08 | 5.45 |
| Jitter range (ms) | [-4.1, 4.3] | [-4.1, 4.43] |
| jitter range = [5%-tile in CDF, 95%-tile in CDF] ms |  |  |

According to the analysis on the traces from SA4, for R17 XR evaluation, the jitter is modelled as truncated Gaussian distribution with statistical parameters shown in Table 5.1.1.2-1 in TR 38.838 [4]. Note that the varying frame encoding delay and network transfer time introduce jitter in packet arrival time at gNB.

Table 5.1.1.2-1: Statistical parameters for jitter (TR 38.838)

| Parameter | unit | Baseline value for evaluation | Optional value for evaluation |
| --- | --- | --- | --- |
| Mean | ms | 0 |  |
| STD | ms | 2 |  |
| Truncation range | ms | [-4, 4] | [-5, 5] |

In R18 XR SI, XR specific power saving and capacity improvement techniques to accommodate XR traffic characteristics including jitter will be studied. It is necessary to take into account the realistic XR traffic characteristics to study XR-specific techniques. According to the R18 XR SID, jitter is one of the important issues for XR study, since many of the XR and CG use cases are characterised by non-integer periodicity traffic.

In SA4, XR traffic characteristics are being studied in FS_XRTraffic SI and further conclusions on XR traffic characteristics are captured in TR 26.926. Similar to R17 XR study, SA4 study on XR traffic characteristics can be the starting point for R18 XR study. We notice that the XR video traces and configurations files in SA4 study are updated and captured in the latest TR 26.926 agreed in SA4 #117-e [3].

We did some analyses based on the traces and configurations files according to the latest permanent document of TR26.926 [3]. Figure 2 shows the timing of V-trace, S-trace and P-trace according to SA4’s input.

Figure 2. Example of timing of V-trace, S-trace and P-trace

- Point A: Time stamp of a frame in V-trace

- Point B: Time stamp of the first slice for the frame in S-trace

- Point C: Time stamp of the last slice for the frame in S-trace

- Point D: Time stamp of the first IP packet for the frame in P-trace

- Point E: Time stamp of the last IP packet for the frame in P-trace

Figure 3 shows the distribution of encoding delay, i.e., Point C – A for each frame, and Table 3 shows the jitter distribution caused by encoding. It can be observed that from Table 3, the encoding delay is varying from around [-9 ms, +9 ms], depending on the configurations.

Figure 3.The statistic of encoding delay by using the P-trace data from SA4

Table 3. Encoding delay jitter distribution deriving from the latest P-trace files provided by SA4

| Parameter | VR 2-1 | VR 2-2 | VR 2-3 | VR 2-4 | VR 2-5 | VR 2-6 | VR 2-7 | VR 2-8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mean (ms) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| STD (ms) | 5.78 | 5.80 | 5.80 | 5.79 | 5.89 | 5.79 | 5.80 | 5.78 |
| Min (ms) | -11.46 | -11.17 | -11.34 | -10.98 | -13.94 | -11.49 | -11.17 | -11.54 |
| Max (ms) | +11.33 | +11.64 | +11.26 | +11.34 | +13.21 | +11.77 | +11.48 | +11.18 |
| Jitter range (ms) | [-8.94, +9.01] | [-9.04, +8.99] | [-8.99, +9.02] | [-8.97, +9.03] | [-9.17, +9.16] | [-9.02, +9.00] | [-8.98, +9.02] | [-9.02, +8.98] |
| jitter range = [5%-tile in CDF, 95%-tile in CDF] ms |  |  |  |  |  |  |  |  |

Figure 4 shows the distribution of network transfer delay, i.e., Point E – C for each frame, and Table 4 shows the jitter distribution caused by network transfer. It can be observed that from Table 4, the network transfer delay is varying from around [-4 ms, +7 ms] depending on the configurations.

Figure 4. The statistic of network transfer by using the P-trace data from SA4

Table 4. Network transfer jitter distribution deriving from the latest P-trace files provided by SA4

| Parameter | VR 2-1 | VR 2-2 | VR 2-3 | VR 2-4 | VR 2-5 | VR 2-6 | VR 2-7 | VR 2-8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mean (ms) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| STD (ms) | 2.72 | 2.65 | 2.75 | 2.66 | 2.90 | 3.94 | 2.87 | 2.75 |
| Min (ms) | -6.89 | -6.73 | -3.87 | -3.64 | -8.77 | -4.46 | -7.33 | -7.05 |
| Max (ms) | +14.49 | 13.33 | 13.52 | 13.20 | 16.65 | 16.19 | 16.56 | 14.02 |
| Jitter range (ms) | [-2.17, +6.43] | [-2.09, +6.37] | [-2.04, +6.58] | [-1.96, +6.42] | [-4.02, +6.19] | [-3.43, +8.43] | [-3.83, +6.46] | [-3.63, +6.21] |
| jitter range = [5%-tile in CDF, 95%-tile in CDF] ms |  |  |  |  |  |  |  |  |

Figure 5 shows the distribution of delay from the frame generation time to the time when its last IP packet arrives at the RAN side, i.e., Point E – A for each frame, and Table 5 shows the corresponding distribution of jitter. It can be observed that based on the latest trace files, the maximum jitter is approximately ranging from -18 ms to +18 ms depending on the configurations, as shown in Appendix A. Considering the range truncated by 5%-tile and 95%-tile in CDF, the stable jitter range is around [-8 ms, +8ms], which is larger than the jitter range (i.e. [-4ms, +4ms]) agreed in R17 XR SI.

It can be seen the jitter range derived according to the latest P-trace files is different from that used for R17 XR study derived from P-trace files by SA4 (S4aV200634 [6]). By further checking the trace and configuration files, it seems there are different parameters in the configurations, which may result in the differences between the latest P-trace and the previous ones. The following shows an example of the comparison between the configurations files of VR2-1 for S4aV200634 and the latest TR 26.926. The trace and configuration files are download from the following link.

http://dash.akamaized.net/WAVE/3GPP/XRTraffic/Traces/Candidate/

| Recommended Configurations in S4aV200634 | Configurations extracted from the trace file |
| --- | --- |
| "Bitrate": { "mode": "cVBR", "bitrate": "30000000", "window_framerate": "12", "QPmin": 0, "QPmax": 53 }, "ErrorResilience": { "mode": "pIntra", "parameter": "1" }, "Slice": { "mode": "fixed", "parameter": "8" }, "PreDelay": { "mode": "equally", "parameter1": "10", "parameter2": "20" }, "EncodingDelay": { "mode": "GaussianTrunc", "parameter1": "4", "parameter2": "3", "parameter3": "1" } | "Bitrate": { "mode": "cVBR", "bitrate": 30000000, "window_framerate": 12, "QPmin": 0, "QPmax": 51 }, "ErrorResilience": { "mode": "pIntra", "parameter": 1 }, "Slice": { "mode": "fixed", "parameter": 8 }, "PreDelay": { "mode": "equally", "parameter1": 10, "parameter2": 30 }, "EncodingDelay": { "mode": "GaussianTrunc", "parameter1": 8, "parameter2": 3 } |

We note that the traces from SA4 are generated by typical configurations. Hence, it is reasonable to consider the XR traffic characteristics based on the latest P-trace files.

As analysed above, jitter model in accordance with the latest SA4’s inputs in TR 26.926 [2] need be considered for R18 XR study, e.g., [-8ms, +8ms].

Figure 5.The statistic by using the P-trace data from SA4

Table 5. Jitter distribution deriving from the latest P-trace files provided by SA4

| Parameter | VR 2-1 | VR 2-2 | VR 2-3 | VR 2-4 | VR 2-5 | VR 2-6 | VR 2-7 | VR 2-8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mean (ms) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| STD (ms) | 4.96 | 5.00 | 4.85 | 4.89 | 5.31 | 4.02 | 5.19 | 5.18 |
| Min (ms) | -15.89 | -16.00 | -11.87 | -11.96 | -18.95 | -14.40 | -16.48 | -16.18 |
| Max (ms) | +11.27 | +10.30 | +10.10 | +10.51 | +18.69 | +8.08 | +11.59 | +11.13 |
| Jitter range (ms) | [-8.39, +7.89] | [-8.42, +7.91] | [-8.26, +7.69] | [-8.30, +7.77] | [-8.74, +8.42] | [-7.05, +6.24] | [-8.82, +8.19] | [-8.79, +8.20] |
| jitter range = [5%-tile in CDF, 95%-tile in CDF] ms |  |  |  |  |  |  |  |  |

Observation 1: SA4 has updated the XR video traces and configurations files which are captured in the latest TR 26.926 agreed in SA4 #117-e.

Observation 2: As per the latest permanent document of TR26.926 agreed in SA4 #117e, the variance of jitter extends to around 5ms and the jitter range (truncated by 5%-tile and 95%-tile in CDF) extends to [-8ms, +8ms].

Therefore, we have the following proposal to send an LS to RAN1/RAN2/SA2 to inform the updated XR traffic model. The draft LS to RAN1/RAN2/SA2 can be found in section 4.

Proposal 1: Send an LS to RAN1/RAN2/SA2 to inform the updated XR traffic model.

- Draft LS to RAN1/RAN2/SA2 on XR traffic model

Title:	[Draft] LS to RAN1/RAN2/SA2 on XR traffic model

Response to:	-

Release:	Rel-18

Work Item:	FS_NR_XR_enh

Source:	[SA4]

To:	RAN1, RAN2, SA2

Cc:

# Contact Person:

# Name:	              TBD

E-mail Address:	 TBD

# 1. Overall Description:

3GPP TSG SA WG4 would like to inform 3GPP TSG RAN WG1/WG2 and SA WG2 on the recent progress for the modelling of XR Traffic for the study of XR.

SA4 has carried out work for the modelling of XR traffic characteristics including jitter with the detailed modelling provided in the attached documents, namely

- S4-220047 [FS_XRTraffic] Proposed Updated TR26.926

- S4-211210 FS_XRTraffic: Permanent document, v0.8.0

# 2. Actions:

# To RAN1/RAN2/SA2

# To take the above information into account

# 3. Date of Next SA4 Meetings:

3GPP SA4#121	 		11 – 14 November 2022

3GPP SA4#122	 		20 – 24 February 2023

- Conclusion

In this contribution, we provide our views on XR traffic model based on the latest TR 26.926. The observations and proposals:

# References

- RP-213587, Study on XR Enhancements for NR, RAN #94e, Dec. 6 - 17, 2021.

- 3GPP TR 26.926 V1.1.0 (2022-02)

- S4-220047, [FS_XRTraffic] Proposed Updated TR26.926, Qualcomm Incorporated, SA4 #117e, Feb. 14 – 23, 2022

- 3GPP TR 38.838 V17.0.0 (2021-12), Study on XR (Extended Reality) Evaluations for NR (Release 17)

- R1-2101765, LS to on XR-Traffic Models, SA4 #111e, Nov.11-20, 2020

- S4aV200634, 	[FS_XRTraffic] Traces and Configurations for VR2, CG and AR2, Qualcomm Incorporated, Mediatek

# Appendix A –VR2: XR Split Rendering

The following is a quote from the latest permanent document of TR26.926 agreed in SA4 #117e [3].

| Configuration | Basic Content Parameters |
| --- | --- |
| VR2-1 | 8 slices per eye buffer, 1 slice per frame is intra coded, 30Mbit/s capped VBR with window 200ms, buffer sent at same time, 1500 byte max packet size |
| VR2-2 | 8 slices per eye buffer, 1 slice per frame is intra coded, 30Mbit/s capped VBR with window 200ms, buffer sent at same time, unlimited packet size |
| VR2-3 | 8 slices per eye buffer, 1 slice per frame is intra coded, 30Mbit/s CBR with window 1 frame, buffer sent at same time, 1500 byte max packet size packets |
| VR2-4 | 8 slices per eye buffer, 1 slice per frame is intra coded, 30Mbit/s CBR with window 1 frame, buffer sent at same time, unlimited packet size |
| VR2-5 | 1 slice per eye buffer, every 8th frame is intra coded, 30Mbit/s capped VBR with window 200ms, buffer sent at same time, 1500 byte max packet size |
| VR2-6 | 8 slices per eye buffer, 1 slice per frame is intra coded, 30Mbit/s capped VBR with window 200ms, buffers sent interleaved, 1500 byte max packet size |
| VR2-7 | 8 slices per eye buffer, 1 slice per frame is intra coded, ++/*45Mbit/s capped VBR with window 200ms, buffer sent at same time, 1500 byte max packet size |
| VR2-8 | 8 slices per eye buffer, 1 slice per frame is intra coded, 45Mbit/s capped VBR with window 200ms, buffer sent at same time, unlimited packet size |