Agenda item: 	10.5

Source: 	Intel

Title: 	Discussion on the usage of 5GMS for iRTCW

Document for	Discussion and Agreement

# Introduction

This contribution discusses the usage of using 5GMS [1] architecture for WebRTC signaling and data channel. It is partially addressing objects 5 and 6 of SP-220214 [2].

# 5GMS for iRTC signaling and data channel.

In a nutshell, WebRTC communication requires two major components: 1) a signaling/control plane and 2) a transport/data plane. The basic design principle of WebRTC is to enable Peer-to-Peer communication without the intervention of the media-aware middlebox. The general guideline is that participants should be minimum depending on various considerations such as UE capability, media codecs, etc.  However, scalability can be achieved when media-aware middleboxes are in place such as an MCU/SFU.

The 5GMS defined media-streaming architecture for both uplink and downlink streaming. A 5GMS-aware application is enabled to utilize the M5 interface for media session handling and the M4 interface for streaming transport handling. This contribution is trying to identify the possible usage of 5GMS for iRTC signaling and streaming based on 5GMS existing interfaces.

We have identified that 1) 5GMS AF may be used as a WebRTC signaling control component 2) 5GMS AS may be used as a media server for WebRTC uplink and downlink streaming.

Two new interfaces are introduced for WebRTC control plane establishment.

- RTC-1: an interface between AF and WebRTC signaling server.

- RTC-2: an interface between the WebRTC signaling server and the STUN/TURN server.

Note: The assumption is STUN/TURN server is a single entity.

Current PD has identified four WebRTC Collaboration scenarios. In this contribution, we dissect each collaboration mode and introduce the basic workflow using 5GMS for WebRTC data and control plane.

2.1 5G support for OTT WebRTC.

Refer to Figure 2-1, both WebRTC signaling and STUN/TURN service are offered by the WebRTC application provider. M5 interface in 5GMS may be used to establish a WebRTC control plane through AF using RTC-1 interface with a Signaling server.  RTC-2 is used between the signaling server and STUN/TURN for UE NAT traversal.

For example, in Figure 2-1, UE-1 registers with PLMN-1. UE-2 and UE-3 registers with PLMN-2. UEs use their respective 5GMS AF to establish WebRTC signaling and control plane establishment with the WebRTC signaling server through the RTC-1 interface.

After the control plane has been established, UE may request a PDU session through 5GMS AF with PCF using the N5 interface.

Assuming the PDU session is created successfully and 5QI has been assigned to UE,gNB and UPF, the data plane may go through m4 interface, which is the PDU session between UE and UPF.

UE-1 may start streaming its WebRTC session using the m4u interface. Media data may be stored and hosted in 5GMS AS.  UE-2 and UE-3 may use the m4d interface to download UE-1’s WebRTC media data through AS.

Figure 2-1 5G Support for OTT WebRTC

2.2 MNO-provided trusted WebRTC functions

Refer to Figure 2-2, the WebRTC signaling service is offered by the WebRTC application provider. STUN/TURN service is offered through individual PLMN. The WebRTC signaling process is similar to clause 2.1.

Figure 2-2 MNO-provided trusted WebRTC Functions

2.3 MNO-facilitated WebRTC services

Refer to Figure 2-3, both WebRTC signaling, and STUN/TURN service are offered by the PLMN. In this case, all UEs are registered to the same PLMN. The WebRTC signaling process is similar to clause 2.1.

Figure 2-3 MNO-facilitated WebRTC services

2.4. Inter-operable WebRTC services

Refer to Figure 2-4, both WebRTC signaling, and STUN/TURN service are offered by the PLMN.

In this case, UE-1 is registered with PLMN-1. UE-2 and UE-3 are registered with PLMN. The WebRTC signaling process is similar to clause 2.1.  In order to support inter-operability among different PLMN, a 5GMS AS entity may be deployed in a WebRTC application provider in order for UEs from different PLMN to upload and download their media traffic.

Figure 2-4 Inter-operable WebRTC services

For each above-mentioned use case, there is a need to:

- Identify the API and parameters necessary for RTC-1 between 5GMS AF and WebRTC signaling server.

- Identify the API and parameters necessary for RTC-2 between the WebRTC signaling server and STUN/TURN server.

# Proposal

We propose to agree and document the 5GMS use cases for the WebRTC signal and data channel mentioned in clause 2 into clause 9 of PD.

# References

[1] TS 26.501 v17.1.0, 5G Media Streaming (5GMS); General description and architecture

[2] SP-220241, Draft New WID on immersive Real-time Communication for WebRTC