Source:	Samsung Electronics Co., Ltd.

Title:		[FS_AI4Media] Basic service architecture for split inference with uplink

Agenda Item:	9.7

Document for:	Agreement

# 1 Introduction

# From previous post SA4 119-e meeting Video SWG adhoc telcos, it was commented that there are different possible configurations for split inferencing between the UE and network. This contribution provides an additional service architecture to reflect the scenario where a UE uplink connection is required during the split inferencing media service.

# 2 Service architecture for split inference with uplink

### 4.2.2	Split AI/ML operation

[

# Figure 4.2.2-1: Service architecture for split inference between the network and UE, with data source in the network

# Figure 4.2.2-1 shows a simple service architecture for split inferences between the network and the UE, as described in scenario 2b) of clause 4.2, where the data source is in the network, and the ordering of split inference is first perform in the network, then on the UE.

# For the split inference (network - UE) scenario, additional components are required:

# In the network:

- An AI model inference engine that receives both the network AI model subset(s), and input data, for network inference.

- An intermediate data delivery function receives the partial inference output (intermediate data) from the network inference engine, and sends it to the UE via the 5GS. This delivery function may also contain functionalities related to QoS requests and monitoring.

# In the UE:

- An intermediate data access function receives the intermediate data from the network via the 5GS, and sends it to the UE inference engine for UE inference.

- The final inference output data is sent to the data destination (e.g. a media player).

# Extra factors should be considered, including those such as:

- Configuration of the split inference between the network and UE. (e.g. definition and selection of the AI/ML model composition into “UE AI model subset” and “network AI model subset”)

- Resource allocation and management for network inference, including ingestion of network AI model data and media data

- Intermediate data delivery pipelines between the network and UE, in particular considering the use of 5GMS defined pipelines to stream intermediate data that is media content data.

- The functionalities of certain components in figure 4.2.1-1 and figure 4.2.2-1 may overlap, and depending on the use case a combined architecture may also be considered FFS.

- Certain components may also overlap with functions defined in 5GMS, clarifications FFS.

# ]

Figure 4.2.2-2: Service architecture for split inference between the UE and network, with data source in the UE

# Figure 4.2.2-2 shows a simple service architecture for split inferences between the UE and the network, as described in scenario 2b) of clause 4.2, where the data source is in the UE, and the ordering of split inference is first performed in the UE, then in the network. The resulting inference output data is then sent back to the UE.

# For the split inference (UE - network) scenario, additional components are required:

# In the UE:

- An AI model inference engine that receives both the network AI model subset(s), and input data (from a UE data source), for UE inference.

- An intermediate data delivery function receives the partial inference output (intermediate data) from the UE inference engine, and sends it to the network via the 5GS. This delivery function may also contain functionalities related to QoS requests and monitoring.

- An intermediate data delivery function receives the inference output data from the network via the 5GS, and sends it to the relevant data destination according to the AI media service.

# In the network:

- An intermediate data access function receives the intermediate data from the UE via the 5GS, and sends it to network inference engine for network inference.

- The final inference output data is sent to the UE via the 5GS, through the inference output delivery function.

# For both split inference scenarios, extra factors should be considered, including those such as:

- Configuration of the split inference between the network and UE. (e.g. definition and selection of the AI/ML model composition into “UE AI model subset” and “network AI model subset”)

- Resource allocation and management for network inference, including ingestion of network AI model data and media data

- Intermediate data delivery pipelines between the network and UE, in particular considering the use of 5GMS defined pipelines to stream intermediate data that is media content data.

- The functionalities of certain components in figure 4.2.1-1 and figure 4.2.2-1 may overlap, and depending on the use case a combined architecture may also be considered FFS.

- Certain components may also overlap with functions defined in 5GMS, clarifications FFS.

# 3 Proposal

We propose to include the edits in section 2 of this contribution to the relevant clause 4.2.2 of the latest version of the Permanent Document.