Source:	Xiaomi

Title:	Proposed updates to EDGAR-1 architecture

## Document for:	Agreement

## Agenda Item:	9.5

# 1	Introduction

At the 3GPP meeting #119-e, SA4 published the Permanent Document (PD) for MeCAR v2.0 [1]. During the post 119-e period, discussion on MeCAR took place during scheduled Video SWG telcos. This contribution proposes updates to the clause 4.2.1 Device architecture for EDGAR-1 device category based on these discussions and suggestions made during those telcos.

# 2	Proposal

# 2.1	General

The proposed changes are as follows:

- Thin arrows for data flows for clearer referencing in the future (from S4aV220894)

- Typos in diagram text (AR Scene Manager)

- Solid line for EDGAR-1 block

- Editorial improvements of the descriptive text (from S4aV220905)

- Avoiding the use of “last second adjustment”

# 2.2	Proposed updated clause

### 4.2.1	Device architecture

[Editor’s note] At SA4#119, this section was added while further improvements were improved.

Figure 2  provides the technical architecture of EDGAR-1 UE.

Figure 2 - Device architecture of EDGAR-1 device

The EDGAR-1 is regular 5G UE with 5G connectivity provided through an embedded 5G modem and 5G system components. The EDGAR-1 UE also features several sensors and user controllers relevant for AR experiences that are cameras, microphones, speakers, display and generic user input. The AR/MR Application is responsible for orchestrating the various device resources to offer the AR experience to the user. In particular, the AR/MR Application can leverage three main internal components on the device which are:

- The Media Access Functions (MAF)

- The AR Runtime

- The AR Scene Manager

The AR/MR Application can communicate with those three components via dedicated APIs called the MAF-API, the AR Scene Manager API and the AR Runtime API. Among other functionalities, those APIs enables the AR/MR Application to discover and query the media capabilities in terms of support as well as available resources at runtime. Regarding rendering, the AR/MR application obtains the head pose information from the AR Runtime which is then provided to the AR Scene Manager. Based on this information, the AR Scene Manager determines the objects visible to the user at a given point in time or more generally the objects that may be needed to be rendered in the next rendering cycles. Upon negotiation or predetermined configuration with the AR Runtime, the AR Scene Manager submits rendered views (sometimes called layers) to the AR Runtime. From those views, the AR Runtime generates the left and right eye buffers possibly based on late adjustment techniques using updated head pose information, if available, commonly known as late stage reprojection (LSR).

Once the AR/MR application is running, the downlink media flows from the 5G System to the MAF in compressed form and then from The MAF to the AR Scene Manger in a decoded form. In parallel, the EDGAR-1 UE is capable of establishing an uplink data flow from the AR Runtime to the MAF wherein the data may be in an uncompressed form and then from the MAF to the 5G System wherein the MAF may have compressed the data in order to facilitate the expected transmission over the network.

# References

- S4-220760, MeCAR Permanent Document v2.0, 3GPP TSG SA WG4 119-e Meeting, 11th – 12th May 2022