Source: 	InterDigital Inc.

Title: 	Interest of transparency information in the context of Edge Architecture

Document for	Discussion and agreement

Agenda item: 	9.5 - MeCAR (Media Capabilities for Augmented Reality)

# Introduction

This contribution takes place in the context of an edge architecture, where the rendering function is split between the Edge device, located in close network proximity, and the AR glasses. The principle of this architecture is to offload from the AR glasses the computationally intensive tasks to an edge device. The purpose is to decrease device heating and battery drain, which are two main concerns for the design of AR glasses.

In the context of the edge-assisted architecture, it may be of interest to only rely on 2D encoding and decoding capabilities for the AR glasses. The 3D-related functions are performed on the Edge device and 3D rendering is propagated to the AR glasses thanks to a stereoscopic effect by the transmission of appropriate 2D videos. In that context, in addition to the 2D video, it may also be of interest to transmit transparency information to the AR glasses.

This contribution discusses the interest to transmit transparency information, introduces a discussion where it can be processed and documents various possibilities for its carriage.

# Transparency information

## Interest of Transparency information

It is desirable to support the transmission of transparency information (alpha_channel) in addition to the colour (e.g., RGB) information. Augmented reality services may overlay of virtual objects on the real world which are accessed directly through the “optical see-through” glasses. The overlay is not a full picture but only part of it, the other pixels of the picture being transparent or partially transparent, in case of a shadow effect for instance.

Figure-1 below depicts the overlay of a virtual dragon on the table of a real living room. If the whole video is overlayed, the dragon may appear in the middle of a rectangle corresponding to the video size.This is illustrated on the left picture. With additional transparency information, only the part of the video corresponding to the dragon is overlayed, as illustrated on the right picture.

Figure 1: Video overlay without or with transparency information

## Processing transparency information

This chapter is a preliminary draft. Further study is required on this topic.

We refer to the EDGAR Type-1 device architecture for MeCAR as depicted in Figure 5 of the permanent document

Depending on how the transparency information is carried, it may be processed at different functional blocs.

For example, transparency information may be processed in the Media Access Function. This may be the case if the transparency information is carried in media tracks. This also includes the decoder if transparency information is carried as auxiliary pictures. Transparency information may also be processed by an AR scene manager, when the transparency information is carried in the scene description.

Wherever the processing is located, transparency information is forwarded to the display so that only the appropriate parts of the picture are displayed.

## Carrying transparency information

### Carriage as auxiliary pictures in the video stream

The carriage of transparency information may be achieved by using the concept of auxiliary pictures defined by both the AVC and HEVC codecs.

## AVC

The AVC (H.264) specification provides guidelines for carrying transparency information.

It defines the concept of alpha-blending in clause 3.5 : “A process not specified by this Recommendation | International Standard, in which an auxiliary coded picture is used in combination with a primary coded picture (…) the samples of an auxiliary coded picture are interpreted as indications of the degree of opacity (or, equivalently, the degrees of transparency) associated with the corresponding luma samples of the primary coded picture.” AVC specification precises in clause 3.7 that “An auxiliary coded picture must contain the same number of macroblocks as the primary coded picture. Auxiliary coded pictures have no normative effect on the decoding process.” It also mentions (clause 3.1) that “In addition to the primary coded picture, an access unit may also contain (…) one auxiliary coded picture”.

Clause 7.3.2.1.2 (Sequence parameter set extension RBSP syntax) of the AVC specification defines fields related to alpha blending (alpha_incr_flag, alpha_opaque_value and alpha_transparent _value) and the semantics are detailed in clause 7.4.2.1.2 (Sequence parameter set extension RBSP semantics). The same clause also explains that “aux_format_idc equal to 1 indicates that exactly one auxiliary coded picture is present in each access unit of the coded video sequence, and that for alpha blending purposes the decoded samples of the associated primary coded picture in each access unit should be multiplied by the interpretation sample values of the auxiliary coded picture in the access unit in the display process after output from the decoding process.”

## HEVC

HEVC also defines how to carry an alpha channel in the same video bitstream as the base video.In this case, each frame contains two parts: a base layer containing the video, and an alpha layer containing the alpha channel information. Both layers are compressed using the HEVC codec. The two layers are signalled by a specific HEVC syntax element, namely a specific alpha channel information SEI message has to be added, so that the decoder knows how to interpret the auxiliary pictures. A decoder incapable of handling this SEI message only decodes the base layer.

The concept of auxiliary picture is defined in Annex F of the HEVC (H.265) specification:

- F.3.5 auxiliary picture: A picture that has no normative effect on the decoding process of primary pictures, and with a nuh_layer_id value such that AuxId[ nuh_layer_id ] is greater than 0.

In the same Annex, Table F.2 details the different types of auxiliary pictures:

Table F.2 – Mapping of AuxId to the type of auxiliary pictures

| AuxId | Name of AuxId | Type of auxiliary pictures | SEI message describing interpretation of auxiliary pictures |
| --- | --- | --- | --- |
| 1 | AUX_ALPHA | Alpha plane | Alpha channel information |
| 2 | AUX_DEPTH | Depth picture | Depth representation information |
| 3..127 |  | Reserved |  |
| 128..159 |  | Unspecified |  |
| 160..255 |  | Reserved |  |

## Carriage in ISOBMFF-based formats

ISO/IEC 14496-12 (ISO Base Media File Format) defines a general format which is used as a basis for defining other carriage formats such as MP4, CMAF, HEIF, AVIF, etc. It defines general concepts for the transport and carriage of data in an ISOBMFF-based container format and therefore nothing specific for the transport of auxiliary pictures is mentioned. Among many other things, ISOBMFF specifies the concept of non-timed items that can be used to store static images, and the concept of tracks that can be used to define the carriage of timed video. The format is flexible enough to allow carriage of auxiliary picture information, such as transparency, in the same structure (e.g., using a single item or a single video track) as well as using multiple tracks or items, with dependencies signaled by the item or track reference concept. For example, in the latter case an auxiliary video track needs to be used together with the 'auxl' track reference type as defined in clause 8.3.3.

HEIF (High Efficiency Image File format, ISO/IEC 23008-12) builds on top of ISOBMFF and defines a format for carriage of images and image sequences. Because the format is derived from ISOBMFF, transparency information can be carried in a single item or using multiple items, although the latter requires an 'auxl' item reference type similar to the video example above.

MIAF (Multi Image Application Format, ISO/IEC 23000-22), which defines additional constraints for HEIF, describes in its clause 7.3.5 how MIAF auxiliary image items can be used to carry alpha planes. In particular, it mentions that “MIAF renderers shall interpret alpha planes and should support alpha blending using alpha-plane auxiliary images. This is especially important for image overlays”. It also details in its clause 6.7 (MAIF renderer processing model) the interpretation of alpha planes.

### Carriage in Scene Descriptions

Possible carriage of transparency information or associated transparency information via the scene description has also to be considered. In particular, this may be a solution when an auxiliary picture carrying transparency information is available and not to rely on the SEI messages which not be decoded by all decoders (see HEVC chapter).

As an example for carrying transperncy information in scene descriptions, clause 3.9 on Materials in the glTF 2.0 specification has a subclause “additional-texture” and a subclause “alpha-coverage” which describe how to use the fourth (alpha) component, if present, of a texture.

# Proposal

We propose to include clause 2 of this contribution as an informative part in the permanent document.

# References

- TS 26.511 “5GMS Profiles, Codecs and Formats”.

- H.264 “Advanced video coding for generic audio visual services”

- H.265 “High Efficiency Video Coding (HEVC) Recommandation ITU_T H.265”.

- ISO/IEC 14496 “Coding of audio-visual objects”

- ISO/IEC 23008-12 “HEVC Still Image file format”

- ISO/IEC 23000-22 “Multi Image Application Format »

- glTF 2.0 specification