Source:	Huawei, Tencent

Title:	AR media types and transport discussion for MeCAR

Document for:	Discussion and Agreement

Agenda Item:	9.5

1	Introduction

At the RTC SWG Telco #4, contribution S4aR220018 was discussed and an agreement was reached (cf. S4-220924, “Report for SA4 RTC SWG 3 August 2022 Teleconference”). This contribution attempts to implement the agreed way forward and provides an opportunity for further discussion before reaching a final decision.

2	AR/MR Media types and real time characteristics

2.1	Introduction

AR/MR applications are expected to deal with various media types that have different characteristics and levels of dependency on real-time transport. This clause introduces how and where some of the AR/MR media types are defined and provides an assessment on their levels of dependency on real-time transport and size/bitrate characteristics.

2.2	Media types definition

| AR/MR Media Type | Definition | Media Type Description (Examples) |
| --- | --- | --- |
| Scene Description | Clause 4.4.2 of 3GPP TR 26.998[1] Scene description is used to describe the composition of a 3D scene, referencing and positioning the different 2D and 3D assets in the scene typically using a tree or a graph structure. | Type: glTF2.0, JSON format Organization: MPEG-I and Khronos |
| Spatial Description | Clause 4.4.7.3 of 3GPP TR 26.998[1] Visual features, keyframes, and spatial maps are used for mapping the real world, typically as part of the SLAM process. | Type: For example: https://www.khronos.org/registry/OpenXR/specs/1.0/html/xrspec.html#XR_FB_spatial_entity Organization: OpenXR |
| 3D Visual Model | Clause 4.6.3.5.2 of 3GPP TR 26.928[2] 3D visual object description as a list of vertices, faces and other elements, along with associated attributes. | Type: PoLYgon Organization: None |
| AR Anchor | The AR anchor is meant to identify a point in the user space to be used to anchoring a visual object (2D or 3D) | Type: Metadata allowing accurate overlaying/rendering of text, graphics or video contents to support Use Case 8 of TR 26.928. Organization: None |
| User Pose | Clause 4.4.3.1 of 3GPP TR 26.998[1] Representation of the user position and orientation | Type: It consists of a quaternion for orientation and a 3D vector for position. Timestamp is represented by a 64 bit monotonically increasing nano-second-based integer. Organization: Khronos OpenXR |
| FOV | Y.6.2.3 of 3GPP TS 26.114[3] The Field of View (FOV) is the extent of observable world at any given moment | Type: It consists of vertical fov and horizontal fov. Organization: None |
| Viewport | Y.7.2 of 3GPP TS 26.114[3] The viewport corresponds to the projection of the user View onto a target display | Type: It shall contain all of the parameters Viewport_azimuth, Viewport_elevation, Viewport_tilt, Viewport_azimuth_range and Viewport_elevation_range Organization: None |
| Gesture | TBD | Type: A array of finger joint position. For example: https://www.khronos.org/registry/OpenXR/specs/1.0/html/xrspec.html#XR_EXT_hand_tracking Organization: OpenXR |
| Body action | TBD | Type: bvh format. Frequency: at least 1kHz Organization: BioVision company |
| Facial expression | OpenXR | Type: An array of key point position. For example: https://www.khronos.org/registry/OpenXR/specs/1.0/html/xrspec.html#XrSystemFacialTrackingPropertiesHTC Organization: None |
| Sensor information | TBD | Type: TBD Organization: None |
| Camera information | TBD | Type: The camera parameters such as focal length, principal points, calibration parameters and the pose of the camera all contribute in understanding the relevance between points in the volumetric scene and pixels in the captured image. Organization: None |
| Projection information | Clause Y.3 of 3GPP TS 26.114 and H264 protocol Parameters associated to the projection from the 3D scene to the user display | Type: Projection mapping information (indicating the projection format in use, e.g., Equirectangular projection (ERP) or Cubemap projection (CMP)), for the projection sample location remapping process as specified in clauses 7.5.1.3 and 5.2 of ISO/IEC 23090-2 [179] Organization: None |

Editor’s note: The list of Media Type is a starting point and other types of AR media will be added.

2.3 Real time characteristics

| AR/MR Metadata Media Type | Real Time Characteristics |
| --- | --- |
| Scene Description | Non timed |
| Spatial Description | Sparsely timed (event based) |
| 3D Model | Non timed |
| AR Anchor | Sparsely timed (event based) |
| User Pose | Continuous |
| FOV | Non timed |
| Viewport | Continuous |
| Gesture | Continuous |
| Body action | Continuous |
| Facial expression | Continuous |
| Sensor information | Non timed |
| Camera information | Sparsely timed (event based) |
| Projection information | Sparsely timed (event based) |

2.4 Traffic characteristics

| AR/MR Metadata Media Type | Typical size or bitrate |
| --- | --- |
| Scene Description | TBD |
| Spatial Description | TBD |
| 3D Model | TBD |
| AR Anchor | TBD |
| User Pose | TBD |
| FOV | TBD |
| Viewport | TBD |
| Gesture | TBD |
| Body action | TBD |
| Facial expression | TBD |
| Sensor information | TBD |
| Camera information | TBD |
| Projection information | TBD |

3 Proposal

We propose to include the Clause 2 of this contribution in the MeCAR PD under a sub-clause 3.x entitled “Media Type Categories and Characteristics”.

References

- 3GPP TR 26.998: "Support of 5G Glass-type Augmented Reality / Mixed Reality (AR/MR) devices ".

- 3GPP TR 26.928: "Extended Reality (XR) in 5G"

- 3GPP TS 26.114: " IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".