Source: 	China Mobile

Title: 	[iRTCW]Functional Requirements for Avatar Animation

Document for	Agreement

Agenda item: 	10.5

Introduction

TR 26.928 identified two use cases and requirements on AR avatar, i.e.AR animated avatar calls and AR avatar multi-party calls (A.18 and A.19 in TR 26.928). The AR glasses user sees an animated avatar of the phone user. Movements of the phone user are used to control the animation of his avatar. These two use cases illustrate the requirements for avatar animation technology, that is, the animation technology can be separated from 3D video capture, animating an off-the-shelf 3D image instead of capturing it in real time during a call. With the real-time animation, the avatar can behave like a human and have the ability to express with language, facial expressions and body movements.

This document evaluates some requirements to address support for avatar animation for iRTCW, since several OTT services conducted by avatars have realized that movements and expressions are synchronized with reality. Real-time animation technology support various complex interactions in the immersive space. In addition to relying on infrastructure such as computing resource and network, it is also a combination of various technologies such as modeling, rendering, animation , and AI. The main implementation methods include vision-driven and speech-driven animation.

Workflow for Avatar Driven Animation

- 2.1 Vision-driven

Vision-Driven refers to capturing the action and facial expression data of live-action people, and then transferring these data into a virtual avatar.

- With the progress of motion capture, posture, expression and other recognition algorithms, ordinary cameras can achieve more accurate motion (for example, iPhone 12 camera can already support simple motion capture).The general workflow is as follows.

- 1.Avatar generation or selection. Users can design or load avatar and multiple avatar roles can be selected for the user to log on.

- 2.Model binding. It includes conducting 3D modeling of face and body, selecting key points and mapping the identified key points to the model.

- 3.Motion capture. It will capture key changes in body shape, expression, eyes, gestures, etc.

- 4.Animation and rendering.

- 5. Generate content and interact.

Therefore, the requirements of the client supporting vision-driven may be as follows

- Support for animating mode selection.

- At least one RGB camera for input.

- Support for vision-driven applications.

- Support expression-based or other avatar driven data and interface requirements.

Speech-driven

For speech-driven, facial expression and specific action of the virtual avatar will be controled in real time mainly through the calculation results of a deep learning model.The general workflow is as follows.

1.Avatar generation or selection.

2.Model binding.

3.Train AI models to determine the final effect.

4.Content production. Based on input speech (or speech transformed from input text), predict lip movement, expression and other parameters.

5.Render and generate the final content.

Therefore, the requirements of the client supporting speech-driven include

- Support for animating mode selection.

- One microphone or a keyboard for input.

-Support for speech-driven applications

-Support TTS(Text To Speech) and interface requirements

-Sufficient computing resource to support AI and rendering, ensuring real-time and high-precision user experience.

- Figure 1  the process of avatar driven

Proposal

It is proposed to make the cameras, microphones, sensors, and other features of the iRTC client to support these avatar-type and animation applications.The following next steps should be taken:

- -Specify the requirements for avatar-type hosting and delivery .

- -The implementation methods of avatar animation can be taken into account proposed in Section 2.

References

- [1] NVIDIA Omniverse Audio2Face  https://www.nvidia.com/en-us/omniverse/apps/audio2face/

- [2]ARKit https://developer.apple.com/documentation/arkit/content_anchors/tracking_and_visualizing_faces

- [3]Horizon World metaverse

- https://www.oculus.com/horizon-worlds/?utm_source=gg&utm_medium=ps&utm_campaign=17154134368&utm_term=horizon%20world&utm_content=&gclid=CjwKCAjwi8iXBhBeEiwAKbUofcHZg5PB7YylkYC0