Source:	Huawei

Title:	Split rendering solution for IBACS

## Document for:	Discussion and Agreement

## Agenda Item:	10.6

# 1	Introduction

At the 3GPP meeting #118-e, the WID S4‑220513 for IMS-based AR communication was approved, then at the 3GPP meeting #120-e, the IBACS PD document S4-221212 was approved.

According to clause 4.2.2 of 3GPP TR 26.998[1], split rendering solution is needed for EDGAR device which has limited media rendering capability. Meanwhile, in clause 6.2.6 of 3GPP TR 26.928[2], a generalized XR Split Rendering solution is defined to show how split rendering works on both UE and XR server.

Based on the consideration of making full use of the terminal's computing power and reducing the interaction delay, the terminal needs to complete the AR media rendering within its own capabilities. When the terminal judges that it cannot independently complete the AR media rendering, it will split the parts (e.g. complex background) which cannot be completed by itself and requests them to be processed by the network side for rendering.

This contribution provides the discussion on split rendering solution based on data channel for IBACS.

# 2	AR IMS Communication Architecture

The following diagram depicts an architecture that has been done by SA2 as part of FS_NG_RTC in TR23.700-87.

Figure 2.1: Enhanced IMS architecture to support AR communication

AR communication architecture is enhanced based on data channel architecture, including the following network functions:

1.	Add a new network function AR Application Server, responsible for AR service control related to AR communication, including AR session media controlling, AR media rendering negotiation and AR service handling.

NOTE 2:	AR Application Server can be deployed in operator's network or belongs to third party.

2.	Add the AR media processing network function ARMF, responsible for AR communication media transmission and media rendering function, including the following functions:

-	AR Rendering Logic: controls the application-based rendering logic of AR communication.

-	AR Media Processing Function: including Vision Engine, 3D Rendering Engine. Vision Engine and 3D Rendering Engine will establish spatial map, and render the scenes, virtual human models and 3D object models according to the field of view, posture, position, etc. which are transmitted from UE using data channel.

NOTE 3:	ARMF is a logic network function element, it can be deployed together with other media functions such as DCMF, it can also be collocated in other functional entities, e.g. UMF (Unified Media Function).

The following reference points are used for the IMS AR communication architecture. It is suggested that SBI is applied to these reference points.

-	DC4: Service based reference point between the AR Application Server and the DCSF for AR service handling and AR session media control.

-	DC5: Service based reference point between the MMTEL AS and the ARMF for AR resource management.

The following media interface are used for the IMS AR communication architecture.

-	MDC2: Reference point of data channel media between the AR Application Server and DCMF for AR media rendering negotiation.

-	MDC4: Reference point of AR media between the AR Application Server and ARMF for AR media transmission.

-	Mb (RTP&DC): The media interface between UE and IMS-AGW, including RTP channel used for audio/video media stream transmission and data channel used for data media stream transmission.

-	Mb (RTP): The media interface between IMS-AGW and ARMF, using RTP channel for audio/video media stream transmission.

-	Mb (HTTP): The media interface between DCMF and ARMF, used by DCMF to transmit data in HTTP channel after transforming data media stream received in data channel to HTTP format.

-    Mb (DC): the media interface between IMS-AGW and DCMF, using data channel for data media stream transmission.

# 3	IMS AR Sessions and transport

According to AR communication scenarios defined by TR 26.998 [14] and TR 26.928 [15], media processing capabilities are different for different AR devices (STAR, EDGAR, and WLAR). The AR communication call flows can be summarized as follows:

1.	Terminal rendering process: When the media processing capability of AR device meets the requirements of AR communication (such as AR anchor and simple AR effect) according to its status such as power status, signal status, computing power status, internal storage status, etc, the AR device can independently implement AR media rendering by itself. The either side of IMS provides AR application to the calling and the called users at the same time. The terminal performs local rendering based on the media obtained locally or sent by the peer.

2.	Network rendering process: When the media processing capability of AR device cannot meet AR communication requirements (such as complex background or high-accuracy 3D virtual model) according to its status, the AR device can decide to request IMS for AR media rendering. The either side of IMS provides AR application to the calling and the called users at the same time, IMS then performs AR media rendering based on AR media received from the calling or the called users, finally IMS sends rendered AR media using normal audio/video streams through RTP channel to the calling or called user.

#### 3.1 Terminal Rendering Process

Figure 3.1: Terminal rendering process

Key steps:

1.	The UE-A initiates an AR communication session and establishes audio and video session connections with the UE-B.

2.	The bootstrap and application data channel connections are established for the UE-A and UE-B according to AR application requirements.

Scenario 1: The UE performs AR media rendering based on the AR media get from local application.

3.	The UE-A gets AR media (such as AR effect) from AR application locally and performs AR media rendering itself.

4.	The UE-A sends the rendered AR audio/video media to the peer through the RTP channel.

Scenario 2: The UE performs AR media rendering based on the AR media get from remote application.

5.	The UE-B sends AR media through application data channel to UE-A.

6.	The UE-A performs AR media rendering itself based on the AR media received, and displays the rendered audio/video media on its screen.

#### 3.2 Network Rendering Process

Figure 3.2: Network rendering process

Key steps:

1.	The UE-A initiates an AR communication session and establishes audio and video session connections with the UE-B. Then the bootstrap data channels are established for the UE-A and UE-B.

AR Media Rendering Negotiation Procedure:

2.	The UE-A decide to request network media rendering based on its status such as power status, signal status, computing power status, internal storage status, etc.

3.	The UE-A initiate the application data channel between UE-A and the DCMF.

4.   The AR AS stores the media point information for DCMF.

5.	The UE-A sends a Media Rendering Negotiation Request message through the application data channel to DCMF through IMS-AGW, carrying the content (e.g. the complex background or high-accuracy 3D virtual model) that needs to be rendered in the network.

6.   The DCMF transfers the request to AR AS.

7.	The AR AS decides whether to provide media rendering function based on the request from UE in step 5 and the available media rendering resource on ARMF.

8.	The AR AS calculates the computing power according to the contents that needs to be rendered by ARMF, and then sends a request with the computing power to DCSF to allocate media rendering resource on ARMF.

9.	The DCSF sends a XR Service Resource Allocation Request to IMS AS.

10.	The IMS AS sends a XR Media Resource allocation request to ARMF. The ARMF allocates the media rendering resource according to the computing power value. When the ARMF resource applied successfully, it returns a successful response carrying the termination addresses information allocated on ARMF for connecting UE-A and UE-B to the ARMF.

11.	The IMS AS sends an event notify to the DCSF, indicating the XR service resource allocation is successful. The DCSF forwards the message to AR AS.

12.	The AR AS stores the media point information of the ARMF.

13.	The AR AS sends a Media Rendering Negotiation Response to indicate the media rendering negotiation is successful.

14. The DCMF transfer the response received from AR AS and sends a Media Rendering Negotiation Response through the application data channel to UE-A through IMS-AGW.

Note: Subsequently, the UE-A can request to change the network rendering content if the status changed in the UE-A, such as low power, low computing power etc.

AR Session Media re-Negotiation Procedure For Network Rendering:

15.	If the XR resource are allocated successfully in step9, the IMS AS sends a DC modification request carrying ARMF’s URL to the DCMF, to connect DCMF and ARMF for AR media transporting.

16.	The IMS AS sends a REINVITE message carrying ARMF address information, to anchor UE-A's audio/video media stream to ARMF through IMS-AGW.

17.	The UE-A sends 200 response carrying UE-A's audio/video media address information.

18.	The IMS AS sends a media modification request to ARMF, to connect ARMF and IMS-AGW for audio/video media transporting.

20. The IMS AS sends a REINVITE message carrying ARMF address information, to anchor UE-B's audio/video media stream to ARMF.

21.	The terminating IMS side sends 200 for REINVITE.

22. The IMS AS sends a media modification request to ARMF, to connect ARMF and UE-B.

Networking Rendering Procedure:

23.	The UE-A starts AR media rendering.

24-25.The UE-A sends AR media through application data channel to DCMF and DCMF transfers the AR media to ARMF for AR media rendering.

26.	The ARMF performs AR media rendering.

27.	The UE-A performs part of the AR media rendering itself.

28-29.The UE-A sends the rendered audio/video media over RTP to ARMF through IMS-AGW. If UE-A does not perform the rendering, the UE-A sends the original audio/video media over RTP to ARMF.

30-31.The ARMF combines the AR media rendered by ARMF and the rendered AR media received from UE-A, and sends the complete audio/video media to UE-B. If UE-A does not perform the rendering itself, step 25 can be omitted, and the ARMF directly sends the rendered media to the UE-B.

32.	The ARMF sends the rendered video RTP media stream to the UE-A when it needs to send video back to the UE-A through the video back channel.

33.	The UE-A hangs up, UE-A sends a BYE message to P-CSCF.

34.	AR AS sends a media resource release request to DCSF to release the allocated resource on DCMF.

35.	The DCSF sends a DC control request to IMS AS to release DC resource.

36. The IMS AS sends a DC media resource delete request to DCMF, to release DC resource on DCMF.

37.	The IMS AS sends the XR Media Resource release request to ARMF.

# 4	Proposal

We propose to agree that the Clause 2 and Clause 3 of this contribution to be included in the IBACS PD.

# References

- 3GPP TR 26.998: "Support of 5G Glass-type Augmented Reality / Mixed Reality (AR/MR) devices ".

- 3GPP TR 26.928: " Extended Reality (XR) in 5G".