Title: 	[IBACS] 3D video call flow

Source: 	Nokia Corporation1

Document For: 	Agreement

Agenda item: 	10.6

# Introduction

The contribution provides a call flow for a basic AR call for IBACS when volumetric/3D video is used.

# 3D video call flow

Figure 2.1 Call flow for bidirectional 3D video call

Steps 1-13 under A. Call Setup, B. Scene description retrieval and C. scene description update will be the same as for the basic AR call flow. The remaining steps are defined below.

D. AR Media and Metadata Exchange

- UE1 will send 3D video (e.g., RGB-D streams, or volumetric media represented as mesh or point cloud) to the MRF.

- UE2 also sends 3D video to MRF.

- The MRF processes AR media, e.g., for viewport-dependent processing (VDP), combining AR objects into a single frame, transcoding from one 3D representation to another, etc. Note that VDP requires a feedback channel from UEs to MRF not shown in the call flow.

- MRF delivers 3D video to UE1.

- MRF delivers 3D video to UE2.

NOTE: Delivery of real-time media over data channel is FFS.

# Proposal

The proposal is to include the text in section 2 to the permanent document of IBACS.