Source: 	Interdigital Finland Ory

Title: 	[FS_AI4Media] Complete/Basic AI/ML model distribution service flow

Agenda item: 	9.7

Document for:	Discussion and Agreement

#### 1 Introduction

A block diagram of the general architecture for the distribution of complete/basic AI/ML models has been proposed and approved in previous meetings.

This contribution proposes a corresponding service flow for the distribution of the complete/basic AI/ML model.

The service flow to be discussed highlights the main interactions between the UE requesting an AI/ML service and the network distributing a model adapted to the UE.

#### 2 Proposed changes

--------------------------------------------- Begin change -----------------------------------------------

## 5.2	Architecture Overview

## 5.3	Service flows

5.3.1	Complete/Basic AI/ML model distribution

Figure X Complete/Basic AI/ML model distribution flow diagram

Figure X shows a simple service flow for the distribution of a complete/basic AI/ML model. The service may be decomposed in several parts as follows for illustration:

First, the application provider announces the availability of the AI/ML service or provisions service information and may ingest a set of service models to the AI model repository.

- The UE sends a model request/subscription to the network. This may include necessary information used by the network to provide an adapted model to the UE. This information may include, for example:

  - AI/ML service information including the type of AI/ML service (e.g., object recognition), the service request (e.g., detail of recognition request), service quality, service requirements (e.g., expected accuracy/precision), inference latency.

  - UE capabilities including operating system, AI/ML engine in use, processing capabilities (CPU/GPU/TPU), and available memory.

  - Network capabilities including distribution mode such as unicast/multicast and available bandwidth

- The network uses the UE request information to select an adapted model for the UE. If such model is not available, the network may either build or ingest a new model. Upon selection, the network sends an answer/notification to the UE including a model description comprising:

  - Model composition including for example model identifier, model name, model size, model subsets.

  - Distribution configurations for model or model subsets including for example for each distribution mean

    - Delivery mode, such as progressive download, DASH streaming, or real-time transport (e.g., RTP),

    - Application server information, such as information to connect the server,

    - Distribution mode, such as Unicast/Multicast/Broadcast,

    - Distribution conditions, such as the required bandwidth to distribute the model or the model subset.

- The UE retrieves the model description and selects a distribution configuration for the model or the model subsets and triggers the AI model access function to perform the delivery.

- The UE initiates the model delivery through the AI model access function which establishes one or more transport sessions with the AI model delivery function that distributes the AI/ML model, which may be divided into multiple parts, towards the AI model inference engine of the UE.

- The AI model access function notifies the AI/ML application about the completion of the delivery.

--------------------------------------------- End change --------------------------------------------------

#### 3 Proposal

We propose to add a new clause dedicated to service flows (clause 5.3) and create a service flow for the distribution of a complete/basic AI/ML model as subclause 5.3.1.