Source:	Dolby Laboratories, Inc.

Title:	On IVAS codec complexity and memory design constraints

Document for:	Discussion

Agenda Item:	7.5

According to IVAS project plan Pdoc IVAS-2, the finalized IVAS design constraints Pdoc IVAS-4 are due at meeting SA4#121. Accordingly, at SA4-e (AH) Audio SWG post 120-e telco on October 21st the source provided a discussion input on IVAS codec complexity and memory design constraints [1].

The present contribution likes to resume the discussion with the provision of a more concrete proposal reflecting the discussion that took place at the Audio SWB telco.

- Discussion

The general background is that the IVAS codec is expected to address a large range of use cases from stereo and immersive telephony and teleconferencing over more advanced XR communications and XR content streaming to any kind of premium extensions of XR communications and XR content streaming that may also include specific server-based operations. Considering that more advanced operations should generally lead to increased quality of experience and that the relevant audio formats for such advanced operations will generally comprise a larger number than constituent audio components (or channels), it is clear that there cannot be a single constraint on complexity and memory. Such a single constraint would either prohibit advanced operations or be too permissive which in turn might make implementations possible only on a very limited set of high-performance devices.

Consequently, there should rather be a set of complexity/memory limits, taking into account that there may be different device classes with different capabilities. It should thus be possible to run certain IVAS baseline operations on a broad class of less capable and presumable less expensive devices and to run certain advanced IVAS operations on more capable devices. Very advanced (or extended) IVAS operations could be limited to selected high-end devices with corresponding capabilities or servers.

This thought leads to defining three functionality levels of the IVAS codec with respectively increased capability demands and accordingly increased complexity/memory limits. The suggested functionality levels are

- Level 1: Baseline Operations enabling immersive voice communications

- Level 2: Advanced Operations enabling XR communications and XR content streaming

- Level 3: Extended Operations enabling premium extensions of XR communications and XR content streaming, and server operations

Each of these levels should then be associated with certain complexity and memory constraints. As EVS complexity and memory requirements are well known and EVS interoperable operations are in any case part of the IVAS codec, it is felt reasonable to define such complexity and memory constraints relative to the corresponding EVS numbers. Below is a proposal for such constraints where supplementary operations such as for JBM should be excluded:

- Level 1:

- Complexity <= 3 * EVS

- RAM <= 3 * EVS

- Level 2:

- Complexity <= 6 * EVS

- RAM <= 6 * EVS

- Level 3:

- Complexity <= 10 * EVS

- RAM <= 10 * EVS

In addition, the EVS interoperability mode should not require substantially increased complexity or memory compared to standard EVS.

ROM and PROM constraints should be independent of the level. The following limits are suggested:

- ROM, PROM <= 10 * EVS

The complexity estimation shall be based on ITU-T G.191 STL 2022 [2] and code instrumentation using the WMC tool offered by VoiceAge [3]. To account for measurement inaccuracies, the limits must not be exceeded with a tolerance of 10%.

In Tdoc S4aA220014 [1] a concrete proposal was made how to map actual encoder/decoder/renderer operations with corresponding audio formats and bit rates to these levels. While such a mapping might allow setting very specific constraints on the operations to be supported by the different levels, the discussions at the Audio SWG telco revealed a possible downside, which is the complexity of the approach and the required level of detail. This would make it difficult to make sure that all required operations within a level are fully consistent and justifiable.

To progress the matter, it is here proposed to adopt a more abstract level approach: Rather than detailing all operation modes of encoder/decoder/renderer to be supported at each level, there should be a requirement that operations support at each of the functionality levels is provided. In addition, there should be a selection deliverable for the codec proponent to document what specific operations are supported at each of the functionality levels.

- Conclusion and Proposal

The contribution offers a practical way to set IVAS complexity and memory design constraints in levels of increased functionality that are associated with IVAS use cases. The following shows how the proposal is implemented in Pdoc IVAS-4:

| Complexity | Complexity/memory limits are defined in 3 levels of increased IVAS functionality. The levels are specified as follows: - Level 1: Baseline Operations enabling immersive voice communications - Level 2: Advanced Operations enabling XR communications and XR content streaming - Level 3: Extended Operations enabling premium extensions of XR communications and XR content streaming, and server operations Candidate codecs must offer operations support according to these functionality levels for which the following level-dependent limits apply excluding supplementary operations like JBM: - Level 1: - Complexity <= 3 * EVS - RAM <= 3 * EVS - Level 2: - Complexity <= 6 * EVS - RAM <= 6 * EVS - Level 3: - Complexity <= 10 * EVS - RAM <= 10 * EVS In addition, the EVS interoperability mode should not require substantially increased complexity or memory compared to standard EVS. The following level-independent ROM and PROM constraints apply: - ROM, PROM <= 10 * EVS The complexity/memory estimation shall be based on ITU-T G.191 STL 2022 (Details of complexity/memory measurement are specified in Pdoc IVAS-7a). To account for measurement inaccuracies, the limits must not be exceeded with a tolerance of 10%. As part of the selection deliverables, proponents shall provide a detailed documentation how and with which specific operation modes their IVAS candidate meets the complexity constraints of the different levels. |
| --- | --- |

…

- References

[1]	S4aA220014: On IVAS codec complexity and memory design constraints, Dolby Laboratories, Inc.

[2]	ITU-T Recommendation G.191: Software tools for speech and audio coding standardization (07/2022)

[3]	S4aA220013: WMC tool – an automated tool for complexity and memory evaluation of IVAS floating-point candidate codecs, VoiceAge Corporation