Agenda item: 	8.6

Source: 	Qualcomm Inc.

Title: 	Split Rendering Architecture and Call Flow

Document for	Discussion and Agreement

# Introduction

In this contribution, we provide an architecture and a call flow for split rendering operations. The focus of this contribution is on the baseline 2D profile of split rendering, where content is fully flattened to 2D or 2.5D. A separate split rendering profile will be needed for the case of split rendering with 3D output.

# Architecture

For defining the architecture, we use OpenXR as a reference API for talking to the XR runtime. The rendering process on the edge application server (EAS) needs to generate pre-rendered content and transmit it to the UE for display on the AR glasses/HMD.

Split rendering may be operated in many different configurations, ranging from complete pre-rendering and all the way to creating simplified 3D content. The split rendering MSE must support the full pre-rendering configurations that can be mapped directly to OpenXR. These are configurations that support the view and composition layer options that OpenXR offers.

The following diagram depicts the client architecture for split rendering:

The entry point to the rendering process will be a description of the pre-rendered content, which may range from 2D to 3D content. A thin presentation engine is responsible for parsing the scene description, requesting the visual media streams, and then mapping the received media data onto OpenXR swapchain images. The Presentation Engine will also map the rendered audio data onto a format that the audio composition engine understands and is able to playback.

The Uplink media management component will query the XR runtime for pose information and user input events and then stream them uplink to the split rendering server.

# Call Flow

The split rendering operation can be described by the following call flow:

The steps are:

- The Presentation Engine discovers the split rendering EAS and sets up a connection to it. It provides information about its rendering capabilities and the OpenXR configuration.

- In response, the split rendering EAS creates a description of the split rendering output and the input it expects to receive from the UE.

- The Presentation Engine requests the buffer streams from the MAF, which in turn establishes a connection to the split rendering EAS to stream pose and retrieve split rendering buffers.

- The Source Manager retrieves pose and user input from the XR runtime.

- The Source Manager shares the pose predictions and user input actions with the split rendering EAS

- The split rendering EAS uses that information to render the frame

- The rendered frame is encoded and streamed down to the MAF

- The MAF decodes the rendered frame data components

- The MAF then passes the raw frame data to the Presentation Engine, which in turn, passes it in the appropriate format to the XR runtime.

- The XR runtime performs any composition operation and displays the resulting content to the viewer.

# Proposal

We propose to document the client architecture and the call flow in the split rendering specification.