Agenda item: 	8.6

Source: 	Qualcomm Inc.

Title: 	Split Rendering Formats

Document for	Discussion and Agreement

# Introduction

Split rendering may be performed in one of a variety of operation points, ranging from complete flattening and projection of the 3D scene to 2D views and to 3D scene pre-processing to meet the UEs capabilities and requirements. The following figure depicts this wide spectrum of split rendering operation points:

In this contribution, we describe a profile of split rendering that corresponds to the full flattening and pre-rendering on the edge server. It supports different configurations that may be mapped directly to OpenXR.

# Full Pre-rendering Profile

## 2.1 	Overview

The full-prerendering profile is restricted to the support of 2D content exclusively. The capabilities of the receiving UE are shared with the split rendering server prior to the start of the split rendering session. These capabilities and configurations would indicate the audio-visual output setup on the UE. For example, it would indicate that the output device is an HMD that supports 2 views and stereo audio.

## 2.2 	Downlink Formats

The visual formats supported in this profile match the options that are supported by OpenXR. The supported view configurations are:

- Mono: a single view

- Stereo: one view per eye

The following composition layers are supported:

- Projection: projection of the scene to a 2D plane using a perspective camera

- Quad: a 2d surface that is composed in the 3D space by the XR runtime

- Equirectangular: an equirectangular projection of the 3D space that is usually used to provide a background

- Cubemap: a set of 6 swapchain images that represent a projection of the 3D scene onto a cube

Each swapchain image will have the following properties:

- Format: RGB, RGB with Alpha (RGBA), and single-channel Depth formats with different precisions. RGB may be recovered from the coded YUV video stream. Depth information may be coded a separate video stream.

- Dimension: width and height of the swapchain image

- Mipmap: count of the level of detail of the swapchain image. The swapchain images maybe created at the UE side. Some Graphics Engines expect that the image dimensions are a power of 2.

For audio, the following formats are to be supported:

- Stereo audio mixed and binauralized based on the viewer’s current pose

- HOA audio mixed based on the viewer’s current position that extracted from the pose

## 2.3 	Uplink Formats

The rendering process relies on the reception of pose predictions and user input. The pose information is formatted as follows:

- An array of multiple pose predictions

- Each pose prediction consists of a position and orientation component as a 3D (coordinates) and 4D (quaternion) vectors respectively.

- The prediction timestamp associated with the predicted pose

- An XR space for which the pose is created. If not present, this defaults to the viewer’s XR space.

The user input may be described following OpenXR’s convention on naming the actions.

# Proposal

We propose to agree the content of section 2 and work on refining the format descriptions.