| 3GPP TS 26.565 V0.3.0 (2023-02) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Split Rendering Media Service Enabler; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	General	8

4.1	Overview	8

4.2	Typical Use Cases	8

5	Reference Architecture and Procedures	8

5.1	Reference Architecture	8

5.1.1	Introduction	8

5.1.2 	Client Architecture	8

5.1.3	End-to-End Architecture	9

5.1.5	User Plane Architecture	9

5.2	Procedures and Call Flows	10

5.2.1 	Call flow for Split Rendering instance discovery	10

5.2.1.1 Client-driven procedures and call flows	10

5.2.1.2 Network-driven procedures and call flows	11

5.2.2 	Call flow for Split Rendering session setup	12

6	Prerequisites	13

6.1	Requirements on 5G System	13

6.2	Requirements on Device APIs and Functionality	13

7	Split Rendering Application Function	14

7.1	Functionality	14

7.2	RESTful APIs	14

7.2.1	Split-Rendering Provisioning procedures (SR-1)	14

7.2.1.1	General	14

7.2.1.2	Create Split-Rendering Configuration	14

7.2.1.3	Read Split-Rendering Configuration properties	14

7.2.1.4	Update Split-Rendering Configuration properties	14

7.2.1.5	Destroy Split-Rendering Configuration	14

7.2.2	 Split-Rendering Provisioning API	15

7.2.2.1	Overview	15

7.2.2.2	Resource structure	15

7.2.2.3	Data model	15

7.2.2.3.1	SplitRenderingConfiguration resource	15

8	Split Rendering User Plane	15

8.1	Split Rendering Signalling Protocols	16

8.2	Split Rendering Formats	16

8.2.1 	General	16

8.2.2 	Pixel Streaming Profile	16

8.2.2.1 	Overview	16

8.2.2.2 	Downlink Formats	16

8.2.2.3 	Uplink Formats	16

8.3	Split Rendering Transport Protocols	17

8.4 	Split Rendering Formats for Session Setup and Negotiation	17

8.4.1 	General	17

8.4.2 	Split Rendering Configuration Format	17

8.4.2.1	Introduction	17

8.4.2.2	 Split Rendering Configuration Format	17

9	Split Rendering Client	18

9.1	Functionality	18

9.2	Client API	18

10 	Security and Privacy Aspects	19

10.1 	Security	19

10.2 	Privacy	19

Annex A	 (Informative): Implementation Guidelines	20

Annex A.1 Guidelines for Application Developers	20

Annex A.2 Guidelines for Split Rendering MSE Implementers	20

Annex A.3 Conformance Testing	20

Annex B (normative): IDL Definition of Client API	21

Annex X (informative): Change history	22

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This specification defines a media service enabler for split rendering in the 5G system.

# 1	Scope

The present document defines a Media Service Enabler for Split Rendering according to the guidelines of TR26.857 [1].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 26.857: "5G Media Service Enablers".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AF		Application Function

AS		Application Server

MAF	Media Access Function

MSH		Media Session Handler

5G-RTC		5G Real-Time Communication

SR 		Split Rendering

SRC		Split Rendering Client

SRS		Split Rendering Server

UE		User Equipment

# 4	General

## 4.1	Overview

## 4.2	Typical Use Cases

# 5	Reference Architecture and Procedures

Editor’s Note: The terminalogy and reference-point naming convention needs to be aligned in  this document.

## 5.1	Reference Architecture

### 5.1.1	Introduction

In this clause,  different variants of the reference architecture for the split rendering MSE are defined, each representing a different perspective and level of details.

The following functions are introduced:

- Split-Rendering Client (SRC): This function is responsible to acquire the UE media capabilities and negotiates with the RTC AS to agree on the split-rendering process at the RTC AS.

- Split-Rendering Server (SRS): This function is responsible for negotiation of SR session with SRC, monitoring the server’s edge resource usage, and managing/running the split rendering process.

- Application Function (AF): responsible for provisioning, QoS allocation, and edge resource discovery.

- Application Service Provider: The application provider that offers the service.

- Application: The application running on UE

- Media Session Handler (MSH): is the entity on UE that is responsible for the control plane communication with the AF.

Editor’s Note: All interfaces are currently defined as SR-interfaces. However, if no SR-specific functionality is identified for an interface, the 5G-RTC interface reference will be used instead.

### 5.1.2 	Client Architecture

Figure 5.1.2-1 depicts the reference client architecture.

Figure 5.1.2-1 – Device architecture of AR UE

The split rendering client consists of the following components:

- The Media Access Functions: allow for fetching and processing of the pre-rendered media in preparation of final display. The MAF is also responsible for the carriage of any metadata or local media to the split rendering server.

- The scene manager and thin Presentation Engine: is responsible for the negotiation of the split rendering session and the parsing of the description of the rendered media as provided by the SR server. It is also responsible for setting up and managing the XR session with the XR runtime.

- The XR source management is responsible for gathering timed metadata such as pose and action information and sending it to the SR server.

### 5.1.3	End-to-End Architecture

Figure 5.1-3 – Split management architecture

As shown in Figure 5.1.3:

1. The 5G Application Providers (AP) provisions the split-rendering through SR-1.

2. In the use cases in which the AP is involved in the media delivey, the SR-2 interface is used for this purpose.

3.The communication between AF and SRS is through SR-3.  This interface is out of the scope of this document. This interface may for instance include the EDGE-3 interface.

4. The signaling as well as the media delivery between SRC and SRS is though SR-4.

5. The SRC discovers the client media capabilities through the SR-7 interface. This interface is out of the scope of this document.

6. The 5G Application and AP interact through SR-8. This interface is out of the scope of this document.

### 5.1.5	User Plane Architecture

Figure 5.1.5-1 depicts the user plane architecture for split rendering.

The SR interfaces are considered to be specializations of their parent RTC interfaces as defined in TS26.506.

In the context of split rendering, the SR-4 interface is further classified as SR-4s and SR-4m sub-interfaces. The SR-4s interface covers all user-plane signaling, including WebRTC and ICE signaling. The SR-4m serves for media and metadata exchange between the split rendering client and the split rendering server.

## 5.2	Procedures and Call Flows

### 5.2.1 	Call flow for Split Rendering instance discovery

#### 5.2.1.1 Client-driven procedures and call flows

Figure 5.2.1-1 demonstrates a call flow for setting up the split rendering by the client.

Figure 5.2.1-1: High-level call flow for initiating a split

Steps:

- The Application Service Provider requests the SRF the provisioning a split management session.

- The split management session is announced to the Application as part of the Service Access Information.

- The Application requests a split of the client media functions from the SRC.

- The SRC inquires the Media Session Handler about the client’s media capabilities.

- The SRC and SRS negotiate on the acceptable capabilities for the device and agree on the split option.

- The SRC starts the split rendering process.

- The SRS acknowledges the SRC that the split-rendering on edge is running and provides its access information.

- The SRC acknowledges the Application that the split-rendering on edge is running.

- The Application requests SRC to start the media delivery.

- The SRC starts the media delivery.

#### 5.2.1.2 Network-driven procedures and call flows

Figure 5.2.1.2-1 demonstrates a call flow for setting up the split rendering by the network.

Figure 5.2.1.2-1: High-level call flow for the network-driven split management

Steps:

- The Application Service Provider requests the provisioning of the split management session.

- The split management session is announced to the Application as part of the Service Access Information.

- The SRS offers the SRC a split of the client media functions.

- The SRC inquires the Media Session Handler about the client’s media capabilities.

- The SRC and SRS negotiate on the acceptable capabilities for the device and agree on the split option.

- The SRS starts the split-rendering process on the edge.

- The SRS acknowledges the SRC that the split-rendering on edge is running and provides its access information.

- The SRC announces to the Application that the split-rendering on edge is running.

- The Application requests the SRC to start media delivery.

- The SRC starts the media delivery.

### 5.2.2 	Call flow for Split Rendering session setup

The split rendering operation can be described by the as depicted in the call flow in the Figure 5.2.2-1.

Figure 5.2.2- 1 High-level call flow for split rendering session setup and operation

The steps are:

- The Presentation Engine discovers the split rendering server and sets up a connection to it. It provides information about its rendering capabilities and the XR runtime configuration, e.g the OpenXR configuration may be used for this purpose.

- In response, the split rendering server creates a description of the split rendering output and the input it expects to receive from the UE.

- The Presentation Engine requests the buffer streams from the MAF, which in turn establishes a connection to the split rendering server to stream pose and retrieve split rendering buffers.

- The Source Manager retrieves pose and user input from the XR runtime.

- The Source Manager shares the pose predictions and user input actions with the split rendering server.

- The split rendering server uses that information to render the frame.

- The rendered frame is encoded and streamed down to the MAF.

# 6	Prerequisites

## 6.1	Requirements on 5G System

## 6.2	Requirements on Device APIs and Functionality

MSE-7

# 7	Split Rendering Application Function

## 7.1	Functionality

## 7.2	RESTful APIs

### 7.2.1	Split-Rendering Provisioning procedures (SR-1)

#### 7.2.1.1	General

These procedures are used by the Application Provider and the AF on SR-1 to provision for the split-rendering process.

#### 7.2.1.2	Create Split-Rendering Configuration

This procedure is used by the AP to create a new Split-Rendering Configuration. The AP shall use the HTTP POST method for this purpose and the request message body shall include a SplitRenderingConfiguration resource.

If the procedure is successful, the AF shall generate a resource identifier representing the new Split-Rendering Configuration. In this case, the AF shall respond with a 201 (Created) HTTP response message and shall provide the URL to the newly created resource in the Location header field. The response message body may include a SplitRenderingConfiguration resource that represents the current state of the Split Rendering Configuration, including any fields set by the AF.

If the procedure is not successful, the AF shall provide a response code as defined in clause XXYY.

#### 7.2.1.3	Read Split-Rendering Configuration properties

This procedure is used by the AP to obtain the properties of an existing Split-Rendering resource from theAF. The HTTP GET method shall be used for this purpose.

If the procedure is successful, the AF shall respond with a 200 (OK) response message that includes the SplitRenderingConfiguration resource in the response message body.

If the procedure is not successful, the AF shall provide a response code as defined in clause XXYY.

#### 7.2.1.4	Update Split-Rendering Configuration properties

The update operation is invoked by the AP to modify the properties of an existing SplitRenderingConfiguration resource. All writeable properties except domainNameAlias may be updated. The HTTP PATCH or HTTP PUT methods shall be used for the update operation.

If the procedure is successful, the AF shall respond with a 200 (OK) and provide the content of the resource in the response, confirming the successful update operation.

If the procedure is not successful, the AF shall provide a response code as defined in clause XXYY.

#### 7.2.1.5	Destroy Split-Rendering Configuration

This operation is used by AP to destroy a Split-Rendering Configuration resource and to terminate the related distribution. The HTTP DELETE method shall be used for this purpose. As a result, the AF will release any associated network resources, purge any cached content, and delete any corresponding configurations.

If the procedure is successful, the AF shall respond with a 200 (OK) response message.

If the procedure is not successful, the A.F shall provide a response code as defined in clause XXYY.

### 7.2.2		Split-Rendering Provisioning API

### 7.2.2.1	Overview

This clause specifies the API that the AP uses at interface SR-1 to provision and manage the SRC though the AF. Each such configuration is represented by a SplitRenderingConfiguration, for which the resource structure is specified in 7.2.2.2 and the data model is specified in clause 7.2.2.3.

### 7.2.2.2	Resource structure

The Split-Rendering Provisioning API is accessible through this URL base path:

{apiRoot}/3gpp-rtc1/{apiVersion}/provisioning-sessions/{provisioningSessionId}/

Table 7.2.2-1 below specifies the operations and the corresponding HTTP methods that are supported by this API. In each case, the Provisioning Session identifier shall be substituted into {provisioningSessionId} in the above URL template and the sub-resource path specified in the second column shall be appended to the URL base path.

Table 7.2.2-1: Operations supported by the Split-Rendering Provisioning API

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create Split-Rendering Configuration | content-hosting-configuration | POST | Used to create a Split-Rendering Configuration resource. |
| Retrieve Split-Rendering Configuration |  | GET | Used to retrieve an existing Split-Rendering Configuration. |
| Update Split-Rendering Configuration |  | PUT, PATCH | Used to modify an existing Split-Rendering Configuration. |
| Delete Split-Rendering Configuration |  | DELETE | Used to delete an existing Split-Rendering Configuration. |
| Purge Split-Rendering Configuration cache | content-hosting-configuration/purge | POST | This operation is used to invalidate some or all cached media resources associated with this Split-Rendering Configuration. |

### 7.2.2.3	Data model

#### 7.2.2.3.1	SplitRenderingConfiguration resource

The data model for the SplitRenderingConfiguration resource is specified in table 7.2.2-2 below:

Table 7.2.2-2: Definition of SplitRenderingConfiguration resource

| Property name | Data Type | Cardinality | Description |
| --- | --- | --- | --- |
| Name | String | 1..1 | A name for this Split Rendering Configuration. |
| Status | Boolean | 1..1 | Indicates whether to the SRS shall use Split-Rendering if possible |
| Configuration | Object | 1..1 | Describes the split-rendering configuration currently used by the SRS. Editor’s Note: The syntax and semantics of this element are TBD. |

# 8	Split Rendering User Plane

MSE-4

## 8.1	Split Rendering Signalling Protocols

## 8.2	Split Rendering Formats

### 8.2.1 	General

### 8.2.2 	Pixel Streaming Profile

#### 8.2.2.1 	Overview

The full-prerendering profile is restricted to the support of 2D content exclusively. The capabilities of the receiving UE are shared with the split rendering server prior to the start of the split rendering session. These capabilities and configurations would indicate the audio-visual output setup on the UE. For example, it would indicate that the output device is an HMD that supports 2 views and stereo audio.

#### 8.2.2.2 	Downlink Formats

supported view configurations are:

- Mono: a single view

- Stereo: one view per eye

The following composition layers are supported:

- Projection: projection of the scene to a 2D plane using a perspective camera

- Quad: a 2d surface that is composed in the 3D space by the XR runtime

- Equirectangular: an equirectangular projection of the 3D space that is usually used to provide a background

- Cubemap: a set of 6 swapchain images that represent a projection of the 3D scene onto a cube

Each swapchain image will have the following properties:

- Format: RGB, RGB with Alpha (RGBA), and single-channel Depth formats with different precisions. RGB may be recovered from the coded YUV video stream. Depth information may be coded a separate video stream.

- Dimension: width and height of the swapchain image

- Mipmap: count of the level of detail of the swapchain image. The swapchain images maybe created at the UE side. Some Graphics Engines expect that the image dimensions are a power of 2.

For audio, the following formats are to be supported:

- Stereo audio mixed and binauralized based on the viewer’s current pose

- HOA audio mixed based on the viewer’s current position that extracted from the pose.

Editor’s Note: Referencing the downlink formats in MeCAR TS 26.119, potentially to OpenXR formats, is TBD.

#### 8.2.2.3 	Uplink Formats

The rendering process relies on the reception of pose predictions and user input. The pose information is formatted as follows:

- An array of multiple pose predictions

- Each pose prediction consists of a position and orientation component as a 3D (coordinates) and 4D (quaternion) vectors respectively.

- The prediction timestamp associated with the predicted pose

- An XR space for which the pose is created. If not present, this defaults to the viewer’s XR space.

Editor’s Note: Referencing the uplink formats in MeCAR TS 26.119, potentially to OpenXR formats, is TBD.

## 8.3	Split Rendering Transport Protocols

## 8.4 	Split Rendering Formats for Session Setup and Negotiation

### 8.4.1 	General

In Figure 5.2.1-1 and 5.2.1-2, step 5 defines the negotiation between the SRC and SRS for the split-rendering configuration. In most simple case, the SRC provides SRS the capabilities of the device and if SRS can accommodate the split-rendering processing that addresses the device, it confirms the configuration. In such scheme, the SRS is responsible to make the decision and no back-and-forth negotiation occurs.

### 8.4.2 	Split Rendering Configuration Format

#### 8.4.2.1	Introduction

The Split Rendering client establishes an XR session locally based on the device configuration and user selection. The SR client defines the view configuration (e.g. mono or stereo views), the projection format (such as projection, equirectangular, quad, or cubemap), the swap chain image configuration, etc.

In addition, XR space and action configurations are negotiated between the SR client and server. This includes defining common XR spaces and defining and selecting actions and action sets.

The format is extensible to support the exchange of additional/future configuration information.

#### 8.4.2.2	 Split Rendering Configuration Format

The session configuration information shall be in JSON format. It shall have the following format:

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| spaceConfiguration | Object | 0..1 | The space configuration is typically sent by the split rendering server to the split rendering client. Upon reception of this information, the SR client uses this information to create the reference and action spaces as well as to agree on common identifiers for the XR spaces. |
| referenceSpaces | Array | 0..1 | An array of reference spaces and their identifiers. |
| id | number | 1..1 | A unique identifier of the XR space in the context of the split rendering session. |
| refSpace | enum | 1..1 | One of the defined reference spaces in OpenXR. These may be: XR_REFERENCE_SPACE_TYPE_VIEW, XR_REFERENCE_SPACE_TYPE_LOCAL, or XR_REFERENCE_SPACE_TYPE_STAGE. |
| actionSpaces | Array | 0..1 | An array of action spaces that need to be defined by the split rendering client in the XR session. |
| id | number | 1..1 | A unique identifier of the XR space in the context of the split rendering session. |
| actionId | number | 1..1 | Provides the unique identifier of the action. |
| subactionPath | string | 1..1 | The subaction path identifies the action, which can then be mapped by the XR runtime to user input modalities. |
| initialPose | Pose | 0..1 | Provides the initial pose of the new XR space’s origin. |
| viewConfiguration | Object | 0..1 | Conveys the view configuration that is configured for the XR session. |
| type | Enum | 1..1 | The type indicates the view configuration. Defined values are MONO and STEREO. Other values may be added. |
| width | number | 1..1 | The recommended width of the swapchain image. |
| height | number | 1..1 | The recommended height of the swapchain image. |
| compositionLayer | string | 1..1 | An identifier of the selected composition layer. |
| actionConfiguration | Array | 0..1 | This contains a list of the actions that are to be defined by the SR client. |
| action | Object | 1..n | A definition of a single action object. |
| id | number | 1..1 | A unique identifier of the action. |
| actionType | enum | 1..1 | The type of the action state. This can be a Boolean, float, vector2, pose, vibration output, etc. |
| subactionPaths | string | 1..n | An array of subaction paths associated with this action. The split rendering client will provide the state of all defined sub-action paths. |
| extraConfigurations | Object | 0..1 | A placeholder for addition configuration information. |

# 9	Split Rendering Client

## 9.1	Functionality

Reference pre-requisites, user plane functionality, control plane

## 9.2	Client API

MSE-6

# 10 	Security and Privacy Aspects

## 10.1 	Security

Signaling for session establishment and exchange of application-specific messages shall use a secure transport channel based on WebSockets as defined in TS26.113.

Media transport shall be secured by the usage of WebRTC.

## 10.2 	Privacy

Editor’s Note: Privacy considerations are FFS.

# Annex A	 (Informative):
Implementation Guidelines

## Annex A.1 Guidelines for Application Developers

## Annex A.2 Guidelines for Split Rendering MSE Implementers

## Annex A.3 Conformance Testing

Annex B (normative):
IDL Definition of Client API

Annex X (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |