Agenda item: 	10.8

Source: 	InterDigital Communications and Qualcomm Inc.

Title: 	[5G_RTP] End-of-burst marking in PDU Set RTP HE

Document for:	Discussion and Agreement

# Introduction

In S4aR230057 the RTC working group has agreed to work on signaling the end-of-burst indication in PDU set information in an RTP header extension.

In this contribution, we discuss the syntax and semantics of end-of-burst signaling in RTP HE and the guidelines on how to set this field in the RTP HE at the application level.

# Data burst

S2-2303879 defines a data burst “A set of multiple PDUs generated and sent by the application in a short period of time”. The definition also includes a note that states “A Data Burst can be composed of one or multiple PDU Sets”.

The concept of a Data Burst is relevant when discussing traffic that is associated with extended reality (XR) services. It is important because XR traffic is often transmitted in bursts. For example, an Application Server (AS) may send a burst of data to a UE. The burst of data may represent audio information, video information, etc. After transmitting the burst of the data to the UE, a period of time may elapse before the Application Server transmits another data burst to the UE.

In S2-2303845, SA2 has agreed that:

- “An indication of End of Data Burst may be provided to the NG-RAN by the UPF, e.g., to configure UE power management schemes like connected mode DRX.” And

- “According to the request and information from the SMF, the UPF identifies the last PDU of a Data burst in the DL traffic based on the End indication according to the Protocol Description and provides an End of Data Burst indication to the NG-RAN over GTP-U of the last PDU of a Data burst.”.

The SMF configuration is based on a protocol description within the PCC Rules. S2-2303845 explains that:

- “The PCF may provision the Protocol Description within PCC Rules based on the information provided by the AF and/or the local operator policies.”

One such mechanism is to indicate the EOB indication in the PDU set information RTP Header Extension. The AS can set the EDB field in the PDU set information RTP HE and inform the UPF about the EOB. The UPF may send an EOB Indication to NG-RAN. When the NG-RAN receives the EOBI in the GTP-U header it understands that that specific PDU is the last PDU of a given Data Burst and it may put the UE to sleep.

# Proposal

The end-of-burst indication can be signalled in the PDU Set information RTP header extension.

  - End-of-Burst signaling

The syntax and semantics of one-byte and two-byte RTP header extension for the marking of PDU sets and end of data burst is as follows:

    - Syntax

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0xBE    |    0xDE       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   | L=5   |E| EDB | PRI   |      PSSN         |     PSN   |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                            PSSize             |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 1. RTP header extension using one-byte header format.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0x10    |    0x00       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID           | L=6           |E| EDB | PRI   |      PSSN

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|   PSN     |                    PSSize                     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 2. RTP header extension using two-byte header format.

    - Semantics

The semantics of the EDB field of the RTP header extension is defined as follows:

- EDB (3 bits): The EDB field is 3-bits in length with the following semantics:

| Value | Description |
| --- | --- |
| 000 | End of data burst marking is not set for this RTP stream and RTP payload type. |
| 001 | This PDU is not the last PDU of the data burst. |
| 010 | This value marks the last PDU of the data burst. |
| 011 | This PDU is part of the last PDU set of the data burst. The end-of-burst indication is signalled for all or some of the last PDUs in the last PDU set of the data burst. |

When the last PDU in a data burst is not delivered to the UPF due to congestion or unstable network conditions, indicating all or some of the last PDUs in the last PDU set of the data burst with EDB flag value 0x011 helps the UPF in identifying the end of the data burst.

  - Guidelines for setting EOB field

The end of a burst in a media stream may be identified by setting the RTP HE EOB field to 0x01 for the last PDU of an Intra Random Access Pictures (IRAP) picture. The IRAP frames in a media stream are larger in size and usually require several PDUs and PDU sets to transmit the coded frame data.

The end-of-burst may be indicated for the last PDU of each coded picture.

The end-of-burst may be indicated for all or some of the last PDUs in the last PDU set of a coded frame, using RTP HE EOB field value 0x11. In this case, the UPF/RAN may detect an EOB when it completely receives the PDUs of the last PDU set (e.g., by comparing the number of bytes received for this PDU set with the PDU set size or checking if the E flag in the RTP HE is set.)

An Application Server (AS) acting as a relay may add PDU set information RTP header extension before transmitting the PDUs to the UPF. In this process, the AS may not know the media timestamp directly. In such case, the AS relies on the RTP timestamp present in the RTP header of the PDUs to calculate the media timestamp.

Typically, all PDUs that are associated with a burst should be associated with the same presentation time stamp. In case of multiple media streams present in a stream, the PDU sets that are associated with the same media time stamp considering the presentation time stamp and the duration of the frame across all the media shall be part of the data burst. To determine end-of-burst, when two or more media flows are part of the same burst, the timestamps in the RTP packet header are not compared directly to each other. Rather, they are compared to a reference clock to determine if they were transmitted at the same time.

In PDUs carrying video NAL units, the end-of-burst may also be detected when the marker (M) bit in the RTP header is set. The marker bit is intended to allow significant events such as frame boundaries to be marked in the RTP packet stream when they carry video NAL units. When a marker bit is set in an RTP header of an RTP packet carrying video NAL units, it is interpreted that the frame boundary is reached and the corresponding PDU and/or PDU set is marked with end-of-burst indication in the RTP HE.

# Proposal

We propose to agree the proposed syntax, semantics, and guidelines for marking the end-of-burst indication to the PDU set information RTP header extension in 5G_RTP permanent document.

# References

- S2-2303879, 23.501 CR3759R6 (Rel-18, 'B'): Support of XR and Media Services, Tencent, et al.

- S2-2303845, 23.501 CR3875R14 (Rel-18, 'B'): Introduction of support for Jitter Measurement and End of Data Burst reporting to the NG-RAN, Ericsson, et al.