3GPP TSG-SA WG4 Meeting #124	S4-230825

Berlin, Germany, 22 – 26 May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.522 | CR |  | rev |  | Current version: | 0.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR on guidelines for marking PDU set importance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Communications, Samsung Electronics Co., Ltd., HUAWEI and KDDI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_RTP |  |  |  |  |  | Date: |  |  | 16-05-2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 1st Change

#### 4.4.2.6	Guidelines for PDU Set Marking

##### 4.4.2.6.2         PDU Set Importance Field

4.4.2.6.2.1        General

The PDU sets that contain audio data are set with highest importance compared with other media PDU sets. PDU sets that belong to audio media are set with an importance value b0000.

NOTE: PDU sets that carry immersive audio data are not set with highest importance compared with other media PDU sets. The importance value of immersive audio PDU sets is FFS.

The PDU sets that contains the reference frames present in the video bitstream are set with higher importance compared with non-reference frames present in the video bitstream. The Intra Random Access Pictures (IRAP) pictures such as Instantaneous Decoder Refresh (IDR) frames, Clean Random Access (CRA) frames, Broken Link Access (BLA) frames and Gradual decoding refresh (GDR) frames are very important in a video stream and are set with higher importance. The importance value for such pictures can be set to b0001.

In video coding, temporal scalability is the option to decode only some of the frames in a video stream instead of the whole stream. This enables a media server to reduce the bitrate sent towards viewers who doesn’t have enough bitrate or CPU to handle the whole stream. Pictures with lowest temporal identifier value are used as reference pictures in the bitstream and are important for decoding the dependent frames.

The following clauses provides the guidelines on setting the importance field in a PDU set RTP header extension for various video codecs.

4.4.2.6.2.2        H.264 Codec

In an H.264 bitstream, NAL units with the nal_unit_type field assigned the value 5 (refer to Table 7.1 in AVC specification [2]) are Intra Random Access Pictures (IRAP) pictures. When the Type field value in the NAL Unit header of an RTP packet is 5, then the corresponding PDUs in that PDU set are set with higher importance value for example with b0001.

The parameter set NAL units such as Sequence Parameter Set (SPS) and Picture Parameter Set (PPS) are important for decoding the bitstream. Therefore, PDU sets with a payload Type field value equal to 7, 8, 13 or 15 (refer to Table 7.1 in AVC specification [2]) in the NAL Unit header of the RTP packet are set with higher importance. The importance value for such PDU sets is set to b0001.

+---------------+

|0|1|2|3|4|5|6|7|

+-+-+-+-+-+-+-+-+

|F|NRI|  Type   |

+---------------+

Figure 1. NAL unit type octet in an RTP packet payload

The NAL unit type octet contains the NRI (nal_ref_idc) field highlighted in Figure 1. A value of b00 indicates that the content of the NAL unit is not used to reconstruct reference pictures for inter picture prediction. Such NAL units can be discarded without risking the integrity of the reference pictures. Values greater than b00 indicate that the decoding of the NAL unit is required to maintain the integrity of the reference pictures.

PDU sets with an NRI field value 0x00 in the NAL Unit header of RTP packet are of lowest important. The importance value in the PDU set header extension for such PDU sets is set to b1111.

The Type and NRI field in the NAL unit header indicates the relative transport priority. They can be used to be set the PDU Set importance. While they can also indicate different QoS requirements, which can be used to provide different protects against transmission losses, e.g. reliabilities (tolerable frame/slice error rate), priorities.

4.4.2.6.2.3        HEVC Codec

In an HEVC bitstream, NAL units with the nal_unit_type field assigned a value in the rang 16 to 23 (inclusive) (refer to Table 7.1 in HEVC specification [3]) are Intra Random Access Pictures (IRAP) pictures. When the Type field value in the NAL Unit header of RTP packet is in the range 16 to 23 (inclusive), then the corresponding PDUs in that PDU set are set with higher importance value for example with b0001.

The parameter set NAL units such as Sequence Parameter Set (SPS), Picture Parameter Set (PPS), Video Parameter Set (VPS) are important for decoding the bitstream. Therefore, PDU sets with payload Type field value in the NAL Unit header of RTP packet in the range 32 to 34 (inclusive) are set with higher importance. The importance value for such PDU sets is set to b0001.

+---------------+---------------+

|0|1|2|3|4|5|6|7|0|1|2|3|4|5|6|7|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|F|   Type    |  LayerId  | TID |

+-------------+-----------------+

Figure 2.  The Structure of the HEVC NAL Unit Header

In HEVC bitstreams, RASL pictures can be discarded. HEVC provides mechanisms to enable specifying the conformance of a bitstream wherein the originally present RASL pictures have been discarded. Consequently, system components can discard RASL pictures, when needed, without worrying about causing the bitstream to become non-compliant.

PDU sets with Type field value equal to 8 or 9 (refer to Table 7.1 in HEVC specification [3]) in the NAL Unit header of RTP packet are RASL pictures and they are of lowest importance. The importance value in the PDU set header extension for such PDU sets is set with high value or b1111.

Besides, PDU sets with TID value 1 (lowest possible value) as shown in Figure 2 can be set with higher importance. The importance value for such pictures may be set to b0001 (for IRAP pictures) or b0010 (for non-IRAP pictures).

Pictures with highest TID value cannot be used as reference pictures and can be discarded at the network level when the throughput is not good, or network conditions are unstable. PDU sets with higher TID values in the NAL Unit header of RTP packet or with higher nuh_temporal_id_plus1 value in the NAL unit header of the bitstream are set with higher importance value compared with the PDU sets with lower TID values.

PDU sets with the highest TID value in the NAL Unit header of RTP packet or with highest nuh_temporal_id_plus1 value in the NAL unit header of the bitstream are set with lowest importance. The importance value for such pictures may be set to b1111.

The Type and TID field in the NAL unit header indicates the relative transport priority. They can be used to be set the PDU Set importance. While they can also indicate different QoS requirements, which can be used to provide different protects against transmission losses, e.g. reliabilities (tolerable frame/slice error rate), priorities.

4.4.2.6.2.4        PDU set importance based on affected PDU sets

When the transport layer is forced to perform immediate dropping/discarding of a PDU set but has a freedom of selection among the PDU sets, the PDU set with smaller degrees of artifact would be the better choice in most cases. Dropping of a PDU set may corrupt the decoded output of itself and the other PDU sets though they may already be transmitted perfectly to the receiving end or yet in a queue waiting to be transmitted. The degrees of artifact can be explicitly transferred from the application layer to the transport layer as the number of affected frames which precedes/follows the PDU set, or can be implicitly transferred as the importance value where the lower value means the higher PDU sets are affected while higher values proportionally mean less number of PDU sets are affected for example. By considering such a quantization of various affected PDU sets can be translated into importance field, using 4 bits to represent 16 possible size ranges is recommended.

The information on the size of propagation error which caused by the dropping of each PDU set may be provided by the application layer to the transport layer. The information may present the size of error propagation implicitly with a proportional mapping of error propagation size to an index such as the importance of the PDU set in the media stream.

The importance value of a PDU Set in PDU set information RTP HE is set as follows

- The error propagation size is mapped to importance field value. For e.g., importance value 15 (b1111) means one frame (self) is affected, importance value 0 means 15 or more frames are affected.

## End of 1st Change

## 2nd Change

# 2	References

- H.264: Advanced video coding specification. https://www.itu.int/rec/T-REC-H.264-202108-I/en

- H.265: High efficiency video coding specification. https://www.itu.int/rec/T-REC-H.265-202108-I/en

## End of 2nd Change