Source:		Samsung Electronics

Title:			Discussion on alignment of MSH variants

Agenda Item:		6.1

Document for:		Agreement

# 1.	Background

Among the various work items in SA4, it was observed that the same terminology of MSH was frequently used without further clarification. In this input, we analysed the usage of this term and proposed the changes to avoid the uncertainty.

# 2.	Variant of MSH and its API(s)

Here are the summary of work items / TSs where MSH was addressed.

| Work Item | TS (version) | Name | Functionalities |
| --- | --- | --- | --- |
| 5GMS | 26.501/512 | MSH | to establish, control and support the delivery of a media session, and that may perform QoE metrics reporting |
| SR-MSE | 26.565 (v0.4.0) | MSH | Same to GA4RTAR case |
| GA4RTAR | 26.506 (v1.2.0) | MSH | Almost similar to 5GMS MSH + additional to WebRTC support (e.g., changes in configuration info such as ICE, WebRTC signalling function) |
| MeCAR | S4-230729(1) | MSH | may be realized as a 5G-RTC MSH, 5GMS Media Session Handler, or any other function |

NOTE 1: MeCAR tdoc 729 was agreed as a baseline, and not implemented into TS.

Additionally, the table below is the APIs related to MSH identified.

| Work Item | TS (version) | API | from | to | Functionalities |
| --- | --- | --- | --- | --- | --- |
| 5GMS | 26.501/512 | M5 | 5GMS AF | MSH | Dynamic policy, consumption reporting, network assistance, etc |
| SR-MSE | 26.565 (v0.4.0) | - | - | - | Referencing to TS 26.506 |
| GA4RTAR | 26.506 (v1.2.0) | RTC-5 | 5G-RTC AF | MSH | M5 + some changes (e.g,. some of context may be added to enable WebRTC support) |
| MeCAR | S4-230729 | IF-5 | 5GS | MSH | Referencing to TS 26.506 (and potentially to 512 as well) |
| 5GMS | 26.501/512 | M1 | 5GMS AF | App provider | Provisioning, content preparation, protocol discovery, etc. |
| SR-MSE | 26.565 (v0.4.0) | SR-1 | RTC AF | App provider | Provisioning specific to split-rendering (SplitRenderingConfiguration) |
| GA4RTAR | 26.506 (v1.2.0) | RTC-1 | 5G-RTC AF | App provider | M1 + some changes (e.g., provisioning of ICE function, MCU/SFU, etc) |
| MeCAR | S4-230729 | N/A (Out of scope) |  |  |  |

# 3.	Proposal

Considering the current definition from each document, it is observed that “MSH in GA4RTAR” should be changed to differentiate one from 5GMS for the following reasons;

- RTC function / API should be different from those defined in 5GMS to host WebRTC-based service.

- They are interacting with different network functions (5GMS AF or 5G-RTC AF)

- They are exposing different APIs (e.g,. M5 vs. RTC-5, M1 vs. SR-1 vs. RTC-1)

- Updating the current 26.512 is not a good idea: versioning or compatibility issue, etc

Thus, it is proposed to changes as follows (marked in red);

| Work Item | TS (version) | Name | Changes |
| --- | --- | --- | --- |
| 5GMS | 26.501/512 | MSH | No change |
| SR-MSE | 26.565 (v0.4.0) | RTC MSH | Same to GA4RTAR change |
| GA4RTAR | 26.506 (v1.2.0) | RTC MSH(1) | Differentiate with the MSH in 5GMS. Details are to be addressed in next version of TS 26.506 |
| MeCAR | S4-230729 | MSH | No change. Referencing |

NOTE 1) The prefix “5G-“ is removed as it does not give any additional value. Same work will apply to all terms in TS 26.506.

| Work Item | TS (version) | API | from | to | Changes |
| --- | --- | --- | --- | --- | --- |
| 5GMS | 26.501/512 | M5 | 5GMS AF | MSH | No change |
| SR-MSE | 26.565 (v0.4.0) | - | - | - | No change. May reference to TS 26.506 |
| GA4RTAR | 26.506 (v1.2.0) | RTC-5 | RTC AF | RTC MSH | Details to be addressed in a new TS (under extended GA4RTAR) (1) |
| MeCAR | S4-230729 | IF-5 | 5GS | MSH | May refer to both 26.512 and 26.506 |
| 5GMS | 26.501/512 | M1 | 5GMS AF | App provider | No change |
| SR-MSE | 26.565 (v0.4.0) | SR-1 | RTC AF | App provider |  |
| GA4RTAR | 26.506 (v1.2.0) | RTC-1 | RTC AF | App provider |  |
| MeCAR | S4-230729 | N/A (Out of scope) |  |  |  |

NOTE 1: See another input to update GA4RTAR WID (S4-230840)

# 4.	Reference

[1]	TS 26.501 v18.1.0, 5G Media Streaming (5GMS); General description and architecture

[2]	TS 26.512 v17.4.0, 5G Media Streaming (5GMS); Protocols

[3]	S4-230727, Draft TS 26.565 v0.4.0, Qualcomm

[4]	S4-230661, Draft TS 26.506 v1.2.0, Samsung

[5]	S4-230729, Merged Draft Specification, Qualcomm, Xiaomi, Tencent