Source:	Samsung Electronics Co., Ltd.

Title:	Reports for PDU Set feature

Document for:	Discussion and Agreement

Agenda Item:	10.8

- Introduction

This contributions proposes to update the considerations on reports for PDU Set feature in clause 6.5 of 5G_RTP PD.

- UE Centric procedure

  - 6.5. Considerations on Reports for PDU Set feature

    - 6.5.1 Available information on PDU Sets

The RTP Header Extension for PDU Set marking provides the following information:

-	End PDU of the PDU Set [E] (1 bit)

-	End of Data Burst [EDB] (3 bits)

-	PDU Set Importance [PSI] (4 bits)

-	PDU Set Sequence Number [PSSN] (10 bits)

-	PDU Sequence Number within a PDU Set [PSN] (6 bits)

-	PDU Set Size [PSSize] (24 bits)

Under the assumption that the above information will be provided by RTP header extension(s), it is expected that an RTP receiver can acquire the following information for an interested time window:

-	Number of completely received PDU Sets,

-	Fraction/Number of incompletely (or completely) received PDU Sets, and

[-	An estimate of the statistical variance of the PDU Set interarrival time]

Under the same assumption, the following information for each PDU Set can be available in an RTP receiver:

-	Indication whether a PDU Set has been completely received,

-	Number of received PDUs in a PDU Set,

-	Fraction/Number of lost PDUs in a PDU Set (when the number of PDUs in a PDU Set is known by the RTP receiver, e.g., when it receives the end PDU of the PDU Set)

[-	PDU Set delay for a completely received PDU Set]

It is noted that the above information can also be organized on a PDU Set Importance basis.

    - 6.5.2 Delivery of Reports on PDU Sets

There are number of possible ways to encapsulate the information discussed in section 6.5.1 into RTCP packets including profile-specific extensions to the sender (PT=200) and receiver report (PT=201) [2], Application-defined RTCP packet (PT=204) [2], Extended Report (XR; PT=207) [3], Generic RTP Feedback (RTPFB; PT=205) [4] and Payload-specific Feedback (PSFB; PT= 206) [4]. As the PDU Set feature should be supported across RTP profiles and payload types, it is recommended to use Extended Report (XR; PT=207) [3] and/or Generic RTP Feedback (RTPFB; PT=205) [4] as a container(s) for PDU Set-based RTCP feedback.

The format of an XR packet with a single report block is as follows [3]:

An XR packet consists of a header of two 32-bit words, followed by a number, possibly zero, of extended report blocks which comprises of BT, type-specific, block length and type-specific block contents. Extended report blocks can be stacked, one after the other, at the end of an XR packet. The value of BT (block type) field identifies the block format and its name space is managed by IANA:

https://www.iana.org/assignments/rtcp-xr-block-types/rtcp-xr-block-types.xhtml

The packet format of an RTCP Feedback Message is as follows [4]:

The PT (payload type) field identifies the packet as being an RTCP FB message. Currently two values are defined by the IANA: PT=205 for RTPFB (transport layer FB message) and PT=206 for PSFB (payload-specific FB message). The FMT (feedback message type) field identifies the type of FB message and is interpreted relative to the payload type (i.e., RTPFB or PSFB). The FMT values for both the RTPFB payload type and the PSBF payload type are managed by IANA:

https://www.iana.org/assignments/rtp-parameters/rtp-parameters.xhtml

NOTE: The detailed format for PDU Set-based RTCP feedback is FFS as it may depend on other aspects of the PDU Set feature including RTP header extension and usage scenarios.

- Proposal

We propose to update the clause 6.5 of 5G_RTP PD according to the text in clause 2 of this contribution.