Source:	Samsung Electronics Co., Ltd.

Title:	UE centric procedure for AR communication

Document for:	Discussion and Agreement

Agenda Item:	10.6

- Introduction

This contributions proposes UE centric procedure for AR communication.

- UE Centric procedure

In UE centric AR communication, AR media traffic is transparent to the IMS network and exchanged between the two peer UEs via the deployed media functions (e.g., IMS AGW/TrGW).

Figure 4.1.X.1 shows a typical procedure to establish a UE centric AR IMS session from UE-A perspective.

Figure 4.1.X.1 Basic AR Call Flow

1.	UE-A initiates an IMS communication with UE-B, including establishment of bootstrap data channel.

2.	The user of UE-A upgrades the IMS session with AR experience. 
UE-A initiates a re-INVITE adding media descriptors required by the AR application to be established E2E, e.g. RTP and/or application data channels. Application data channel may be anchored in the MF/MRF.

3.	UE-A captures and encodes AR media.

4.	UE-A sends the AR media to the peer through the established media connection(s).

5.	UE-A receives AR media from the peer through established media connection(s).

6.	UE-A decodes and displays the received AR media on its screen.

In Step 2, a scene description can be generated and distributed by the MF/MRF using an application data channel of the AR application. The distribution of the scene description using a bootstrap data channel using “mpeg-sd” sub-protocol, as scene description-based overlay for ITT4RT, is FFS.

- Proposal

We propose to integrate the text in clause 2 to the next version of the IBACS PD.