3GPP SA4 #124	S4- 230847

Berlin, 22-26 May 2023

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.806 | CR | pseudo | rev | - | Current version: | 1.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_SmarTAR] Methods for achieving desired end-to-end packet error rate and bit rate |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_SmarTAR |  |  |  |  |  | Date: |  |  | 15/05/2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The TR26.806 mentioned end-to-end QoS metrics including packet error rate and bit rate without saying whether it is feasible to provision such QoS metrics. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Providing potential solutions for achieving desired end-to-end packet error rate and bit rate. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TR is not complete without the proposed change. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

===== BEGIN OF CHANGE  =====

#### 6.1.2.2	Other End-to-end QoS metrics

For other end-to-end QoS metrics, such as packet error rate and bit rate, the 5G network can adjust the metrics within the 5G network to achieve desired end-to-end metrics.

First, consider the packet error rate. Assuming that the packet error rate in the 5G network (denoted pc) and the effective packet error rate in the non-5G networks (denoted pn) are independent, the end-to-end packet error rate (denoted pe2e) is then

pe2e=1-1-pc1-pn.

This suggests that if pn can be estimated, then the 5G network can adjust pc to achieve a desired value for pe2e. The above equation also suggests a method to estimate pn: if pe2e is measured, then pn can be solved for as

pn=pe2e-pc1-pc

Thus a method for the 5G network to set a packet loss rate to achieve a desired end-to-end packet loss rate is as follows:

- The 5G network estimates the packet error rate in the 5G network pc.

- The end-to-end packet error rate pe2e is measured.

- The collective packet error rate of the non-5G networks pn is computed according to pn=(pe2e-pc)/(1-pc )

- The 5G network adjusts the packet error rate within he 5G network to pc' to meet the desired end-to-end packet error rate pe2e, desired

pe2e, desired=1-1-pc'1-pn.

Note that to measure the end-to-end packet error rate, the required number of increases with the packet error rate. For example, if the end-to-end packet error rate is 0.1%, there is one packet error event expected to occur in 1000 packets. This implies a slow response when the packet error rate is low. This is different from delay measurement, where a single packet can provide a valid measurement regardless of the range of the delay.

Next consider the bit rate. The bit rate depends on the bit rate allocation of the network segments in the end-to-end path and rate control and congestion control. However, there is a simple relationship among the maximum allowed bit rates that can be exploited to determine the bit rate allocation. It is observed that the maximum allowed end-to-end bit rate (re2e) is the minimum of the maximum allowed bit rate in the 5G network (rc) and the maximum allowed bit rate in the non-5G networks (rn), i.e.,  re2e=min(rc, rn). Thus the 5G network only needs to provide an maximum allowed bit rate that is higher than the desired end-to-end bit rate.

===== END of CHANGE  =====