| 3GPP TR 26.806 V1.2.0 (2023-04) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Tethering AR Glasses – Architectures, QoS and Media Aspects (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Motivation and Background	9

4.1	Summary of TR 26.998	9

4.2	Guiding Use Cases	9

4.2.1	Introduction	9

4.2.2	Cycling Glasses	9

4.3	PIN (Personal IoT Network)	10

4.4	Device Architectures for Tethered Glasses	13

4.4.1	General	13

4.4.2	Tethered Standalone AR Glasses	15

4.4.3	Tethered Display AR Glasses	16

4.4.4	Tethered AR Glasses with 5G Relay	17

5	System Architectures and Call Flows	18

5.1	System Architecture	18

5.2	Call flows	18

6	Identified Key Issues and Potential Solutions	23

6.1	Key Issue #1: How to provide End-to-End QoS for the 5G relay architecture	23

6.1.1	Description of the key issue	23

6.1.2	Potential solution	24

6.1.2.1	End-to-end latency	24

6.1.2.2	Other End-to-end QoS metrics	24

6.2	Key Issue #2: How to determine the non-5G delay for the 5G relay architecture	24

6.2.1	Description of the key issue	24

6.2.2	Solution: Segment-by-segment delay measurement	25

6.2.3	Solution: End-to-end delay measurement	25

6.2.4	Time measurement protocol	29

6.3	Key Issue #3: What and how to report the non-5G delay	29

6.3.1	Description of the key issue	29

6.3.2	Potential solutions	29

6.4	Key Issue #4: Formats and Connectivity of Tethered Glass	31

6.4.1	Description	31

6.4.2	Background	31

6.4.3	Assumptions	33

6.4.4	Problem Statements	34

6.4.4.1	 General	34

6.4.4.2	 Case 1: Restricted Raw formats	34

6.4.4.3	 Case 2: Transcoding Problem	34

6.4.5	Potential Solutions	34

6.4.5.1	Layer-2 or Layer-3 Relay	34

6.4.5.2	Raw Format adaptation	35

6.4.5.3	Pass-through Compressed Media Formats for XR Runtimes	35

6.4.6	Conclusions	37

6.5	Key Issue #5: Compute distribution across UE and network for tethered glasses	38

6.5.1	Introduction	38

6.5.2	Background	38

6.5.3	Assumptions	39

6.5.4	Problem Statement	39

6.5.5	Potential Solutions	39

6.5.6	Conclusions	39

6.6	Key Issue #6: Usage of PIN for Tethered Glasses	39

6.6.1	Description	39

6.6.2	Potential solutions	39

7	Potential Next Steps	40

8	Conclusions and Recommendations	40

Annex A (informative): QoS Control of Relay WLAR UE when 5G Sidelink Used for Tethering Link	40

Annex <X> (informative): Change history	42

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

For initial AR experiences, an expected prominent setup will be wirelessly tethered of AR glasses, typically connected to a 5G UE. The tethering technology between a UE and an AR Glasses may use different connectivity, for example provided through WiFi or 5G sidelink. Different architectures for tethering result in different QoS requirements, session handling properties, and also media handling aspects. For enhanced end-to-end QoS and/or QoE, AR glasses may need to provide functions beyond the basic tethering connectivity function. Generally, smartly tethering AR glasses is an important aspect for successful AR experiences using the 5G System.

Based on this, the present document introduces Smartly Tethering AR Glasses (SmarTAR) in such user experience may be maximized using the 5G System.

# 1	Scope

The present document addresses architectures, QoS and media handling aspects of when tethering AR Glasses to 5G UEs based on initial discussions in TR 26.998 [2]. In particular, the following aspects are in scope:

-	Definition of different tethering architectures for AR Glasses including 5G sidelink and non-5G access based on existing 5G System functionalities

-	Documentation of the relationship between AR Glasses tethering and AR glasses considered as PIN (Personal IoT Network) elements according to TR 22.859 [3] and the derived service requirements in TS 22.261 [4].

-	Documentation of end-to-end call flows for session setup and handling

-	Identification media handling aspects of different tethering architectures

-	Identification of end-to-end QoS-handling for different tethering architectures and define supporting mechanisms to compensate for the non-5G link between the UE and the AR glasses

-	Providing recommendations for suitable architectures to meet typical AR requirements such as low power consumption, low latency, high bitrates, security and reliability.

-	Collaboration with relevant other 3GPP groups on this matter

-	Identification of potential follow-up work on this matter

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 26.998: "Support of 5G glass-type Augmented Reality / Mixed Reality (AR/MR) devices".

[3]	3GPP TR 22.859: "Study on Personal Internet of Things (PIoT) networks".

[4]	3GPP TS 22.261: "Service requirements for the 5G system".

[5]	3GPP TR 23.700-78: "Study on Application layer support for Personal IoT and Residential Networks".

[6]	3GPP TR 23.700-88: "Study on architecture enhancements for Personal IoT Network (PIN)"

[7]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[8]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[9]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[10]	The Khronos Group, "The OpenXR Specification", https://registry.khronos.org/OpenXR/specs/1.0/html/xrspec.html

[11] 	3GPP TS26.531, “Data Collection and Reporting; General Description and Architecture”, V17.1.0, Sept. 2022.

[12]	 J. Postel, "RFC 792: Internet Control Message Protocol", 1981.

[13] 	IEEE-1588-2019, "Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control Systems".

# 3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

API	Application Programming Interface

AR	Augmented Reality

BLE	Bluetooth Low Energy

FFS	For Further Study

FLUS	Framework for Live Uplink Streaming

GPS	Global Positioning System

GPU	Graphics Processing Unit

MAF	Media Access Function

MSH	Media Session Handler

PCM	Pulse Code Modulation

PEGC 	PIN Element with Gateway Capability

PEMC 	PIN Element with Management Capability

PIN 	Personal IoT Network

PINAPP 	Personal IoT Network Application

PTP	Precision Time Protocol

QoS	Quality-of-Service

RGB	Red-Green-Blue

RTT	Round-Trip Time

STAR	Standalone AR glasses

WLAR	WireLess tethered AR glasses

WTAR	Wired Tethered AR glasses

# 4	Motivation and Background

## 4.1	Summary of TR 26.998

The 5G WireLess Tethered AR UE is introduced in [2] as one functional structural device type. It is further split into two sub-types, Type 3a: 5G Split Rendering WireLess Tethered AR UE and Type 3b: 5G Relay WireLess Tethered AR UE. For Type 3a, the tethering 5G Phone provides both the network connectivity and the rendering/pre-rendering assistant functionalities to the AR glass. For Type 3b, the tethering 5G Phone only provides the IP network connectivity to the AR glass.

Note:	The 5G Phone, as a tethering device, initiates the “tether” action to an AR Glasses which belongs to the tethered device.

Table 1:	Functionality splitting for Wireless Tethered AR Glasses device

| Functionality splitting | Type 3a: Split Rendering WLAR UE | Type 3b: Relay WLAR UE |
| --- | --- | --- |
| 5G connectivity | Tethering device | Tethering device |
| Media Access Function User Plane (Media Client) | Tethering device | AR Glass |
| Media Access Function Control Plane (Media Session Hander) | Tethering device | Tethering device |
| AR runtime | Local and uses from sensors, audio inputs or video inputs, but may be assisted by functionalities on tethering devices. | Local and uses from sensors, audio inputs or video inputs. |
| Media Processing | May be done on the AR glasses and energy intensive AR/MR media processing may be done on the AR/MR tethering device or split. | Either done on the Glasses device or it is split with the network. |

Editor’s Note: The Media Processing, including spatial computing and scene rendering, may be split among AR glasses, tethering 5G Phone and Edge AS in the network. How the media processing is splitted is FFS.

Different from other types of AR UE, the end-to-end path includes one more wireless/wireline tethering link between AR Glasses and the tethering 5G Phone. In order to fulfil the end-to-end QoS requirements for the AR session, the AR UE need to acquire the tethering link status via measurement tests or empirical values, and takes it into account when determining the QoS for the 5G system link. With the tethering link status, the Media Access Function may communicate with AF for dynamic QoS policy adjustment accordingly.

## 4.2	Guiding Use Cases

### 4.2.1	Introduction

This clause provides several guiding use cases in order to simplify the analysis in the course of this report.

### 4.2.2	Cycling Glasses

Assume the following use case: Thomas got new AR glasses for his 51st birthday. He plays around and can connect the glasses to his mobile phone using a dedicated WiFi connection. Thomas wants to go cycling so he uses the AR glasses. When Thomas goes cycling, he does use four main apps in parallel:

-	A navigation app that provides directions on where to go

-	A fitness app that tracks all different kinds of environmental as well as body data

-	A music app to play his favourite live music radio station

-	A live video sharing app such that his family and friends can track him and cheer him

In addition, as Thomas is a nerd, he also loves to get notifications from major apps such as on social media, work e-mails, Teams notifications from Imed, and so on. All these applications run on the phone and the phone sends notifications to the peripherals. The notifications create certain A(udio)/V(isual)/H(apical) signals. Of these additional apps, you may even then kick off and create and render additional information on the peripherals (headset, glass). Now wearing a Glasses during cycling, including an audio headset and some haptical notification.

Thomas’s wife is scared when he wears smart glasses on his bike and she wants to be absolutely sure that he does not get motion sick on the bike and possibly falls off.

In addition, the navigation app uses the camera feeds from the Glasses and GPS location of the phone to send exact navigation instructions. The camera feeds may have to use an edge for exact detection of the environment and may make the AR glasses provide overlays and information on the environment. But at the same time it will need to send notifications from other apps. These notifications may be any sense or combined senses. So the phone needs to render eye buffers, audio output and provide haptical information to the glasses.

## 4.3	PIN (Personal IoT Network)

## 4.3.1 Media share within PINs use case

“Personal IoT networks” (PINs) in 3GPP TR 22.859 [3] are of a type of private network typically consisting of a user smartphone, wearables and home automation devices. These networks are very different to commercial IoT device, they are usually less rugged, most highly battery constrained and lifespan of the battery typically a couple of days or weeks. User plane traffic typically stays with a constrained environment, around the body or in the home i.e., within the PIN. Notifications can be received on smartphones that events have occurred within the PIN. A typical wearable PIN is depicted in Figure 4.3.1.

Figure 4.3.1: Wearable PINs (from [3])

One use case in clause 5.3 of [3] is called "Media share within PINs". A sub-use case to be noted consists of watching a movie on a smartphone and then switching to watching the movie on AR glasses.

3GPP TR 22.859 [3] also includes Potential Consolidated Requirements to be considered for 5GS evolution. These potential requirements include control plane requirements such as Device and Service Discovery, Privacy and Security, PIN Management and Charging. It also includes user plane type of potential requirements such as Gateway capability, Direct Communications, Connectivity and QoS.

## 4.3.2 Definitions and service requirements of PIN

According to Service requirements for the 5G system 3GPP TS 22.261 [4], A Personal IoT Network (PIN) consists of PIN Elements that communicate using PIN Direct Connection or direct network connection and is managed locally (using a PIN Element with Management Capability). Examples of PINs include networks of wearables and smart home / smart office equipment.  Via a PIN Element with Gateway Capability, PIN Elements have access to the 5G network services and can communicate with PIN Elements that are not within range to use PIN Direct Connection. A PIN includes at least one PIN Element with Gateway Capability and at least one PIN Element with Management Capability.

PIN Element related definitions can be found from TS 22.261 [4]:

PIN Element: UE or non-3GPP device that can communicate within a PIN.

PIN Element with Gateway Capability: a UE PIN Element that has the ability to provide connectivity to and from the 5G network for other PIN Elements.

NOTE 5C:	A PIN Element can have both PIN management capability and Gateway Capability.

PIN Element with Management Capability: A PIN Element with capability to manage the PIN.

Service requirements for the 5G system TS 22.261 [4] include both control and user plane requirements such as:

-	General: including user plane connectivity requirements such as “the 5G system shall support a data path not traversing the 5G network for intra-PIN communications via direct connections.” And “The communication path between PIN Elements may include licensed and unlicensed spectrum as well as 3GPP and non-3GPP access.”

-	Gateways: including e.g. “The 5G system shall be able to support access to the 5G network and its services via at least one gateway (i.e. PIN Element with Gateway Capability […]) for authorised UEs and authorised non-3GPP devices in a PIN[…].”

-	Operation without 5G core network connectivity: “The 5G system shall allow PIN Elements to communicate when there is no connectivity between a PIN Element with Gateway Capability and a 5G network.  For a Public Safety PIN licensed spectrum may be used for PIN direct communications otherwise unlicensed spectrum shall be used.”

-	PIN discovery: e.g. “The 5G system shall enable a UE or non-3GPP device in a  […] PIN to discover other UEs or non-3GPP devices within the same  […] PIN subject to access rights.” and PIN element capability discovery.

Editor’s note: Although discovery is in the TS 22.261 [4], the agreement for normative work is to divide discovery into two layers: if transport layer is based on 3GPP PC5, the existing procedures defined for 5G ProSe Direct Communication are re-used. If the transport layer functionality based on non-3GPP communication specification is outside the 3GPP scope. The application layer for PIN and PIN Element discovery and selection is not specified by SA2.

-	Relay selection for PIN direct connection: “The 5G system shall support a mechanism for a PIN Element to select a relay for PIN direct connection that enables access to the target PIN Element.”

-	Authentication, Privacy and Security: “The 5G system shall provide user privacy; location privacy, identity protection and communication confidentiality for non-3GPP devices and UEs that are using the PIN Element with Gateway Capability […].”

-	QoS monitoring and control requirements don’t apply to PINs but to UEs in CPNs (Customer Premises Networks)

Editor's note:	Despite the lack of specific Stage 1 QoS requirements on PIN, Stage 2 includes a solution on QoS management for PINs: “Solution #11: Differentiated QoS between a PINE and 5GS when a PEGC is used for the relay”

-	Charging: “The 5G system shall support charging data collection for data traffic to/from individual UEs in a […] PIN (i.e., UEs behind the PIN Element with Gateway Capability […])”

-	PIN Creation and Management: “The 5G system shall support mechanisms for a network operator or authorized 3rd party (e.g., a PIN User) to create, remove and manage a PIN”

## 4.3.3 PIN Architecture related study in 3GPP

There’s existing study in 3GPP regarding PIN network and application architectures can be potentially considered for AR Glasses tethering and media delivery. The Figure 4.3.3-1 shows the application architecture for enabling PINAPP (“Personal IoT Network Application”) as described in 3GPP TR 23.700-78 [5].

Figure 4.3.3-1: PINAPP architecture (from [5])

Figure 4.3.3-2 shows the PIN network architecture in 5GS as described in TR 23.700-88[6].

Figure 4.3.3-2: Personal IoT Networks Architecture in 5GS

As shown in Figure 4.3.3-2, the system architecture contains the following reference points:

P1:	Reference point between the PINE and the PEGC. This reference point is based on non-3GPP access (e.g. WIFI, Bluetooth).

P2:	Reference point between the PEMC and the PEGC. This reference point is based on non-3GPP access (e.g. WIFI, Bluetooth) or 5G ProSe Direct Communication.

P3:	Reference point between the PEMC and the PIN Application Server. This reference point can be based on the direct user plane path to 5GS, relay path via the PEGC, or other communication path via Internet.

P4:	Reference point between the PEGC and the PIN Application Server. This reference point is based on the user plane path between PEGC and 5GS.

When considering the device architectures for Tethered Glasses in clause 4.4, and the PIN architecture aspects described in 3GPP TR 23.700-88 [6] there is a potential solution to use PINs for AR glasses. tethering and media delivery, when wireless tethered connectivity between 5G device and AR glasses device is provided through non-3GPP access e.g., WiFi or BLE:

-	The 5G Phone is a 3GPP UE which can act as a PEGC (PIN Element with Gateway Capability) and PEMC (PIN Element with Management Capability)

-	The AR glasses device can act as a PINE (PIN element).

-	The AR glasses device and the 5G device can then be considered a PIN.

## 4.4	Device Architectures for Tethered Glasses

### 4.4.1	General

Based on the guiding use case in clause 4.2.2 as well as the discussions in TR 26.998 [2], this clause identifies the architectures and media handling for different tethered AR glasses.

Looking at existing AR Glasses, based on the study in TR 26.998 [2] and based on information from chipset manufacturers on existing and emerging devices, an AR Glasses designed for AR experiences does integrate complex functionalities and many of those relate to capabilities. Figure 4.4.1-1 is a picture providing an overview of an AR glasses.

Hinge

SoC Media

Connectivity

Eye Tracking + Camera/Sensor Aggregator

Figure 4.4.1-1 - Overview of an AR glasses

Typical functions of such a AR Glasses consists of:

- 	Peripherals including

-	Displays

-	Cameras

-	Microphones

-	Sensors

-	Camera/Sensor Aggregators

-	Perception functionality: Eye Tracking, Face Tracking, etc.

-	SoC Media

-	Display Processing

-	GPU functionalities: Composition/Reprojection

-	Decoding

-	Decryption

-	Camera Front ends

-	Perception functionality: 6DoF, etc.

-	Encoding

-	Connectivity

-	Wi-Fi, Bluetooth, 5G, etc.

An interesting aspect to consider from the above is that the device consists of different thermal islands, hence division in multiple chips in the headset is highly desirable. This means that both minimizing the power consumption per thermal island as well as minimizing the overall power consumption is an essential design constraint for the device battery life. Such devices require to partition workloads to remote devices or the cloud to some extent to balance the power load. Based on this, media capabilities are also possibly required on UE that acts as a hub for a tethered glass. Architectures and processing for this will the main subject of discussion in this Technical Report.

It should be noted that such AR glasses are predominantly served with media that can directly be rendered by the peripherals, or produce media captured on the device and sent to remote processing. Initial System-on-Chip (SoC) media will likely rely on existing hardware, for example from lower end mobile chipsets. Some people consider XR even a hack that uses existing components in a smart manner. However, a core aspect of XR experiences different from traditional mobile devices is the concurrent operation of multiple encoders and/or decoders to address different sensors, eye buffers, layers and so on, as well as the rendering to GPU instead of directly going to the display. Only over time, such hardware will get added specific functionalities, but not in the near and mid-term. Expected in the future are higher render and display resolutions, multi-layer composition, etc.

Figure 4.4.1-2 provides an 5G AR UE as a framework. In this context the XR application is offered several functionalities on the device as well as connectivity options to create an XR experiences as defined in TR 26.998, clause 4.2, namely:

-	XR Runtime: The XR Runtime is a device-resident software or firmware that implements a set of XR APIs to provide access to the underlying AR/MR hardware, including capability discovery, session management, input and sensors, composition, and other XR functions. An example for such APIs is provided by OpenXR.

-	XR Scene Manager: A Scene Manager is a software component that is able to process a scene description and renders the corresponding 3D scene. To render the scene, the Scene Manager typically uses a Graphics Engine that may be accessed by well-specified APIs such as defined by Vulkan, OpenGL, Metal, DirectX, etc. Spatial audio is also handled by the Scene Manager based on a description of the audio scene. Other media types may be added as well.

-	Media Access Functions: supports the application to access and stream media. For this purpose, a Media Access Function includes: media processing, codecs, content delivery protocols, content protection, QoS control, metrics collection and reporting, etc.

-	5G System: supports the XR application to access the network through the 5G system, either directly or through the MAF.

Figure 4.4.1-2 – 5G AR UE Framework

Given that many functionalities are defined through Khronos OpenXR [10], defining capabilities for example by mandating or recommending support of certain APIs or parameter settings on API may be relevant. In some cases it may not even be possible to define capabilities, but for example rely on test signals and benchmarking requirements that estimate the performance of a device.

In the following, two different approaches for tethering AR Glasses are identified, identifying how:

-	Tethered Standalone AR Glasses: In this case, the AR Glasses runs an XR application that uses the capabilities of the Glasses to create a service. The AR Glasses is tethered to a 5G device and potentially uses the capabilities of the phone to support the application. For details refer to clause 4.4.2.

-	Display AR Glasses: In this case, the AR Glasses is tethered to a 5G device that includes the application and the XR functions. The 5G device runs the application that uses the capabilities of the 5G device to run an AR/MR experience. The AR Glasses is connected to the 5G Device, but the XR runtime API is exposed to the 5G device/phone. For details refer to clause 4.4.3.

### 4.4.2	Tethered Standalone AR Glasses

Figure 4.4.2-1 provides the technical architecture of a typical stand-alone AR Glasses device. The AR Glasses runs an XR application that uses the capabilities of the Glasses to create a service. The AR Glasses is tethered to a 5G device and potentially uses the capabilities of the phone to support the application.

Figure 4.4.2-1 – Tethered Standalone AR glass-based device architecture

The XR application is responsible for orchestrating the various device resources to offer the AR experience to the user. In particular, the XR application can leverage three main internal components on the device which are:

-	The Media Access Functions (MAF)

-	The XR Runtime

-	The XR Scene Manager

The XR application can communicate with those three components via dedicated APIs called the MAF-API, the XR Scene Manager API and the XR Runtime API. Among other functionalities, those APIs enables the XR application to discover and query the media capabilities in terms of support as well as available resources at runtime.

The XR runtime features several sensors and user controllers relevant for AR experiences such as cameras, microphones, speakers, display and generic user input. The XR Runtime typically also deals with the composition of primitive buffers that are mapped to the eye buffer display taking into account device characteristics as well as the latest pose information to apply late stage reprojection.

The XR scene manager is typically very lightweight and with no or very limited GPU capabilities. It maps raw media primitive buffers such as texture and depth information

Once the XR application is running, the downlink media is accessed by the MAF in compressed form and then from then MAF to the  XR Scene Manger in primitives. The device may also establish an uplink data flow from the XR Runtime to the MAF wherein the data may be in an uncompressed form and then from the MAF to the remote device, it is typically compressed the data in order to facilitate the expected transmission over the network.

In order to analyse the use cases and tethering architectures in more details in terms of bitrates and processing, the following assumptions may be made on Tethered Standalone AR Glasses:

-	Video Playback and decoding:  H.265 Main 10 Profile with maximum processing: up to 8,294,400 Macroblocks per second (corresponding to 8192x4320 @ 60fps)

-	Video recording and encoding: H.265 Main 10 Profile with maximum processing: up to 3,888,000 Macroblocks per second (corresponding to 3840x2160 @ 120fps), low-latency encoding, error-robustness, slicing, intra refresh, long term prediction.

-	Maximum number of combined encoding and decoding instances: 16 for video, 16 for audio

-	Audio capabilities that allow to encode several  audio inputs with low-latency and to decode multiple audio streams  for binaural playback in parallel.

-	The scene manager is very lightweight and passes through primitive buffers to be consumed by swap chains of the XR run-time. Swapchain images are typically 2D RGB.

Editor’s Note: More detailed assumptions on the rendering capabilities needs to be documented. These assumptions may be aligned with what MeCAR defines

### 4.4.3	Tethered Display AR Glasses

Figure 4.4.3-1 provides the technical architecture of a typical tethered display AR Glasses device. The AR Glasses is tethered to a 5G device that includes the application and the XR functions. The 5G device runs the application that uses the capabilities of the 5G device to run an AR/MR experience. The AR Glasses is connected to the 5G Device, but the XR runtime API is exposed to the 5G device/phone.

Figure 4.4.3-1 – Tethered Display AR glass-based device architecture

In this case, the connection between the phone and Glasses is hidden to the application and tethers the XR Runtime API on the 5G phone to the XR Runtime core functions on the glasses. The overall function is referred to as XR Link.

In order to analyse the use cases and tethering architectures in more details, it is assumed that the media access and rendering functions of a high-end smart phone can be used.

Editor’s Note: More detailed assumptions on the capabilities needs to be documented

### 4.4.4	Tethered AR Glasses with 5G Relay

This architecture corresponds to the “Type 3b: 5G Relay WireLess Tethered AR UE” from TR26.998 [3], but is redrawn in line with the 5G AR UE Framework shown in Figure 4.4.1-2.

A Basic XR application runs on the AR Glasses. It performs functions such as initiating the XR application that triggers a corresponding XR application in the Cloud/Edge, and initiating the setup of the tethering connection.

The XR runtime is split between the AR Glasses and the Cloud/Edge. An XR Runtime API is located in the Cloud/Edge. From the perspective of the XR application on the Cloud/Edge, the XR Runtime including the core functions (which are located on the AR glasses device) appears local.

The Media Access Function on the 5G Device/Phone performs QoS measuring and reporting for the tethering connection.

Figure 4.4.4-1 – Tethered AR glasses with 5G relay

# 5	System Architectures and Call Flows

## 5.1	System Architecture

The basic problem to solve is shown in Figure 5.1-1 below. An Application Server, for example on the edge, uses the 5G System to distribute content (e.g., content generated in response to the video/audio/pose input from the user) to a 5G phone. The 5G phone is connected with a pair of tethered glasses. The question is now how to set up connectivity and media call flows to provide a best user experience under latency and processing constraints on different links.

Figure 5.1-1 The system architecture.

Architectural decisions can be made based on different applications, capabilities of the glasses and UE, link qualities and so on.

## 5.2	Call flows

For the network architecture for the standalone AR glasses-based device architecture as shown in Figure 4.4.2.1 , the call flow is provided in Figure 5.2-1, which is similar to the call flow in subclause 4.3.1 of TR 26.998.

Note that the 5G device serves as a relay, and may optionally be offloaded to perform some "phone based processing functions".

The call flow is described below.

1.	The AR glasses device and the 5G device/phone sets up the tethering link.

2.  The application contacts the application provider to fetch the entry point for the content. The acquisition of the entry point may be performed in different ways and is considered out of scope. An entry point may for example be a URL to a scene description.

3.	Session set up:

3a.	In case when the entry point is a URL of a scene description, the application initializes the Scene Manager using the acquired entry point.

3b.	The Scene Manager retrieves the scene description from the scene provider based on the entry point information.

3c.	The Scene Manager parses the entry point and creates the immersive scene.

3d.	The Scene Manager requests the creation of a local AR/MR session from the XR Runtime.

3e.		The XR Runtime creates a local AR/MR session and performs registration with the local environment.

Then steps 4 and 5 run in parallel:

4:	XR Media Delivery Pipeline: In case when entry point is a scene URL, a delivery session - for accessing scenes (new scenes or scene updates) and related media over the network is established. This can basically use the MAF as well as the scene manager and the corresponding network functions. Details are introduced in Figure 5.3-2.

NOTE:	The realization of XR media delivery pipeline may vary in different architectures.

5:	XR Spatial Compute Pipeline: A pipeline that uses sensor data to provide an understanding of the physical space surrounding the device to determine the device’s position and orientation and placement of AR objects in reference to the real world and uses XR Spatial Description information from the network to support this process.

6:	Steps 4 and 5 run independently, but the results of both pipelines (e.g., media organized in a scene graph and pose of the AR device) are inputs of the XR Scene Manager function. This function handles the common processing of the two asynchronous pipelines to create an XR experience.

Figure 5.2-1: Call flow for the tethering architecture standalone AR glasses-based device architecture.

The XR media delivery pipeline (step 4 of Figure 5.2-1) in Figure 5.2-1 is provided in Figure 5.2-2, which is similar to the media delivery pipeline in subclause 4.3.2 of TR 26.998.

For an XR Media Delivery Pipeline:

1.	The Scene Manager initializes XR scene delivery session.

2.	The MAF establishes XR scene delivery session.

3.	The MAF may receive updates to the scene description from the scene provider

4.	The MAF passes the scene update to the Scene Manager.

5.	The Scene Manager updates the current scene.

6.	The Scene Manager acquires the latest pose information and the user’s actions

7.	The Scene Manager in the device shares that information with the Scene Manager in edge/cloud

The media rendering loop consists of the following steps. Note that steps 8, 9 and 10 are running as 3 parallel loops:

8.	For each new object in the scene:

a.	The Scene Manager triggers the MAF to fetch the related media.

b.	The MAF creates a dedicated media pipeline to process the input.

c.	The MAF establishes a transport session for each component of the media object.

9.	For each transport session:

a.	The media pipeline fetches the media data. It could be static, segmented, or real-time media streams.

b.	The media pipeline processes the media and makes it available in buffers.

10.	For each object to be rendered:

a.	The Scene Manager gets processed media data from the media pipeline buffers

b.	The Scene Manager reconstructs and renders the object

11.	The Scene Manager passes the rendered frame to the XR Runtime for display on the tethered standalone AR glass-based device.

Figure 5.2-2: Media delivery pipeline for call flow in Figure 5.3-1.

For the network architecture corresponding to the display AR glasses-based device architecture without edge rendering, as shown in Figure 4.4.3.1, the call flow is shown in Figure 5.2-3. The call flow is different from the one in Figure 5.3.-1 mainly in that the XR application, Scene Manager and Media Access Function reside on the 5G Device. Additionally, an XR Runtime API is on the 5G Device, the XR Runtime is on the AR Glasses Device, and the interactions between the XR Runtime API and the XR Runtime may be based on open standards, for instance Khronos OpenXR [10], . This simplifies the design from the point of view of the XR application.

The call flow is similar to the one in Figure 5.2-3 and the main difference is on the procedures related to XR Runtime. For example, step 3d: Establish  XR session now points to XR Runtime API, instead of XR Runtime.

Figure 5.2-3: Call flow for the network architecture for tethered display AR glasses-based device without edge rendering.

The XR media delivery pipeline (step 4 of Figure 5.2.-3) in Figure 5.2-3 is provided in Figure 5.2-4.

Figure 5.2-4: Media delivery pipeline for call flow in Figure 5.2-3.

# 6	Identified Key Issues and Potential Solutions

Editor’s Note: Providing recommendations for suitable architectures to meet typical AR requirements such as low power consumption, low latency, high bitrates, security and reliability.

## 6.1	Key Issue #1: How to provide End-to-End QoS for the 5G relay architecture

### 6.1.1	Description of the key issue

To an XR application, the QoS metrics that matter are the end-to-end QoS metrics, including:

-	Delay

-	Packet loss rate

-	Bit rate

However, for the tethered AR glasses with 5G relay architecture, as shown in figure 6.1-1, a typical AR/MR end-to-end path traverses both the 5G network and non-5G networks. The non-5G networks, e.g., Wi-Fi, the Internet, do not provide guaranteed QoS. A key challenge is how to provide end-to-end QoS with a mix of a 5G network and non-5G networks.

### 6.1.2	Potential solution

#### 6.1.2.1	End-to-end latency

Figure 6.1-1 depicts a breakdown of the end-to-end delay and suggests a solution: the 5G network adjusts the delay within the 5G network to compensate for the delays incurred in non-5G networks such that the end-to-end delay meets an end-to-end delay requirement. Note that this solution has its limitation in that it is impossible for the 5G network to compensate for the delay in the non-5G networks if the aggregate delay in the non-5G network exceeds the delay imposed by the end-to-end delay requirement.

Figure 6.1-1: End-to-end delay breakdown

To meet the end-to-end latency requirement for the AR/MR session, some entities must determine the delay on the tethering link Dn,1, and the delay on the Internet (between the UPF and the edge application server) Dn,2.

Additionally, the determined delay components need to be reported to the 5G system which uses the reported delays and the target end-to-end delay to determine the delay to be provisioned within the 5G system.

Then, Figure 6.1-1 suggests that to meet a desired end-to-end latency requirement De2e,  desired, the 5G network provides a delay Dc' as follows:

Dc'=De2e,  desired-Dn

The above description can be extended to support delay provisioning within the 5G network in a statistical sense. For example, by taking into account the variation of the non-5G delays, the 5G network can determine the delay in the 5G network to make the end-to-end delay below a desired delay value 99% of the time.

#### 6.1.2.2	Other End-to-end QoS metrics

For other end-to-end QoS metrics, such as packet error rate and bit rate, further study on the potential solutions is needed.

## 6.2	Key Issue #2: How to determine the non-5G delay for the 5G relay architecture

### 6.2.1	Description of the key issue

In an end-to-end connection that includes a tethering link (e.g., Wi-Fi link), a 5G network and the Internet, the Wi-Fi segment and the Internet segment typically cannot guarantee latency. To achieve low end-to-end latency, one approach is to make the latency in the 5G network very conservative such that the end-to-end latency is below a target value. This, however, comes at a cost, because provisioning an unnecessarily low latency in the 5G network means excessive resource allocation (e.g., to support a more robust modulation-and-coding scheme (MCS)) or pre-empting many other traffic flows.

An alternative approach is to dynamically adjust the delay in the 5G network in accordance with the total delay incurred elsewhere on the end-to-end path. The delay on a Wi-Fi link may change over time depending on the interference generated by other nearby Wi-Fi networks operating on the same frequency. Similarly, the delay between the UPF and the application server depends on the location of this selected UPF and the network congestion level. Therefore, measurements may be used to estimate these time-varying delays on the non-5G segments.

For delay measurement, it is important that the measured delay is representative of the delay to be experienced by the data packets. Delay measurements based on delay measurement messages such as the ping message (ICMP Echo and Echo Reply) may not accurately reflect the delay experienced by the data packets for two reasons: (1) the delay measurement message uses a protocol number (e.g., 1 for ping) that is different from the protocol number for the data packet (e.g., 17 if the data packet is sent with RTP/UDP), resulting in different IP 5-tuples and consequently different QoS treatment in the communication network; (2) the packet size of a delay measurement typically is much smaller than that of a data packet, resulting in different transmission delays which are part of the overall delay.

There are two ways to measure the latency and they fill in the details for step 10 in Figure 5.2-5 in clause 5.2.

### 6.2.2	Solution: Segment-by-segment delay measurement

The delay on Wi-Fi link and the delay between the UPF and the application server are measured separately. One simple solution is to use the ICMP ping protocol (ICMP Echo and Echo Reply, IETF RFC792 [12]). The 5G phone sends a ping request to the AR glasses, which replies with a ping response. The 5G phone then obtains the RTT over the Wi-Fi link. Similarly, the UPF sends a ping request to the application server, which replies with a ping response, and the UPF obtains the RTT between the UPF and the application server. The respective RTTs can then be halved to get estimates of the one-way delays for the two non-5G segments.

In step 4, the MAF reports the one-way delay estimate Dn,1 to the AF.

In step 7, the UPF reports the one-way delay estimate Dn,2 to the SMF, which forwards the estimate to the AF.

NOTE:	How UPF retrieves the RTT between the UPF and the application server and further exposes the latency results to the AF are not supported in SA2 in current release.

In step 9, the AF determines the desired value for the delay in the 5G network needed to compensate for the variation in the delay in the non-5G segments in order to meet the end-to-end latency requirement for the application, and sends a delay request to the PCF.

Figure 6.2.2-1: segment-by-segment delay measurement

### 6.2.3	Solution: End-to-end delay measurement

The delay measurement is carried out in an end-to-end fashion. This avoids the potential rejection of a measurement message that originates from the UPF and reaches the application server. The AR glasses sends a ping request message to the application server, which replies with a ping response. The AR glasses then estimate the UL one-way end-to-end delay De2e by halving the RTT. The 5G network estimates the UL one-way delay within the 5G network Dc, e.g., by recording the time when the ping request arrives at the phone and the time when the ping request reaches the UPF and takes the difference. or using the QoS monitoring mechanism to obtain the UL one-way delay within the 5G network.

Editor’s Note:	How UPF detects and reports the arrival time of the ping test is not supported yet in SA2.

The estimated UL one-way delay on the non-5G segments is then Dn=De2e-Dc as shownin Figure 6.2.3-1.

Figure 6.2.3-1: End-to-end delay measurement

To make the end-to-end delay measurements accurately reflect the end-to-end delay experienced by the data packets, one potential solution is to use in-band measurement as shown in Fig. 6.2.3-2. A timestamp message is piggybacked to an RTP packet that carries data when needed. The payload type field in the RTP packet header is determined by the data, and an RTP header extension is added to instruct the receiver how to separate the timestamp message and the data. This way, the RTP packet is treated as a data packet with packet size substantially the same as an RTP packet that carries data only. Similarly, a timestamp reply message is piggybacked to an RTP packet in the reverse direction. This mechanism is payload-format independent, i.e., it can be applied to all payload formats. The order of the Data and the Timestamp msg (or Timestamp Reply Msg) may be switched.

Fig. 6.2.3-2 In-band end-to-end delay measurement by piggybacking a delay measurement message to an RTP packet.

The timestamp message may be an ICMP Timestamp message (as defined in RFC792 [12]) with a timestamp field of 32 bits, which represents the number of milliseconds with respect to midnight Universal Time (UT). However, the achievable accuracy is limited by the 1ms granularity of the timestamp format. Alternatively, Precision Time Protocol or PTP (IEEE 1588-2019 [13]) may be used for better accuracy due to a finer granularity of 1 nanosecond of its timestamp format.

The target precision for the measurement depends on the overall latency and the use case. TR 26.998 listed 50-60ms for the pose-to-render-to-photon latency. For better user experience, we may need to consider better target precision. .

Instead of using RTP, using RTCP for delay measurement may be a candidate solution (as allowed in RFC3550), but it may have the following drawbacks:

- First, although RTCP and RTP may use the same port number as allowed by RFC 5761 which was motivated to simplify NAT and firewall management, it is not guaranteed that they always use the same port number. When they use different port numbers, RTCP and RTP packets will be mapped to different QoS flows (in the 5G core network) and receive different QoS treatment.

- Second, the payload type for RTCP packetrs (200 for sender report (SR) and 201 for receiver report (RR)) are different from those of RTP packets, and if a network takes into account the payload type in provisioning QoS (e.g., in Wi-Fi an RTP packet carrying video may be mapped to the Video access category, while RTCP packets may be mapped to the Best Effort access category), RTCP packets and RTP packets may receive different QoS.

Third, the overhead of RTCP packets may be greater, because a separate packet, which includes the UDP packet header, the IP packet header and lower layer packet headers, needs to be sent.

TS 23.501 [9] offers two measurement methods for measuring the delay in the 5G system Dc, originally intended for QoS monitoring to assist URLLC service. The first method, termed “Per QoS Flow per UE QoS Monitoring”, leverages the GTP-U headers to carry the timestamps, and the second method, termed “GTP-U Path Monitoring”, leverages the GTP-U Echo protocol. The first method is shown in Figure 6.2.3-3.

Figure 6.2.3-3: Measuring the delay in the 5G system: Per QoS Flow per UE QoS Monitoring in TS23.501  [9]

The PCF generates the QoS monitoring policy based on the request from the AF (directly or via NEF) (step 2).

The SMF initiates a QoS monitoring request to the NG-RAN (step 3) and the PSA UPF (step 4).

Step 6: Time stamp T1 is taken in the PSA UPF, indicating the time when the PSA UPF sends a monitoring packet to the NG-RAN (i.e., gNB).

Step 7: the PSA UPF sends a monitoring packet to the NG-RAN, containing T1, QFI and QoS Monitoring Packet (QMP) indicator in the GTP-U header.

Step 8: Time stamp T2 is taken when the monitoring packet is received by the NG-RAN.

Step 10: Time stamp T3 is taken when the NG-RAN forwards an UL packet, or generate a dummy UL packet, where for either case the NG-RAN puts UL/DL packet delay results of RAN part, T1, T2, T3 and the QMP indicator in the GTP-U header.

Step 12: Time stamp T4 is taken when the UL packet is received.

Step 13: Between the NG-RAN and the PSA UPF, if they are synchronized, then the UL delay will be T4-T3, and the DL delay will be T2-T1. If they are not synchronized, then the procedure computes the average one-way delay (T2-T1 + T4-T3)/2.

Step 14: The delay on the access network (between the UE and the NG-RAN) can be added to the results in step 13 to get the total delays in the 5G system, i.e. UL, DL or RT latency.

Finally, the UPF reports the QoS monitoring results to SMF and SMF further reports to PCF. The PCF then exposes the QoS monitoring results to the AF directly or via NEF as requested and the AF eventually obtains the UL/DL or average delay within the 5G System.

Piggybacking timestamps rather than the timestamp message or timestamp reply message to an RTP data packet may reduce the communication overhead. For example, the Timestamp message or Timestamp Reply message are of the following format (RFC792 [12]) as shown in Figure 6.2.3-4.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|     Type      |      Code     |          Checksum             |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|           Identifier          |        Sequence Number        |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|     Originate Timestamp                                       |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|     Receive Timestamp                                         |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|     Transmit Timestamp                                        |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 6.2.3-4: The packet format of Timestamp Reply message in RFC792[12]

For the Timestamp message, the size is 12 bytes. If only the timestamp portion – the Originate Timestamp – is piggybacked, the size is reduced to 4 bytes. To let the receiver know the type of the information (i.e., how many timestamps are contained), the Type field can be added. As a result, the total size is 5 bytes. The savings is 7 bytes. This may not look much but can be significant if frequent measurements are needed.

Similarly, for the Timestamp Reply message, the size is 20 bytes. If again only the timestamps and the Type information is included in the piggybacked RTP packet, the size is reduced from 20 bytes to 13 bytes.

Assuming the number of Timestamp messages are the same as the number of Timestamp Reply messages, the average saving is 44%.

### 6.2.4	Time measurement protocol

The ICMP ping protocol uses two timestamps generated at the transmitter to get an estimate of the RTT. The measured delay includes the time gap between the reception of the ping request message and the transmission of the ping response message at the receiver. The time gap contributes to the estimation error, and it depends on the operating system used at the receiver and may become significant for low latency applications.

Alternatively, ICMP timestamp approach (IETF RFC792 [12]) can be used, which, compared to ICMP ping, provides the source two timestamps, one for the reception of the timestamp message and the other for the transmission of the timestamp reply message. The two timestamps are carried back to the source, which can use the difference to calculate the time gap and get a more accurate estimate of the RTT.

## 6.3	Key Issue #3: What and how to report the non-5G delay

### 6.3.1	Description of the key issue

In order for the 5G network to provide an appropriate delay over the 5G segment of the end-to-end path to meet a target end-to-end delay requirement, the non-5G delay needs to be reported to the 5G system.

The delay over a non-5G network may vary over time. A key issue is what should be reported to the 5G network, and how.

### 6.3.2	Potential solutions

In the segment-by-segment measurement method (clause 6.2.2), the 5G device (e.g., 5G phone) can report the delay between the AR glasses and the 5G device via  the Direct Data Collection Client as indicated in 3GPPTS26.531 [11], Other 5G functions on the 5G device is not excluded. The the UPF can report the delay between the UPF and the application server to SMF.

In the end-to-end measurement method (clause 6.2.3), the AR glasses measures the end-to-end delay, forwards the measurement to the 5G device. The 5G device reports it through MAF to the 5G core network.

The reporting methodology may depend on the tethering architectures. Figure 6.3-1 shows a possible call flow for delay reporting for the end-to-end measurement approach. The reported statistics of the end-to-end delay may include the mean and standard deviation.

Figure 6.3-2 shows a possible call flow for delay reporting for the partial path measurement approach, where

-	DWM: delay between Wi-Fi and Media Access Function (MAF) on the 5G Device

-	DWW: delay between Wi-Fi on the AR Glasses Device and Wi-Fi on the 5G Device

-	DME: delay between MAF and the Edge/Cloud

The end-to-end delay De2e = DWW+ DWM + DME.

In step 15, the Direct Data Collection Client reports to the Data Collection AF a desired value for the delay within the 5G network Dc.

Figure 6.3-1: reporting statistics of end-to-end delay based on end-to-end measurement

Figure 6.3-2: reporting statistics of delays based on partial path measurement

Note that in Figure 6.3-2, the Media Access Function is mapped to an instance of the UE Application in the reference architecture for data collection and reporting in 3GPP TS26.531 [11], i.e., the reporting is via the R7 reference point. This also applies to step 8, where an application may be used to read the log on Wi-Fi and the application is an instance of the UE Application via the R7 reference point.

## 6.4	Key Issue #4: Formats and Connectivity of Tethered Glass

### 6.4.1	Description

Split Rendering across a proprietary link may have limitations in terms of formats that can be used, as well as on the supported connectivity and associated bitrates. The knowledge of the capabilities of a tethered glass, accessible through the XR Runtime API on the phone, can support the operation of the phone over the 5G Network in terms of required bitrates as well as in terms of preferable formats. This clause discusses the workflow and provides relevant conclusions in terms of capabilities and related signalling.

### 6.4.2	Background

The architecture of relevance for the device is shown in Figure 4.4.1-2. The key issue is the handling of media data in the workflow. In a typical implementation aligned with OpenXR processing, the following applies for visual media:

-	To present images to the user, the runtime provides images organized in swapchains for the application to render into.

-	The XR runtime may support different swapchain image formats and the supported image formats may be provided to the application through the runtime API. XR runtimes is expected to at least support R8G8B8A8 and R8G8B8A8 sRGB formats. Details may depend on the graphics API specified in xrCreateSession. Options include DirectX or OpenGL. For example, support for OpenGL ES as a reference may be assumed, i.e. an extension equivalent to the functionalities provided in XR_KHR_opengl_es_enable. OpenGL ES is platform independent and suited for embedded systems.

-	Swapchain images can be 2D or 2D Array. Arrays allow to extract a subset of the 2D images for rendering

-	The application or scene manager can offload the composition of the final image to a XR runtime-supplied compositor. By this, the rendering complexity is significantly lower since details such as frame-rate interpolation and distortion correction are performed by the XR runtime. It is assumed that the XR Runtime provides these functionalities.

-	A runtime on a XR device typically supports OpenXR composition, namely Projection, Quad, Cube, Cylinder, Equirectangular.

An OpenXR application life cycle is shown in Figure 6.4.2-1. In this case, after creating an OpenXR session, the application starts a frame loop. The frame loop is executed for every frame. The frame loop consists of the following steps:

1)	Synchronize actions: this step consists of retrieving the action state, e.g. the status of the controller buttons and the associated pose. During this step, the application also establishes the location of different trackables. The application may also send haptics feedback.

2)	Start a new frame: this step starts with waiting for a frame to be provided by the XR runtime. This step is necessary to synchronize the application frame submission with the display. The xrWaitFrame function returns a frame state for the requested frame that includes a predictedDisplayTime, which is a prediction of when the corresponding composited frame will be displayed. This information is used by the application to request the predicted pose at display. Once the xrWaitFrame function completes, the application calls xrBeginFrame to signal the start of the rendering process.

3)	Retrieve rendering resources: the application starts by locating the views in space and time by calling the xrLocateViews function, provided with the predicted display time and the XR space. It then acquires the swap chain image associated with every view of the composition layer. It waits for the swap chain image to be made available so it can write into it.

4)	Rendering: the application then performs its rendering work. It iterates over the scene graph nodes and renders each object to the view. This step usually uses a Graphics Framework such Vulkan, OpenGL, or Direct3D to perform the actual graphics operations.

5)	Release resources: once the rendering is done for a view, the application releases the corresponding swap chain image. Once all views are rendered, it sends them for display by calling the xrEndFrame function.

Figure 6.4-2-1: OpenXR application life cycle

For audio media, similar processes as video typically apply. OpenXR and OpenSL ES aligned terminology is used as a reference, A typically possible decomposition of steps for immersive audio rendering is as follows: An interface to the XR runtime is available to hand over raw audio buffers to determine how the XR application and scene manager would access a device’s audio capabilities. In order address a concrete implementation example, the model of OpenSL ES is used as a reference for. OpenSL ES supports both file-based and in-memory data sources, as well as buffer queues, for efficient streaming of audio data from memory to the audio system. Buffer queues in OpenSL may be viewed as equivalent to visual swap chains. OpenSL ES may be viewed as companion to 3D graphic APIs such as OpenGL ES. The 3D graphics engine will render the 3D graphics scene to a two-dimension display device, and the OpenSL ES implementation will render the 3D audio scene to the audio output device. In today’s implementations, in addition to the functionalities from such buffer queues, different types of audio signals may be provided, and additional/alternative processing steps may be carried out. Audio signals (i.e. the combination of metadata and buffer queues) may be

-	non-immersive or also known as non-diegitic, i.e. they are not rendered according to the pose.

-	Immersive and describe a full 6DoF experience in the reference space of the XR session. In this case, the XR runtime will create a rendered signal according to the latest pose.

-	Immersive and pre-rendered for a specific render pose. In this case, the signals have been prepared such that the runtime can use the audio signal and the associated render pose and supplementary data for a pose correction to the latest pose.

-	a mixture of such signals that are jointly presented.

-	the signals may originate from different source, for example some may be generated locally, others may be part of a pre-rendering or a full scene created in the network

The audio data is considered to be uncompressed.

### 6.4.3	Assumptions

Assuming now a device as documented in clause 4.4.3 as a tethered AR glasses, then in a typical deployment, Figure 6.4.3-1 provides a summary of the operation.

-	Swap chain images are provided by phone-based XR Scene manager to OpenXR

-	Actions and pose information are provided to the application

-	A proprietary system is operated south of the OpenXR API for split rendering, i.e. depicted left of the OpenXR API. This is very typical in implementations today for which the phone and the glass are provided by the same vendor.

-	The OpenXR API supports a set of typical formats, that are raw formats according to the discussion in clause 6.4.2.

Figure 6.4.3-1: Tethering based on proprietary split rendering operation

In an extension to Figure 6.4.3-1, Figure 6.4.3-2 provides more details of the typical functions carried out in the split rendering, namely converting the raw OpenXR formats, encoding and providing a security framework, setting up a connection together with a protocol and match the bitrate of the link. On the receiving end, the signals sent over the tethered link are decrypted, decoded and the provided to the XR runtime on the glass for final pose correction. Similar processing happens on the uplink to provide encoded pose and actions.

Figure 6.4.3-2: Split rendering operations on proprietary link

### 6.4.4	Problem Statements

#### 6.4.4.1		General

Now a couple of scenarios exist, in case the media is not generated on device, but received over the 5G System. In the regular operation, the formats on the two links, i.e. the 5G system and the proprietary link, would operate completely independently.

#### 6.4.4.2		Case 1: Restricted Raw formats

However, this misses the following aspects when the raw formats are decoded and provided:

- The formats on the AR Glass may be restricted. Hence, only a reduced set of media data, for example in terms of resolution, may be usefully provided to the AR glass

- The bitrate on the tethered link may be restricted, or may even dynamically change. In this case, the quality of the formats provided at the input may not be maintained end-to-end

- The security framework between the glass and the phone is such that it is unusable for certain formats.

Examples for such cases may be in Media Streaming, for which only formats and signals are downloaded that can be processed by the raw formats supported over the tethered link. For conversational applications, also such restrictions may apply, for example for audio only stereo signals are supported.

#### 6.4.4.3		Case 2: Transcoding Problem

Even more severe, if the content is provided from the 5G System in a compressed from, then the operation in the phone results in a typical transcoding operation with the following drawbacks

- Addition of latency because of algorithmic and processing delays

- Additional distortion may be added in the transcoding process

- Unnecessary power consumptions for the encoding and decoding processes

- A trusted security point may be interrupted, put in clear and re-encoded

- The bitrate on the tethered link may be restricted, or may even dynamically change. In this case, the quality of the formats provided at the input may not be maintained end-to-end

- The security framework between the glass and the phone is such that it is unusable for certain formats.

Examples for such cases may be in Media Streaming, but likely to a limited attempt. For conversational applications, also such restrictions may apply, for example for audio only stereo signals are supported. More prominent is the case in conversational speech cases and most prominent is the case in split rendering context, i.e. if the buffer view is pre-rendered on the edge or the cloud. In this case, transcoding is most impactful.

### 6.4.5	Potential Solutions

#### 6.4.5.1	Layer-2 or Layer-3 Relay

In order to avoid format or transcoding conversion on the UE in case 1 from above, a simple relay can be done according to the architecture in clause 4.4.4. However, this has several downsides:

- The phone is excluded in the rendering and the application. No locally generated data can be added to the rendering. The application needs to reside on the edge, and the UE may upload data for rendering.

- If the connection to the cloud/edge is not available or interrupted, no service can be provided.

- The latency that is possibly incurred by this operation may be to high

#### 6.4.5.2	Raw Format adaptation

Raw format adaptation primarily addressed case 2 from 6.4.4. In this case, the application on the UE not only queries the supported raw formats, but it also gets information from the XR Runtime on information related to the resolution of the formats, the quality of the link in a static and dynamic fashion and other information that the phone can use in the communication between across the 5G System as well as in the rendering in the device to properly match the formats received and generated on the phone to match those of the one supported on the proprietary link.

The key extensions are as follows:

- Support in the runtime query for supported formats additional information that include

-	the resolution of the format

-	the quality degradation incurred by the combination of the link and the coding

-	the security framework and capabilities of the device

-	dynamically providing the metrics and information of the signal quality on the proprietary link

- Support of usage of this provided static and dynamic information for example in

-	the Media streaming client to select the appropriate content codecs, bitrates, possibly in a dynamic fashion

-	a communication client to negotiate with the network to support the appropriate content formats, bitrates and qualities, possibly in a dynamic fashion

- Announcing appropriate information on the 5G System network side to the 5G Phone in order to be able to make such selections.

The details of these extensions need to be added to for example a streaming manifest, the session description protocol or to a scene description.

#### 6.4.5.3	Pass-through Compressed Media Formats for XR Runtimes

This case primarily addressed the case 2 from 6.4.4, namely the issue of transcoding. In order to address this case, it is considered according to Figure 6.4.5.3-1, the media format is passed through from the 5G network over the tethering link, using the OpenXR or the general run-time API. This allows to also add local data as an example that can be added on different layers. Pass-through may include pasing only the compressed media data, but may also include the security frame work or the entire protocol. This depends on the capability of the endpoint on the glass.

Figure 6.4.5.3-1: Pass-through compressed media format.

The key extensions are as follows:

- Support in the runtime query for supported formats additional information that include

-	the resolution of the format

-	the quality degradation incurred by the combination of the link and the coding

-	the security framework and capabilities of the device

-	dynamically providing the metrics and information of the signal quality on the proprietary link

-	the audio and video decoding capabilities of the glass

-	the security capabilities of the glass

-	the security framework and capabilities of the device

-	statically and dynamically the bitrate and delay of the link

- Support of usage of this provided static and dynamic information for example in

-	the Media streaming client to select the appropriate content formats, bitrates, codecs and qualities, possibly in a dynamic fashion

-	a communication client to negotiate with the network to support the appropriate content formats, codecs bitrates and qualities, possibly in a dynamic fashion

-	in split rendering for which the formats are provided accordingly as shown in Figure 6.4.5.3-2

- Announcing appropriate information on the 5G System network side to the 5G Phone in order to be able to make such selections.

The details of these extensions need to be added to for example a streaming manifest, the session description protocol or to a scene description.

Figure 6.4.5.3-2: Pass-through compressed media format.

A simplified approach is provided in Figure 6.4.5.3-3, for which compressed formats ate exchanged via the XR runtime API.

Figure 6.4.5.3-3: Simplified architecture: Compressed formats are exchanged via XR Runtime API

In order to address these extensions in the  Swapchain Image Management of OpenXR are provided here https://registry.khronos.org/OpenXR/specs/1.0/html/xrspec.html#swapchain-image-management and summarized in clause 6.4.2, the following extensions are considered:

- In a regular operation, the xrEnumerateSwapchainFormats functional call enumerates the texture formats supported by the current session. The type of formats returned are dependent on the graphics API specified in xrCreateSession.

- As an example, Vulkan permits compressed image formats https://registry.khronos.org/vulkan/specs/1.3-khr-extensions/html/chap45.html relying on compressed data formats from Khronos: https://registry.khronos.org/DataFormat/specs/1.3/dataformat.1.3.html

- However, none of the APIs refer to using any video compression formats for each of the swap chain images.

- Hence, inr order to support the above operation of pass-through compressed formats, OpenXR or any runtime is extended to allow a format that adds compressed video formats as swap chain images for which

  - Time stamp is target display time (for example RTP time stamp)

  - the compressed format includes render pose

  - compressed texture format is handed over as part of the swap chain management

Note that this swap chain image management with compressed data only applies for parts of the submitted swap chain buffers, one layer may be sent over in compressed from, whereas a locally generated layer may be sent over in raw form. The synchronization needs to be done by the runtime.

For audio similar principles apply, for which a compressed bitstream is handed to the glass.

For the uplink media data, action and pose data may be passed on in compressed form as received in the glass to the network. The XR Runtime API may even provide such data in compressed (for sending to network) and in raw form (for processing on the phone)

### 6.4.6	Conclusions

In order to support tethered links and glass-based endpoints properly, it is beneficial to provide a framework that allows to adapt to the end point capabilities to maximize end-to-end quality in terms of signal quality, latency, power consumption, etc. In order to support this, extensions considered above include:

- Support in the runtime query for supported formats additional information that include

-	the audio and video decoding capabilities of the glass

-	the security capabilities of the glass

-	the security framework and capabilities of the device

-	statically and dynamically the bitrate and delay of the link

- Support of usage of this provided static and dynamic information for example in

-	the Media streaming client to select the appropriate content formats, bitrates and qualities, possibly in a dynamic fashion

-	a communication client to negotiate with the network to support the appropriate content formats, bitrates and qualities, possibly in a dynamic fashion

-	in split rendering which the formats are provided from the cloud/edge rendering

- Announcing appropriate information on the 5G System network side to the 5G Phone in order to be able to make such selections.

These extensions are preferably addressed in extensions OpenXR, 5G Media Streaming, 5G Real-time communication and the related stage-3 protocols such as DASH, ISO BMFF, SDP, RTP/RTCP, etc.

## 6.5	Key Issue #5: Compute distribution across UE and network for tethered glasses

### 6.5.1	Introduction

In the tethered display AR Glasses context, the compute functions are distributed across the AR Glasses, as well as possibly the UE (phone) and the network. Even within the network the compute may be done in an edge or in the cloud.  The decision how to distribute the compute across different network entities highly depends on the use case and application, the available capabilities in different network entities, the available network connections, economical reasons and possibly many other constraints. Generally, the situation may even change over time, for example due to changes in the application, varying network connections or load re-distribution. It is not expected that a specification will solve the distribution of the compute resources. However, what is essential is that the decision making entity has as much static and dynamic information in order to make informed decisions.

This clause provides some background on different distribution scenarios. The main focus is the derivation of relevant static and dynamic status and capability information to establish proper work flows.

### 6.5.2	Background

Based on TS 26.119, once a session is running and in focussed state aa rendering loop is executed following Figure 6.5.2:

a)	The XR Application retrieves the action state, e.g. the status of the controllers and their associated pose. The application also establishes the location of different trackables.

b)	Before an application can begin writing to a swapchain image, it first waits on the image to avoid writing to it before the Compositor has finished reading from it. Then an XR application synchronizes its rendering loop to the runtime. In the common case that an XR application has pipelined frame submissions, the application is expected to compute the appropriate target display time using both the predicted display time and predicted display interval. An XR Runtime is expected to provide and operate a swapchain that supports a specific frame rate.

c) 	Once the wait time completes, the application initiates the rendering process. In order to support the application in rendering different views the XR Runtime provides access to the viewer pose and projection parameters that are needed to render the different views. The view and projection info is provided for a particular display time within a specified XR space. Typically, the target/predicted display time for a given frame.

d)	the application then performs its rendering work. Rendering work may be very simple, for example just directly copying data from the application into the swap chain or may be complex, for example iterating over the scene graph nodes and rendering complex objects. Once all views/layers are rendered, the application sends them to the XR Runtime for final compositing including the expected display time as well as the associated render pose.

e) 	An XR Runtime typically supports (i) planar projected images rendered from the eye point of each eye using a perspective projection, typically used to render the virtual world from the user’s perspective, and (ii) quad layer type describing a posable planar rectangle in the virtual world for displaying two-dimensional content. Other projection types such as cubemaps, equirectangular or cylindric projection may also be supported.

f)	The XR application offloads the composition of the final image to an XR Runtime-supplied compositor. By this, the rendering complexity is significantly lower since details such as frame-rate interpolation and distortion correction are performed by the XR Runtime. It is assumed that the XR Runtime provides a compositor functionality for device mapping. A Compositor in the runtime is responsible for taking all the received layers, performing any necessary corrections such as pose correction and lens distortion, compositing them, and then sending the final frame to the display. An application may use multiple composition layers for its rendering. Composition layers are drawn in a specified order, with the 0th layer drawn first. Layers are drawn with a "painter’s algorithm," with each successive layer potentially overwriting the destination layers whether or not the new layers are virtually closer to the viewer. Composition layers are subject to blending with other layers. Blending of layers can be controlled by layer per-texel source alpha. Layer swapchain textures may contain an alpha channel. Composition and blending is done in RGBA.

g)	After the compositor has blended and flattened all layers, it then presents this image to the system’s display. The composited image is then blend with the user’s view of the physical world behind the displays in one of three modes, based on the application’s chosen environment blend mode:

h)	Meanwhile, while the XR Runtime uses the submitted frame for compositing and display, a new rendering process may be kicked off for a different swap chain image.

Figure 6.5.2-1 Rendering loop for visual data

Once this loop is running, the rendering can statically or dynamically be adjusted as long as operation northbound of the rendering loop is consistent.

### 6.5.3	Assumptions

### 6.5.4	Problem Statement

### 6.5.5	Potential Solutions

### 6.5.6	Conclusions

## 6.6	Key Issue #6: Usage of PIN for Tethered Glasses

### 6.6.1	Description

In order to use PIN (Personal IoT Network) for AR Glasses tethering, a functional mapping between PIN architecture [6] and the device architecture for tethered AR Glasses in clause 4.4 of this document is needed.

AR Glasses may support different connectivity solutions, e.g., it can be a 3GPP UE, or a Wi-Fi device. A key issue is to clarify the usage of PIN for AR Glasses tethering for different scenarios.

### 6.6.2	Potential solutions

When AR Glasses considered as a 3GPP UE, and typically in the Tethered Standalone AR glass-based device architecture in clause 4.4.2 and assumed that a 3GPP UE can act as PEGC and/or PEMC [6], AR Glasses can act as PEGC and/or PEMC. It means that AR glasses can directly use the PC5 interface for tethering. In this situation, it is not necessary to use PIN network for AR glasses tethering.

When AR Glasses considered as a non-3GPP device, wireless tethered connectivity is provided through non-3GPP access e.g., Wi-Fi or BLE, With the consideration of the Tethered Display AR glass-based device architecture in clause 4.4.3 and Tethered AR glasses with 5G relay in clause 4.4.4, a solution for functional mapping can be found as follows, shown in Figure 6.6.2-1:

-	The 5G Device/Phone is a 3GPP UE which can act as a PEGC and PEMC

-	The Tethered AR Glasses can act as a PINE

-	The XR application server can be considered as a PIN Application server

Figure 6.6.2-1 – Using PIN for Tethered AR glasses

Accordingly, reference point P1 between the PINE and the PEGC, which is based on non-3GPP access, can be used for AR Glasses tethering connection.

Reference point P2 between the PEMC and the PEGC is based on non-3GPP access (e.g. Wi-Fi or BLE) or 5G ProSe Direct Communication. Reference point P3 between the PEMC and the PIN Application Server is based on the direct user plane path to 5GS, relay path via the PEGC, or other communication path via Internet. PEMC can be used for PIN management (create/modify/delete/activate/deactivate a PIN) and add/remove of PINE and PEGC via P2 and P3, to manage the tethering connection of AR Glasses.

Reference point P4 between the PEGC and the PIN Application Server, which is based on the user plane path between PEGC and 5GS, can be used for supporting the relay path between PINE and 5GS for the Tethered AR glasses with 5G relay architecture in clause 4.4.4.

# 7	Potential Next Steps

Editor’s Note: Identify concrete work topics that need to be addressed based on the analysis

# 8	Conclusions and Recommendations

Editor’s Note: Provide conclusions on how to address the potential next steps in clause 8

Annex A (informative):
QoS Control of Relay WLAR UE when 5G Sidelink Used for Tethering Link

For Relay WLAR UE, the 5G sidelink communication is used for the tethering link. The tethering 5G Phone providing the IP network connectivity can be seemed as a 5G ProSe Layer-3 UE-to-Network Relay and the tethered AR Glasses can be seemed as a Remote UE as specified in TS 23.304 [7]. The QoS requirements for the end-to-end AR session can be satisfied by the corresponding QoS control for the tethering link between AR Glasses and 5G Relay UE (i.e. PC5 QoS control), and the QoS control through the 5G system (i.e. the PDU Session between UE and UPF). The tethering link QoS and the 5G System QoS are separately controlled with corresponding QoS rules and QoS parameters (e.g. 5QI, PQI) as specified in TS 23.287 [8] and TS 23.501 [9].

As shown in figure A-1 below, the end-to-end QoS can be met only when the QoS requirements are properly translated and satisfied over the two legs respectively.

Figure A-1: End-to-End QoS translation for 5G Layer-3 Relay operation

To achieve this, the QoS mapping can be pre-configured or provided to the 5G Relay UE from the 5GC. The QoS mapping includes combinations of the 5QIs for the 5G link and the PQIs for the tethering link as entries. Both 5QIs and PQIs have standardized values as specified in TS 23.501 [9] and TS 23.287 [8].

If the QoS setup of 5G system link is initiated by network, the 5G Core Network can generates the QoS parameters (e.g. 5QI) and signal to the 5G Relay UE. Then the 5G Relay UE determines the tethering link QoS parameters based the pre-retrieved QoS mapping and then setup the tethering link between AR glasses and the 5G Relay UE.

If the AR Glasses initiates QoS setup or modification for the tethering link, it provides the QoS Info to the 5G Relay UE. The QoS Info (i.e. PQI, etc.) are interpreted as the end-to-end QoS requirements by the 5G Relay UE for the traffic transmission through the 5G system. The 5G Relay UE would check if the end-to-end QoS requirements can be supported, and decide the 5GS QoS and tethering link QoS parameters based on the QoS mapping.

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-03 | SA4#118 | S4-220361 |  |  |  | Initial version | 0.0.1 |
| 2022-04 | SA4#118 | S4-220563 |  |  |  | Version agreed during SA4#118e | 0.1.0 |
| 2022-05 | SA4#119 | S4-220841 |  |  |  | Version agreed during SA4#119e | 0.2.0 |
| 2022-08 | SA4#120 | S4-221127 |  |  |  | Version agreed during SA4#120 | 0.3.0 |
| 2022-11 | SA4#121 | S4221488 |  |  |  | Version agreed during SA4#121 | 0.4.0 |
| 2022-12 |  |  |  |  |  | Version created by MCC for presentation to TSG | 1.0.0 |
| 2023-02 | SA4#122 | S4-230362 |  |  |  | Version agreed during SA4#122 | 1.1.0 |
| 2023-04 | SA4#123 | S4-230734 |  |  |  | Version agreed during SA4#123e | 1.2.0 |