3GPP TSG-SA WG4 Meeting #124	S4-230855

Berlin, Germany, 22 – 26 May 2023

Source:	Apple

Title:	Discussion on new Feasibility Study on new HEVC profiles and operating points

Agenda item:	9.10

Document for:	DISCUSSION

# Overview

The results of the SA4 Study on 5G Video Codec Characteristics (FS_5Gvideo), which was documented in TR 26.955, showed that the HEVC coding standard demonstrated satisfactory performance to fulfil existing video service needs. It also recommended that "3GPP consider upgrading specifications to support profiles, levels, and possibly features available in HEVC. Features may include better support for screen content and computer-generated content, XR/AR type of services, as well as low and very low latency services." There hasn't been work done in SA4 after this study to include such new features, while significant advancements have been made in services and applications that would benefit from them. Some of these are outlined in the following. This study aims to define these emerging applications for video coding, gather evidence that specific new tools can provide advantage for specific services and applications, and conclude on these aspects.

## 1.1 Multiview/Stereoscopic 3D content

In the past two years there is a renewed interest in the distribution, including streaming, of 3D movie content, as evident by media coverage of recent 3D movie releases. Support for the multiview profile of HEVC mostly involves SW level modifications since this is a high-level coding tool. The objective performance results of MV-HEVC video coding vs. MVC (AVC-based multi-view) are documented in [1], while subjective results also including Simulcast HEVC (each view is coded independently) are documented in [2].

## 1.2 4:4:4

There is considerable interest in the distribution of 4:4:4 video, especially for applications such as screen sharing, gaming, and even for new immersive applications that utilize the new V3C family of standards. There is already considerable device support for HEVC 4:4:4 capable profiles. The performance evaluation results are documented in [3].

## 1.3 Scalable Video

The use of scalability could further enhance the behavior of adaptive streaming systems but also provide additional benefits to end user devices, such as power adaptation. Support for the scalable profile of HEVC mostly involves SW level modifications since this is a high-level coding tool. The performance assessment for SHVC is documented in [4], [5].

# Proposal

It is proposed to conduct a study on this matter and feedback from the group is sought.

# References

[1] ISO/IEC JTC1/SC29/WG11 MPEG2011 M22746, "AVC/MVC anchor coding for MFC", November 2011, Geneva, Switzerland.

[2] ISO/IEC JTC1/SC29/WG11 N16050, "MV-HEVC Verification Test Report", San Diego, US, Feb. 2016.

[3] Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCTVC-U0184, "Interlaced video and format range extensions verification test results," 21st Meeting: Warsaw, PL, 19–26 Jun. 2015.

[4] ISO/IEC JTC1/SC29/WG11 N16051, "SHVC verification test report", February 2016, San Diego, USA.

[5] ISO/IEC JTC1/SC29/WG11 N16268, "Supplemental SHVC verification test report", June 2016, Geneva, CH.