Source: 	China Unicom, Huawei

Title: 	Discussion on the device-related QoE metric

Document for	Discussion and Agreement

Agenda item: 	9.8- FS_ARMRQoE (Feasibility Study on AR and MR QoE Metrics)

Introduction

AR/MR device can provide immersive experience by combining the virtual elements and objects with physical world. This contribution takes MR device “Hololens 2” published by Microsoft as an example, and discusses and analyses device information relevant to user experience of AR/MR service.

AR/MR device-related QoE metrics

Microsoft has published MR product “Hololens 2” as the below figure.

Figure 2-1: Microsoft MR product “Hololens 2” [2]

Its device property information are listed as below table 2-1 [2]:

Table 2-1 Device property information of “Hololens 2”

| Category | Property | Specifications |
| --- | --- | --- |
| Display | Optics | See-through holographic lenses (waveguides) |
|  | Resolution | 2k 3:2 light engines |
|  | Holographic density | >2.5k radiants (light points per radian) |
|  | Eye-based rendering | Display optimization for 3D eye position |
| Sensors | Head tracking | 4 visible light camera |
|  | Eye tracking | 2 IR cameras |
|  | Depth | 1-MP time-of-flight (ToF) depth sensor |
|  | IMU | Accelerometer, gyroscope, magnetometer |
|  | Camera | 8-MP stills, 1080p30 video |
| Human understanding | Hand tracking | Two-handed fully articulated model, direct manipulation |
|  | Eye tracking | Real-time tracking |
|  | Voice | Command and control on-device; natural language with internet connectivity |
|  | Windows Hello | Enterprise-grade security with iris recognition |
| Environment understanding | 6DoF tracking | World-scale positional tracking |
|  | Spatial Mapping | Real-time environment mesh |
|  | Mixed Reality Capture | Mixed hologram and physical environment photos and video |

The above parameters are not needed to be observed or measured since they are inherent device features. Some property information have impacts on user experience, e.g. resolution, hand tracking, eye tracking, spatial mapping…, which are the part of AR/MR device-related QoE metrics.

Detailed QoE metrics

The QoE metrics relevant with AR/MR device as listed in Table 2.1-1 is necessary for assessment of device impact on user experience.

Table 2.1-1: QoE metrics relevant with AR/MR device

| Key |  |  |  | Type | Description |
| --- | --- | --- | --- | --- | --- |
| DeviceInformationList |  |  |  | List | A list of device information objects. |
|  | Entry |  |  | Object | A single object containing new device information. |
|  |  | resolution |  | Object | Display resolution for each eye |
|  |  |  | videowidth | Integer | Number of pixels in display width |
|  |  |  | videoheight | Integer | Number of pixels in display height |
|  |  | eyetrackingCapability |  | Set | Information of end device eye tracking capability. |
|  |  | handtrackingCapability |  | Set | Information of end device hand tracking capability. |
|  |  | spatialmappingCapability |  | Set | Information of end device spatial mapping capability. |
|  |  | … |  |  |  |

Additional AR/MR device-related QoE metrics are FFS.

Proposal

We propose to agree with AR/MR device-related QoE metrics and capture it into the TR 26.812.

References

- S4-230699 TR 26812_040

- https://www.microsoft.com/zh-cn/hololens/hardware