3GPP TSG-WG SA4 Meeting #124	S4-230871

Berlin, Germany, May 22 – 26, 2023	(revision of S4-230xxx)

Source:	Huawei, HiSilicon

Title:	Candidate solution for network slice migration when multiple slices provisioned

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_MS_NS_Ph2 / Rel-18

# 1. Introduction

In last SA4#123e meeting, the candidate solution for provisioning of multiple network slices is introduced and agreed in the draft TR 26.941. However, once the provisioning is done, how to trigger the network slice change by the media session handler is still unclear. This paper proposes to add further details on that.

# 2. Text Proposal

# * * * First change * * * *(all new)

#### 6.1.2.2 Candidate solution #2: Network Slice change based on the provisioned multiple network slices

Pre-requisites and assumptions:

-	The one or more Network Slices are already provisioned and activated. Appropriate Slice and DNN identifiers are known to the 5GMS Application Provider.

-	The policy template with multiple pairs of Network Slice and DNN has been provisioned to the 5GMS AF and the Media Session Handler already obtains the Service Access Information from the 5GMS AF.

Then when the service requirements change, i.e., the gaming user may choose to pay for a higher performance with the ASP and then the Media Session Handler need to notify the 5GMS AF regarding this network slice change notification:

- The Media Session Handler understands the application needs to switch to the target Network Slice.

- The MSH invokes the dynamic policy API exposed by the 5GMS AF with the target network slice and/or DNN inside, indicating that the current 5GM service needs to be migrated to the target network slice and/or DNN.

- When receiving the dynamic policy API with the target network slice and/or DNN, the 5GMS AF can know that the previous network slice/DNN is to be changed to the target pair. Then the 5GMS AF invokes the Nnef_AFSessionWithRequiredQoS service operation to provide guidance for URSP determination to 5G system via NEF as described in clause 4.15.6.10 of TS 23.502 [15].

- When receiving the updated URSP rule, the UE re-runs the URSP evaluation and decides to re-establish a new PDU Session or reusing another existing PDU Session with the target Slice/DNN to carry to the 5GMS service.

# * * * * End of changes * * * *