SA WG4 Meeting #124	S4-230885

May 22-26, 2023, Berlin, Germany

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on reusing existing IMS Data Channel connections

Document for:	Discussion & Agreement

Agenda Item:	10.11

Work Item / Release:	TEI18 / Rel-18

# 1.	 Discussion

Currently it’s up to UE implementation to decide when to establish a bootstrap data channel in the context of a MMTel session. This might lead to the situation that some UEs setup bootstrap data channel for every MMTel session.

In principle, existing IMS data channel connections can be re-used. For example, one bootstrap data channel connection can easily be re-used to download the menu list and several DC applications. Even the application data channel connection might be re-used by different applications (e.g., in case they have the same QoS requirements and terminate in the DC server). Re-using existing data channel connections is more efficient and reduces implementation complexity and need for additional resource on both UE and network side.

The proposed solution for re-using data channels is to add a "a=connection:" attribute in the SDP offer when a bootstrap or application data channel is going to be established. This attribute indicates whether to create a new data channel connection or re-use an existing one. It is up to UE implementation when to set the "a=connection:" attribute in the SDP offer (e.g., based on UE configuration).

- If "a=connection:new" attribute is present in SDP offer or the attribute is not present in SDP offer, then a new DC connection shall be established.

- If "a=connection:existing" attribute is present in SDP offer, then an existing DC connection in this media component shall be re-used.

Originally RFC 4145 (meanwhile updated by RFC 8122) on connection oriented media introduced "a=connection:" attribute in SDP for TCP-based media transport. The RFC8124 introduced this attribute for WebSocket connections.

SDP example using the "a=connection: " attribute:

SDP offer

m=application 52718 UDP/DTLS/SCTP webrtc-datachannel b=AS:500 a=max-message-size:1024 a=sctp-port:5000 a=setup:passive a=fingerprint:SHA-1 4A:AD:B9:B1:3F:82:18:3B:54:02:12:DF:3E:5D:49:6B:19:E5:7C:AB a=tls-id: abc3de65cddef001be82 a=dcmap:10 subprotocol="http" a=3gpp-qos-hint:loss=0.01;latency=100 a=connection:new

SDP offer

m=application 52718 UDP/DTLS/SCTP webrtc-datachannel b=AS:500 a=max-message-size:1024 a=sctp-port:5000 a=setup:passive a=fingerprint:SHA-1 4A:AD:B9:B1:3F:82:18:3B:54:02:12:DF:3E:5D:49:6B:19:E5:7C:AB a=tls-id: abc3de65cddef001be82 a=dcmap:10 subprotocol="http" a=3gpp-qos-hint:loss=0.01;latency=100 a=connection:existing

Note: In principle other parameters, attributes or indications in SDP or SIP can be used to indicate to the peer side that an existing connection should be re-used or a new connection is required.

When an existing data channel connection associated with a MMTel session is to be re-used, the UE and network don’t require to allocate new media resources for this connection.

# 2. Proposal

It is proposed to adopt the solution as described above. An accompanying CR to TS 26.114 is provided in S4-230884.