3GPP TSG-S4 Meeting #124	S4-230892

# Berlin, Germany, 22. – 26. May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.565 | CR | <CR#> | rev | <Rev#> | Current version: | <Version#> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [SR_MSE] Correction of the architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SR_MSE |  |  |  |  |  | Date: |  |  | 16.5.2023 |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The SR_MSE architecture contains a couple of inconsistencies wrt referencing other specifications (like 26.506) and usage of terminology (e.g. MSH) |  |  |  |  |  |  |  |  |
| Summary of change: |  | The architecture is corrected and a non overlapping terminology is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

# 5	Reference Architecture and Procedures

Editor’s Note: The terminalogy and reference-point naming convention needs to be aligned in  this document.

## 5.1	Reference Architecture

### 5.1.1	Introduction

In this clause, different variants of the reference architecture for the split rendering MSE are defined, each representing a different perspective and level of details.

The following functions are introduced:

- Split-Rendering Client (SRC): This function is responsible to acquire the UE media capabilities and negotiates with the RTC AS to agree on the split-rendering process at the RTC AS.

- Split-Rendering Server (SRS): This function is responsible for negotiation of SR session with SRC, monitoring the server’s edge resource usage, and managing/running the split rendering process.

- Application Function (AF): responsible for provisioning, QoS allocation, and edge resource discovery.

- Application Service Provider: The application provider that offers the service.

- Application: The application running on UE

- Media Session Handler (MSH): is the entity on UE that is responsible for the control plane communication with the AF.

Editor’s Note: All interfaces are currently defined as SR-interfaces. However, if no SR-specific functionality is identified for an interface, the 5G-RTC interface reference will be used instead.

### 5.1.2 	Client Architecture

Figure 5.1.2-1 depicts the reference client architecture.

Figure 5.1.2-1 – Device architecture of AR UE

The split rendering client consists of the following components:

- The Media Access Functions: allow for fetching and processing of the pre-rendered media in preparation of final display. The MAF is also responsible for the carriage of any metadata or local media to the split rendering server.

- The scene manager and thin Presentation Engine: is responsible for the negotiation of the split rendering session and the parsing of the description of the rendered media as provided by the SR server. It is also responsible for setting up and managing the XR session with the XR runtime.

- The XR source management is responsible for gathering timed metadata such as pose and action information and sending it to the SR server.

### 5.1.3	End-to-End Architecture

Figure 5.1-3 – Split management architecture

As shown in Figure 5.1.3:

1. The Split Rendering Application Providers (AP) provisions split-rendering through SR-1.

2. In the use cases in which the SR AP is involved in the media delivery, the SR-2 interface is used for this purpose.

3.The communication between SR RTC AF and SR RTC AS is through SR-3.  This interface is out of the scope of this document.

4. The SR related signaling as well as the media delivery between SRC and SR RTC AS is though SR-4.

6. The SRC discovers the client media capabilities through the SR-7 interface. This interface is out of the scope of this document.

7. The SR aware Application and SR AP interact through SR-8. This interface is out of the scope of this document.

### 5.1.5	User Plane Architecture

Figure 5.1.5-1 depicts the user plane architecture for split rendering.

The SR interfaces are considered to be specializations of their parent Trusted RTC reference points as defined in TS26.506.

In the context of split rendering, the SR-4 interface is further classified as SR-4s and SR-4m sub-interfaces. The SR-4s interface covers all user-plane SR related signaling, including ICE signaling. The SR-4m serves for media and metadata exchange between the split rendering client and the split rendering server.

## 5.2	Procedures and Call Flows

### 5.2.1 	Call flow for Split Rendering instance discovery

Generic Procedures

#### 5.2.1.1	Call flow for edge server and split rendering session setup

Figure 5.2.1.1-1 demonstrates a general call flow for split-rendering.

Figure 5.2.1-1: High-level call flow for split-rendering

Steps:

- In this optional step, the Application Provider requests and sets up the edge server(s) used for the split-rendering as described in TS 26.501 clauses 8.1 or 8.2. The Application provider may use any other method to allocation edge servers, or leave it to the MNO to set up appropriate edge servers to run the split-rendering process.

- The Application Provider provisions the split-rendering session using SR-1 and SR-3, as defined in call flows of clauses 5.2.1.1 or 5.2.1.2. If the edge servers were provisioned in step 1, the edge servers ids are provided in this session to employ them for split-rendering.

NOTE: In the case of the client-driven edge management (TS 26.501 8.1), only the client-driven split-rendering (5.2.1.1) is applicable.

- The split-rendering session is set up according to clause 5.2.2.

#### 5.2.1.2 Client-driven procedures and call flows

Figure 5.2.1.2-1 demonstrates a call flow for setting up the split rendering by the client.

Figure 5.2.1.2-1: High-level call flow for initiating a split

Steps:

- The Application Service Provider requests the SRF the provisioning a split management session.

- The split management session is announced to the Application as part of the Service Access Information.

- The Application requests a split of the client media functions from the SRC.

- The SRC inquires the Media Session Handler about the client’s media capabilities.

- The SRC and SRS negotiate on the acceptable capabilities for the device and agree on the split option.

- The SRC starts the split rendering process.

- The SRS acknowledges the SRC that the split-rendering on edge is running and provides its access information.

- The SRC acknowledges the Application that the split-rendering on edge is running.

- The Application requests SRC to start the media delivery.

- The SRC starts the media delivery.

#### 5.2.1.3 Network-driven procedures and call flows

Figure 5.2.1.3-1 demonstrates a call flow for setting up the split rendering by the network.

Figure 5.2.1.3-1: High-level call flow for the network-driven split management

Steps:

- The Application Service Provider requests the provisioning of the split management session.

- The split management session is announced to the Application as part of the Service Access Information.

- The SRS offers the SRC a split of the client media functions.

- The SRC inquires the Media Session Handler about the client’s media capabilities.

- The SRC and SRS negotiate on the acceptable capabilities for the device and agree on the split option.

- The SRS starts the split-rendering process on the edge.

- The SRS acknowledges the SRC that the split-rendering on edge is running and provides its access information.

- The SRC announces to the Application that the split-rendering on edge is running.

- The Application requests the SRC to start media delivery.

- The SRC starts the media delivery.

### 5.2.2 	Call flow for Split Rendering session setup

The split rendering operation can be described by the as depicted in the call flow in the Figure 5.2.2-1.

Figure 5.2.2- 1 High-level call flow for split rendering session setup and operation

The steps are:

- The Presentation Engine discovers the split rendering server and sets up a connection to it. It provides information about its rendering capabilities and the XR runtime configuration, e.g the OpenXR configuration may be used for this purpose.

- In response, the split rendering server creates a description of the split rendering output and the input it expects to receive from the UE.

- The Presentation Engine requests the buffer streams from the MAF, which in turn establishes a connection to the split rendering server to stream pose and retrieve split rendering buffers.

- The Source Manager retrieves pose and user input from the XR runtime.

- The Source Manager shares the pose predictions and user input actions with the split rendering server.

- The split rendering server uses that information to render the frame.

- The rendered frame is encoded and streamed down to the MAF.

**** Last Change ****