Source:	Nokia Corporation

Title:	Proposed updates to Test plan

Document for:	Discussion & Agreement

Agenda Item:	7.5 – IVAS_Codec

# 1. Introduction

IVAS-8a Test plan for the selection phase is targeted for completion at this meeting.

This contribution provides proposals and clarifications to progress the P.SUPPL800 experiments for spatial speech and spatial speech + background.

The related experiments are:

- P800-4: FOA Clean speech

- P800-5: FOA Speech + Background

- P800-8: MASA Clean speech

- P800-9: MASA Speech + Background

# 2. Discussion

There is currently rather limited information available on spatial room impulse responses (SRIR) that could be submitted for IVAS Selection [1], [2]. Another input to this meeting [3] provides discussion on additional SRIR and background signal (or “noise”) databases that could be submitted. Hopefully, there will be even more, e.g., considering suitable pairing of SRIRs and the background signals.

The test signal categories and scene definitions to be used in the P.SUPPL800 experiments depend on the availability of relevant SRIRs and background signals. Thus, either the test plan needs to specify these sufficiently completely for acquisition of corresponding SRIRs and backgrounds or the test plan will need to be completed with sufficient freedom to utilize the databases that will eventually be submitted for use in IVAS Selection.

The source considers that there are currently not enough spatial databases to provide sufficient coverage for the testing. While additional databases are being prepared, the timeline until start of Selection tests is short and appears challenging. Therefore, it should be considered the latter approach to define the categories and scenes with sufficient freedom, e.g., in terms of guidelines, to allow a final decision on the materials (assumed by the MC) based on their later availability.

# 3. Proposal for Speech with background

IVAS-8a currently includes the following in square brackets for P.SUPPL800 experiments:

•	Speech with background: the background comprises car, street, and office noise.

Editor’s note: What this category comprises is for further discussion

More diverse background signals are relevant for immersive use cases than what was tested, e.g., with EVS. While the EVS noise types remain relevant, better coverage would seem desirable for IVAS Selection. Since exact available background signal materials are currently unknown, it could be enough to specify the high-level use cases in each experiment and category at this point. Such high-level use cases could be:

- indoors (e.g., office or meeting environment),

- outdoors (e.g., street, nature, or event), and

- in-transit (predominantly car noise but, e.g., bus/train could be considered if available).

The source has already adopted this approach to define content type categories and scenes for the spatial speech experiments P800-4, P800-5, P800-8, and P800-9 in the following section.

# 4. Proposals for Experiments

The following updates are proposed to IVAS-8a against version contributed to this meeting [4].

## 4.1 P800-4: FOA Clean Speech

The proposed updates in E.4.2 mainly address the room/environment description and background description. In addition, it seems possible to remove the reverb column as the room characteristics depend on the room/environment choice (where there should be different kinds of environments). Talker positions depend on room/environment selection.

The information below the table provides guidelines for talker position selection.

The idea with the environment descriptors is as follows:

- room: specifies an indoor environment

- [X]: specifies selection one of number of similar environment types

- FOA: specifies experiment type

Additionally, for background descriptors we have:

- cleanbg: specified clean background suitable for preceding environment

It is proposed to cover at least 3, and up to 6, different indoor environment + clean background combinations.

A scene is then constructed as environment + background + talker positions.

Start change 1

| Category | Environment(1 |  | Level [dB] | Background(2 | SNR [dB] | Overtalk [s](3 | Bandwidth | Talker positions(4 | Talker selection by panel(5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | room_1_FOA |  | -26 | room_1_cleanbg_FOA | [50] | 1 | Max | tbd | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | room_[1/4]_FOA |  | -26 | room_[1/4]_cleanbg_FOA | [50] | -1 | Max | tbd | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 | room_2_FOA |  | -26 | room_2_cleanbg_FOA | [50] | 1 | Max | tbd | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 | room_[2/5]_FOA |  | -26 | room_[2/5]_cleanbg_FOA | [50] | -1 | Max | tbd | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |
| cat 5 | room_3_FOA |  | -26 | room_3_cleanbg_FOA | [50] | 1 | Max | tbd | P1: f3m3 P2: m1f1 P3: f2m2 P4: m3f3 P5: f1m1 P6: m2f2 |
| cat 6 | room_[3/6]_FOA |  | -26 | room_[3/6]_cleanbg_FOA | [50] | -1 | Max | tbd | P1: m2f2 P2: f3m3 P3: m1f1 P4: f2m2 P5: m3f3 P6: f1m1 |

Notes:

(1 The specific room/environment characteristic and resulting reverb characteristic will be defined by the choice of the specific Spatial Room Impulse Responses used in the convolution process with the raw mono sentences, according to the pertaining stipulations of the test plan IVAS-8a.

(2 Background is defined by the chosen background noise file according to the pertaining stipulations of the test plan IVAS-8a. Backround name ‘clean_bg_[X]_FOA’ indicates a low-noise background corresponding to environment [X], e.g., with low air-conditioning/fan noise.

(3 Overtalk [s] means the duration in seconds by which the two sentences in the sound item uttered by different talkers are overlapping. A negative number means that there is a corresponding pause between the two sentences.

(4 The talker positions are part of the scene definition of the different categories. They should be chosen in a way from the available set of SRIRs for the used room making sure that there is a good coverage of different possible positions. Different selections should be made for the different listener panels.

(5 All sentences by the 6 talkers shall be unique.

Editor’s note: HD 650 are open-back headphones which may cause issue for P.SUPPL800 listening, depending how acoustically isolated are the listening booths. An alternative that was mentioned were  Beyer Dynamic DT 770 Pro.

Editor’s note: FER proposal need to be aligned with Section 5.

Editor’s note: The 50 dB Snr needs clarification.

Editor’s note: The current working assumption is to specify the scenarios in IVAS-8a rather than in IVAS-7a.

End change 1

## 4.2 P800-5: FOA Speech + Background

The proposed updates in E.5.2 mainly address the room/environment description and background description. In addition, it seems possible to remove the reverb column as the room characteristics depend on the room/environment choice (where there should be different kinds of environments). Talker positions depend on room/environment selection.

The information below the table provides guidelines for talker position selection.

The idea with the environment descriptors is as follows:

- [car/out/room]: specifies an environment corresponding to in-car, outdoors, or indoors

- [X]: specifies selected one of number of similar environment types

- FOA: specifies experiment type

Similarly, for background descriptors we have:

- [car/street/park/nature/event/etc.]: specifies a specific background type suitable for the selected environment

- [X]: specifies selected one of number of similar background types

- bg: specifies this is an active background

- FOA: specifies experiment type

It is proposed to cover at least 3, and up to 6, different environment + background combinations.

A scene is then constructed as environment + background + talker positions.

Start change 2

| Category | Environment(1 |  | Level [dB] | Background(2 | SNR [dB] | Overtalk [s](3 | Bandwidth | Talker positions(4 | Talker selection by panel(5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | car_1_FOA |  | -26 | car_1_bg_FOA | 10 | -1 | Max | tbd | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | car_[1/2]_FOA |  | -26 | car_[1/2]_bg_FOA | 10 | -1 | Max | tbd | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 | out_1_FOA |  | -26 | street_1_bg_FOA | [15] | -1 | Max | tbd | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 | out_[1/2]_FOA |  | -26 | [park_1_bg_FOA / nature_1_bg_FOA / event_1_bg_FOA / street_[1/2]_bg_FOA] | [15] | -1 | Max | tbd | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |
| cat 5 | room_[X]_FOA |  | -26 | office_1_bg_FOA | [15] | -1 | Max | tbd | P1: f3m3 P2: m1f1 P3: f2m2 P4: m3f3 P5: f1m1 P6: m2f2 |
| cat 6 | room_[X]_FOA |  | -26 | [cafeteria_1_bg_FOA / mall_1_bg_FOA/ office[1/2]_bg_FOA] | [15] | -1 | Max | tbd | P1: m2f2 P2: f3m3 P3: m1f1 P4: f2m2 P5: m3f3 P6: f1m1 |

Notes:

(1 The specific room/environment characteristic and resulting reverb characteristic will be defined by the choice of the specific Spatial Room Impulse Responses used in the convolution process with the raw mono sentences, according to the pertaining stipulations of the test plan IVAS-8a.

(2 Background is defined by the chosen background noise file according to the pertaining stipulations of the test plan IVAS-8a.

(3 Overtalk [s] means the duration in seconds by which the two sentences in the sound item uttered by different talkers are overlapping. A negative number means that there is a corresponding pause between the two sentences.

(4 The talker positions are part of the scene definition of the different categories. They should be chosen in a way from the available set of SRIRs for the used room making sure that there is a good coverage of different possible positions. Different selections should be made for the different listener panels.

(5 All sentences by the 6 talkers shall be unique.

Editor’s note: HD 650 are open-back headphones which may cause issue for P.SUPPL800 listening, depending how acoustically isolated are the listening booths. An alternative that was mentioned were  Beyer Dynamic DT 770 Pro.

Editor’s note:Types of background need to be aligned with Section 4.5.2.

Editor’s note: The current working assumption is to specify the scenarios in IVAS-8a rather than in IVAS-7a.

End change 2

## 4.3 P800-8: MASA Clean Speech

Clarification on test scope:

Start change 3

Stereo-MASA inputs are tested in IVAS selection phase. The stereo-MASA inputs can have 1-direction spatial metadata (based, e.g., on MASA analysis [28] of original FOA content) or 2-direction spatial metadata (based, e.g., on MASA analysis [28] of original HOA2 content).

End change 3

Proposed details on test item generation in Table E.8.1:

Start change 4

| Common Conditions |  |  |
| --- | --- | --- |
| Test item generation: pre-processing incl. spatialization | 1 | Model-based generation according to convolution of raw mono clean speech sentences with FOA Spatial Room Impulse Responses corresponding to the talker positions relative to a capture point and spatial FOA background. MASA format generation from FOA according to MASA analysis [28]. |

End change 4

Reverting details that were incorrectly changed in harmonization in Table E.8.1:

Start change 5

| ESDRU | 3 | α = [0.85, 0.675, 0.455] |
| --- | --- | --- |

End change 5

The following change updates the conditions in Table E.8.2 (according to corrections in Table E.8.3):

Start change 6

| Trial # | Label | Sample | Condition | Bitrate | FER/Profile |
| --- | --- | --- | --- | --- | --- |
| 1 | c33 |  | EVS | 4x16.4 | [5%, profile] |
| 2 | c02 |  | MNRU Q=32 dB | - | - |
| 3 | c06 |  | ESDRU α = 0.85 | - | - |
| 4 | c13 |  | EVS | 4x16.4 | No errors |
| 5 | c08 |  | ESDRU α = 0.45 | - | - |
| 6 | c21 |  | EVS | 4x7.2 | [5%, profile] |
| 7 | c04 |  | MNRU Q=22 dB | - | - |
| 8 | c01 |  | Reference | - | - |
| 9 | c14 |  | EVS | 4x24.4 | No errors |
| 10 | c07 |  | ESDRU α = 0.65 | - | - |
| 11 | c05 |  | MNRU Q=17 dB | - | - |
| 12 | c24 |  | EVS | 4x24.4 | [5%, profile] |

End change 6

The following update is to correct the number of conditions and parameters in Table E.8.3:

Start change 7

| Label | Condition | Bitrate [kbps] | FER/Profile | ToR |
| --- | --- | --- | --- | --- |
| c01 | Reference | - | - |  |
| c02 | MNRU Q=32 dB | - | - |  |
| c03 | MNRU Q=27 dB | - | - |  |
| c04 | MNRU Q=22 dB | - | - |  |
| c05 | MNRU Q=17 dB | - | - |  |
| c06 | ESDRU α=0.85 | - | - |  |
| c07 | ESDRU α=0.675 | - | - |  |
| c08 | ESDRU α=0.545 | - | - |  |
| c09 | EVS | 3x7.2 | No errors |  |
| c10 | EVS | 4x7.2 | No errors |  |
| c11 | EVS | 4x8 | No errors |  |
| c12 | EVS | 4x9.6 | No errors |  |
| c13 | EVS | 4x16.4 | No errors |  |
| c14 | EVS | 4x24.4 | No errors |  |
| c15 | EVS(+unquantized metadata) | 2x7.2 | No errors |  |
| c16 | EVS(+unquantized metadata) | 2x8 | No errors |  |
| c17 | EVS(+unquantized metadata) | 2x9.6 | No errors |  |
| c18 | EVS(+unquantized metadata) | 2x16.4 | No errors |  |
| c19 | EVS(+unquantized metadata) | 2x24.4 | No errors |  |
| c20 | EVS | 3x7.2 | [5%, profile] |  |
| c21 | EVS | 4x7.2 | [5%, profile] |  |
| c22 | EVS | 4x8 | [5%, profile] |  |
| c23 | EVS | 4x16.4 | [5%, profile] |  |
| c24 | EVS | 4x24.4 | [5%, profile] |  |
| c25 | CuT | 13.2 | No errors | NWT c09 OR NWT c15 |
| c26 | CuT | 16.4 | No errors | NWT c10 OR NWT c15 |
| c27 | CuT | 24.4 | No errors | NWT c11 OR NWT c16 |
| c28 | CuT | 32 | No errors | NWT c12 OR NWT c17 |
| c29 | CuT | 48 | No errors | NWT c13 OR NWT c18 |
| c30 | CuT | 64 | No errors | NWT c14 OR NWT c19 |
| c31 | CuT | 80 | No errors | NWT c14 OR NWT c19 |
| c32 | CuT | 13.2 | [5%, profile] | NWT c20 |
| c33 | CuT | 16.4 | [5%, profile] | NWT c21 |
| c34 | CuT | 24.4 | [5%, profile] | NWT c22 |
| c35 | CuT | 48 | [5%, profile] | NWT c23 |
| c36 | CuT | 64 | [5%, profile] | NWT c24 |

End change 7

The proposed update adds clause E.8.2 to address content type categories and scene definitions. Talker positions depend on room/environment selection and are still tbd.

The information below the table provides guidelines for talker position selection.

The idea with the environment descriptors is as follows:

- room: specifies an indoor environment

- [X]: specifies selection one of number of similar environment types

- MASA: specifies experiment type

Additionally, for background descriptors we have:

- cleanbg: specified clean background suitable for preceding environment

It is proposed to cover at least 3, and up to 6, different indoor environment + clean background combinations.

A scene is then constructed as environment + background + talker positions.

Start change 8

E.8.2 Content type categories and scene definitions (Exp P800-8: Clean speech)

| Category | Environment(1 | Level [dB] | Background(2 | SNR [dB] | Overtalk [s](3 | Bandwidth | Talker positions(4 | Talker selection by panel(5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | room_1_MASA | -26 | room_1_cleanbg_MASA | [50] | 1 | Max | tbd | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | room_[1/4]_MASA | -26 | room_[1/4]_cleanbg_MASA | [50] | -1 | Max | tbd | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 | room_2_MASA | -26 | room_2_cleanbg_MASA | [50] | 1 | Max | tbd | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 | room_[2/5]_MASA | -26 | room_[2/5]_cleanbg_MASA | [50] | -1 | Max | tbd | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |
| cat 5 | room_3_MASA | -26 | room_3_cleanbg_MASA | [50] | 1 | Max | tbd | P1: f3m3 P2: m1f1 P3: f2m2 P4: m3f3 P5: f1m1 P6: m2f2 |
| cat 6 | room_[3/6]_MASA | -26 | room_[3/6]_cleanbg_MASA | [50] | -1 | Max | tbd | P1: m2f2 P2: f3m3 P3: m1f1 P4: f2m2 P5: m3f3 P6: f1m1 |

Notes:

(1 The specific room/environment characteristic and resulting reverb characteristic will be defined by the choice of the specific Spatial Room Impulse Responses used in the convolution process with the raw mono sentences, according to the pertaining stipulations of the test plan IVAS-8a.

(2 Background is defined by the chosen background noise file according to the pertaining stipulations of the test plan IVAS-8a. Backround name ‘clean_bg_[X]_MASA’ indicates a low-noise background corresponding to environment [X], e.g., with low air-conditioning/fan noise.

(3 Overtalk [s] means the duration in seconds by which the two sentences in the sound item uttered by different talkers are overlapping. A negative number means that there is a corresponding pause between the two sentences.

(4 The talker positions are part of the scene definition of the different categories. They should be chosen in a way from the available set of SRIRs for the used room making sure that there is a good coverage of different possible positions. Different selections should be made for the different listener panels.

(5 All sentences by the 6 talkers shall be unique.

End change 8

## 4.4 P800-9: MASA Speech + Background

Clarification on test scope and correction of details:

Start change 9

Stereo-MASA inputs are tested in IVAS selection phase. The stereo-MASA inputs can have 1-direction spatial metadata (based, e.g., on MASA analysis [28] of original FOA content) or 2-direction spatial metadata (based, e.g., on MASA analysis [28] of original HOA2 content).

Tables E.9.1 to E.9.3 show conditions to be used for this experiment, list of preliminaries and full list of conditions, respectively.

Table E.9.1: Factors and conditions for Experiment P800-9 (Speech+Background)

End change 9

Proposed details on test item generation in Table E.9.1:

Start change 10

| Common Conditions |  |  |
| --- | --- | --- |
| Test item generation: pre-processing incl. spatialization | 1 | Model-based generation according to convolution of raw mono clean speech sentences with FOA Spatial Room Impulse Responses corresponding to the talker positions relative to a capture point and spatial FOA background. MASA format generation from FOA according to MASA analysis [28]. |

End change 10

Reverting details that were incorrectly changed in harmonization in Table E.9.1:

Start change 11

| ESDRU | 3 | α = [0.85, 0.675, 0.455] |
| --- | --- | --- |

End change 11

Correcting preliminary conditions and reverting details that were incorrectly changed in harmonization in Table E.9.2:

Start change 12

| Trial # | Label | Sample | Condition | Bitrate | DTX |
| --- | --- | --- | --- | --- | --- |
| 1 | c22 |  | EVS | 4x16.4 | on |
| 2 | c02 |  | MNRU Q=32 dB | - | - |
| 3 | c06 |  | ESDRU α = 0.85 | - | - |
| 4 | c13 |  | EVS | 4x16.4 | off |
| 5 | c08 |  | ESDRU α = 0.45 | - | - |
| 6 | c19 |  | EVS | 4x7.2 | on |
| 7 | c04 |  | MNRU Q=22 dB | - | - |
| 8 | c01 |  | Reference | - | - |
| 9 | c14 |  | EVS | 4x24.4 | off |
| 10 | c07 |  | ESDRU α = 0.65 | - | - |
| 11 | c05 |  | MNRU Q=17 dB | - | - |
| 12 | c23 |  | EVS | 4x24.4 | on |

End change 12

Reverting details that were incorrectly changed in harmonization in Table E.9.3 and correcting typos:

Start change 13

| c06 | ESDRU α=0.85 | - | - |  |
| --- | --- | --- | --- | --- |
| c07 | ESDRU α=0.675 | - | - |  |
| c08 | ESDRU α=0.545 | - | - |  |
| c09 | EVS | 3x7.2 | off |  |
| c10 | EVS | 4x7.2 | off |  |
| c11 | EVS | 4x8 | off |  |
| c12 | EVS | 4x9.6 | off |  |
| c13 | EVS | 4x16.4 | off |  |
| c14 | EVS | 4x24.4 | off |  |
| c15 | EVS(+unquantized metadata) | 2x7.2 | on |  |
| c16 | EVS(+unquantized metadata) | 2x9.6 | on |  |
| c17 | EVS(+unquantized metadata) | 2x16.4 | on |  |

End change 13

The proposed update adds clause E.9.2 to address content type categories and scene definitions. Talker positions depend on room/environment selection and are still tbd.

The information below the table provides guidelines for talker position selection.

The idea with the environment descriptors is as follows:

- [car/out/room]: specifies an environment corresponding to in-car, outdoors, or indoors

- [X]: specifies selected one of number of similar environment types

- MASA: specifies experiment type

Similarly, for background descriptors we have:

- [car/street/park/nature/event/etc.]: specifies a specific background type suitable for the selected environment

- [X]: specifies selected one of number of similar background types

- bg: specifies this is an active background

- MASA: specifies experiment type

It is proposed to cover at least 3, and up to 6, different environment + background combinations.

A scene is then constructed as environment + background + talker positions.

Start change 14

E.9.2 Content type categories and scene definitions (Exp P800-9: Speech+Background)

| Category | Environment(1 | Level [dB] | Background(2 | SNR [dB] | Overtalk [s](3 | Bandwidth | Talker positions(4 | Talker selection by panel(5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cat 1 | car_1_MASA | -26 | car_1_bg_MASA | 10 | -1 | Max | tbd | P1: f1m1 P2: m2f2 P3: f3m3 P4: m1f1 P5: f2m2 P6: m3f3 |
| cat 2 | car_[1/2]_MASA | -26 | car_[1/2]_bg_MASA | 10 | -1 | Max | tbd | P1: m3f3 P2: f1m1 P3: m2f2 P4: f3m3 P5: m1f1 P6: f2m2 |
| cat 3 | out_1_MASA | -26 | street_1_bg_MASA | [15] | -1 | Max | tbd | P1: f2m2 P2: m3f3 P3: f1m1 P4: m2f2 P5: f3m3 P6: m1f1 |
| cat 4 | out_[1/2]_MASA | -26 | [park_1_bg_MASA / nature_1_bg_MASA / event_1_bg_MASA / street_[1/2]_bg_MASA] | [15] | -1 | Max | tbd | P1: m1f1 P2: f2m2 P3: m3f3 P4: f1m1 P5: m2f2 P6: f3m3 |
| cat 5 | room_[X]_MASA | -26 | office_1_bg_MASA | [15] | -1 | Max | tbd | P1: f3m3 P2: m1f1 P3: f2m2 P4: m3f3 P5: f1m1 P6: m2f2 |
| cat 6 | room_[X]_MASA | -26 | [cafeteria_1_bg_MASA / mall_1_bg_MASA/ office[1/2]_bg_MASA] | [15] | -1 | Max | tbd | P1: m2f2 P2: f3m3 P3: m1f1 P4: f2m2 P5: m3f3 P6: f1m1 |

Notes:

(1 The specific room/environment characteristic and resulting reverb characteristic will be defined by the choice of the specific Spatial Room Impulse Responses used in the convolution process with the raw mono sentences, according to the pertaining stipulations of the test plan IVAS-8a.

(2 Background is defined by the chosen background noise file according to the pertaining stipulations of the test plan IVAS-8a.

(3 Overtalk [s] means the duration in seconds by which the two sentences in the sound item uttered by different talkers are overlapping. A negative number means that there is a corresponding pause between the two sentences.

(4 The talker positions are part of the scene definition of the different categories. They should be chosen in a way from the available set of SRIRs for the used room making sure that there is a good coverage of different possible positions. Different selections should be made for the different listener panels.

(5 All sentences by the 6 talkers shall be unique.

End change 14

# 5. Summary

IVAS-8a Test plan for the selection phase is targeted for completion at this meeting.

Relating to IVAS-8a, the source proposes to agree:

- the updates and clarifications to IVAS-8a as detailed in clause 4,

- that the following important high-level use cases (clause 3) are covered by the categories:

  - outdoors (e.g., street noise, nature noise, event noise, etc.),

  - indoors (e.g., office noise, cafeteria noise), and

  - in transit (preferably car noise), and

- that decisions on the categories and the scene descriptions (talker positions, etc.) to be used in IVAS Selection are based on materials available for the Selection tests.

It is proposed to discuss suitable way to implement the above proposals in IVAS-8a.

# References

[1]	S4aA230025: “Talker impulse responses and background noises for IVAS processing”, HEAD acoustics GmbH

[2]	S4aA230033: “On Spatial Room Impulse Recordings”, Dolby Sweden AB

[3]	S4-230899: ” On immersive recordings for IVAS Selection tests”, Nokia Corporation

[4]	S4-230761: IVAS Permanent Document IVAS-8a: Test Plan for Selection Phase, v.0.8.4