3GPP TSG-SA WG4 Meeting #124	S4-230938

# 22nd May 2023 – 26th May 2023, Berlin, Germany

Title:	LS on network slice replacement procedure

Response to:	—

Release:	Rel-18

Work Item:	FS_MS_NS_Ph2

Source:	3GPP SA4

To:	3GPP SA2

Cc:

Contact person:	Prakash Kolan

p dot kolan at samsung dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

As part of its study on Network Slicing Extensions for 5G media services, SA4 is looking into the impact of network slice replacement procedure specified in clause 5.15.19 of TS 23.501 on 5G Media Streaming procedures specified in TS 26.512.

Clause 7.9 of TS 26.512 specifies Policy Templates Provisioning API using which a 5GMS Application Provider configures a set of Policy Templates at the 5GMS AF that can be applied to downlink or uplink media streaming sessions belonging to that Application Provider. A Policy Template is used to specify the traffic shaping and charging policies to be applied to these media streaming sessions. While provisioning each Policy Template, the 5GMS Application Provider includes application session context information to identify application flows to which the policy can be applied. The application session context, among other information, includes network context information such as the S-NSSAI and DNN.

The dynamic policy invocation configuration information is fetched by the Media Session Handler in the UE from the 5GMS AF using the M5 Service Access Information API specified in clause 11.2 of TS 26.512. When the UE wants a different network QoS policy for an application flow in 5G Media Streaming session, the Media Session Handler picks an applicable Policy Template whose application session context matches the session context of the application flow, and requests the 5GMS AF to apply that policy for the application flow. When the 5GMS AF receives this request from Media Session Handler, it interacts with PCF to assist in applying that QoS policy.

Due to the network slice replacement procedure specified in clause 5.15.19 of TS 23.501, if the primary S-NSSAI is replaced with an Alternate S-NSSAI, then it is possible that when the Media Session Handler wants a different network QoS policy, it may not have any Policy Templates that match the current session context of the application flow because the 5GMS Application Provider may not have configured Policy Templates applicable for Alternative S-NSSAI at the 5GMS AF.

The 5GMS Application Provider may intend to provide similar Policy Templates citing the Alternative S-NSSAI as that of the primary S-NSSAI. SA4 has had significant discussion whether the Policy Templates Provisioning API include Alternative S-NSSAI information.

From SA4 perspective, knowing the following information helps in understanding the impact of SA2 defined network slice replacement procedure on SA4 defined 5G Media Streaming procedures:

-	Whether the Alternative S-NSSAI provides similar guarantees as that of the S-NSSAI that it replaces

-	Whether the network slice replacement procedure is transparent to the UE application

-	Whether the Alternative S-NSSAI information is notified to the Application Function prior to, or after, the network slice replacement procedure

-	Whether the Alternative S-NSSAI information is notified to the Application Provider prior to, or after, the network slice replacement procedure

SA4 would like to request that SA2 provide clarification on the above questions. In case the above aspects are not yet looked into in SA2, SA4 would like to request that SA2 provide a response whether SA2 is intending to look into these aspects in Rel-18 or beyond.

# 2	Actions

To SA2

ACTION: 	SA4 kindly requests SA2 to comment on the above questions on whether the Alternative S-NSSAI provides similar guarantees as that of the S-NSSAI it replaces, whether the UE application is aware of network slice replacement, and whether the Alternative S-NSSAI information is shared with or exposed to the Application Function or the Application Provider prior to, or after, the network slice replacement procedure.

ACTION: 	SA4 kindly requests SA2 to inform it of any existing or planned work in this area.

# 3	Dates of next TSG SA WG 4 meetings

SA4#125	21st–25th August 2023		Goteborg, Sweden

SA4#126	13th–17th November 2023		Chicago, US