3GPP TSG-SA4 Meeting #124	S4-230975

# Berlin, Germany, 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.517 | CR | 0001 | rev | 10 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5MBP3] General Updates and Corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBP3 |  |  |  |  |  | Date: |  |  | 2023-05-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Several missing aspects. Inconsistent implementation of semantics Two representation formats were initially added, JSON and XML. XML was considered for backward compatibility with MBMS User service, but not compatibility is achieved. Having two formats is highly confusing and of no benefit for interoperability. JSON was recommended format. Based on this XML is preferably removed The parameters are not described with stage 2 in TS 26.502 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adds bug fixes - Adds tables for semantics - Connect semantics to stage 2 parameters - Remove XML Create annexes fo IANA registration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing features Lack of compatibility and interoperability Not aligned with stage 2 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 3.4, 4, 5, 6.2.1, 6.2.2.2, 6.2.3.3, 7.2.1, A.1, A.2.1, C (new), D (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[6]	3GPP TS 26.502: "5G multicast–broadcast services; User Service architecture".

[7]	3GPP TS 26.346: “MBMS; Protocols and Codecs".

[8]	IETF RFC 8866: "Session Description Protocol".

[9]	W3C: "XML Schema Part 2: Datatypes".

[10]	3GPP TS 23.003: "Numbering, addressing and identification".

[11]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[12]	IETF RFC 3926: "FLUTE - File Delivery over Unidirectional Transport".

[13]	IETF RFC 2616: "Hypertext Transfer Protocol -- HTTP/1.1".

[14]	IETF RFC 3629: "UTF-8, a transformation format of ISO 10646".

[15]	IETF RFC 4648: "The Base16, Base32, and Base64 Data Encodings".

[16]	IETF RFC 8141: "Uniform Resource Names (URNs)".

[17]	ISO 639-2: "Codes for the representation of names of languages – Part 2: Alpha-3 code".

[18]	IETF RFC 6381: "The 'Codecs' and 'Profiles' Parameters for "Bucket" Media Types".

[19]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[20]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3"

=====  CHANGE =====

## 3.4	Syntax documentation conventions

The conventions from TS 29.501 [20] apply to this document with the following extensions for structured data type definitions as defined in clause TS 29.501 [20], clause 5.2.4.1

- Attributes are noted in codestyle. Depending on the data type, the attributes may be in bold face.

=====  CHANGE =====

# 4	System overview

The present document defines protocols and formats for MBS User Services as defined in TS 26.502 [6] and conveyed using the 5G Multicast–Broadcast capabilities of the 5G System defined in TS 23.501 [2], TS 23.502 [3] and TS 23.247 [5].

MBS User Services enable high-level applications to make use of the low-level features of the MBS System. An MBS User Service is provided by the MBSF and MBSTF working in combination to support configuration option 2 and configuration option 3 defined in annex A of TS 23.247 [5]. The MBS User Services architecture is defined in clause 4.2.2 of TS 26.502 [6] and shows the MBS-related entities involved in providing MBS User Services delivery and control.

The MBSF and MBSTF offer service layer functionality for sending MBS data via MBS Sessions. The MBSF offers control plane functionality while the MBSTF offers user plane functionality. The MBSTF acts as a User Plane anchor when it sources IP multicast traffic. Clause 4.3.1 of TS 26.502 [6] defines the set of functional entities involved in supporting MBS User Services, including client functions in the UE.

=====  CHANGE =====

## 5.0	Overview

MBS User Service Announcement is needed in order to advertise MBS User Services in advance of, and potentially during, the MBS User Service Sessions described. MBS User Service Announcement (as defined in clauses 4.5.7 and 4.5.8 of TS 26.502 [3]) is provided by means of an MBS User Service Description, the syntax of which is defined in this clause.

The MBS User Service Announcement is aligned with the MBMS User Service announcement as defined in TS 26.346 [7], but it is simplified and extended for the needs of MBS.

=====  CHANGE =====

### 5.1.1	General

An MBS User Service Description is described by a set of metadata documents that are delivered as described in clause 4.3.2 of TS 26.502 [3]. The data model defined in this clause subdivides the parameters defined in [3] and groups them into a set of metadata documents.

Each metadata document is divided into metadata units. A metadata unit is a single uniquely identifiable block of metadata. The metadata itself describes details of services. An obvious example of a metadata unit would be a single SDP document [8].

The metadata consists of:

-	An MBS User Service Bundle Description metadata unit (see clause 5.2.2) describing a bundle of one or more MBS User Services, and containing one or more:

-	MBS User Service Description metadata unit (see clause 5.2.3) describing an MBS User Service Session that is associated with:

-	One or more MBS Distribution Session Description metadata units (see clause 5.2.4), each of which references a Session Description document [8] that may be packaged with the MBS User Service Bundle Description, and each of which may optionally reference an Object Repair Parameters document (see clause 5.2.7) describing the object repair parameters for the MBS Distribution Session.

-	Zero or more MBS Application Service Description metadata units (see clause 5.2.5), each of which references an Application Service Entry Point document that may be packaged with the MBS User Service Bundle Description. Additional resources referenced by the entry point document may also be packaged with the MBS User Service Bundle Description.

-	Zero or one MBS Schedule Description metadata unit (see clause 5.2.6) advertising the delivery schedule for the MBS User Service Session.

Figure 5.1-1 illustrates the relationships between these metadata units for a single MBS User Service Bundle using the JSON-based representation specified in clause A.2.

NOTE:	“N” means any number in each instance.

Figure 5.1-1: User Service Data Model simple description

An MBS User Service Bundle Description document shall contain one or more instances of the MBS User Service Description metadata unit, each of which describes a single MBS User Service Session within the MBS User Service Bundle.

Each instance of the MBS User Service Description metadata unit shall include at least one MBS Distribution Service Description metadata unit describing the set of MBS Distribution Sessions currently associated with the MBS User Service Session.

-	The MBS Distribution Session Description metadata unit shall refer to one Session Description document.

-	Each MBS Distribution Session Description metadata unit may contain a reference to an Object Repair Parameters document.

Each instance of the MBS User Service Description metadata unit may include zero or more MBS Application Service Description metadata units, each one referencing an Application Service Entry Point document (e.g. a DASH MPD, HLS Master Playlist or HTML document) which describes the root of the Application Service associated with this MBS User Service. When multiple Application Service Entry Point documents are referenced, an MBS Client shall select only one on the basis of a distinct MIME content type indicated in the Application Service Description.

Each instance of the MBS User Service Description metadata unit may include an MBS Schedule Description metadata unit. If included, the MBS Schedule Description shall refer to a Schedule Description document, and the UE can expect to receive MBS User Service data during the time periods described in the Schedule Description document.

In the case of the Object Distribution Method, the Schedule Description document may include an object transmission schedule for objects associated with the MBS User Service Session. The UE may select which objects to receive based on the object transmission schedule information published in the Schedule Description document.

=====  CHANGE =====

## 5.1A	Encoding

The MBS User Service Announcement shall be a JSON instance document formatted according to the schema in clause A.2 with the following constraints:

-	The MIME type of the MBS User Service Description document is specified in clause D.2.

-	The encoding of the MBS User Service Description document shall be UTF-8 as defined in IETF RFC 3629 [14].

-	The extension of the schemas specified in clause A.2, in particular the addition of information elements, is reserved to 3GPP.

-	Additional rules for schema extension are for future study.

=====  CHANGE =====

## 5.2	Syntax and semantics

### 5.2.1	General

The following description in this clause presumes a JSON encoding of the metadata units comprising the MBS User Service Announcement specified in clause 5.1A.

The data types in Table 5.2.1-1 from other 3GPP specifications are reused in the JSON encoding.

Table 5.2.1-1: Externally defined data types used by
MBS User Service Announcement

| Data type | Comments | Reference |
| --- | --- | --- |
| Uri | A Uniform Resource Locator | TS 29.571 [19] |
| DateTime | A date–time value. |  |
| MbsServiceArea | An MBS Service Area. |  |
| MbsFsaId | An MBS Frquency Selection Area identifier. |  |
| DurationSec | A time duration expressed in seconds. |  |

The data types in Table 5.2.1-2 are defined in this specification:

Table 5.2.1-2: Defined data types in this specification

| Data type | Clause |
| --- | --- |

### 5.2.2	MBS User Service Bundle Description metadata unit

The MBS User Service Bundle Description metadata unit conveys one or more MBS User Service Description metadata units. Table 5.2.2-1 provides the detailed semantics for the BundleDescription array that conveys these metadata units.

Table 5.2.2-1: Semantics of BundleDescription array

| Attribute name | Data Type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| BundleDescription | array(User‌Service‌Description) |  | 1..N | MBS User Service Bundle Description metadata unit containing one or more MBS User Service Description metadata units that are announced in this bundle (see clause 5.2.3). |  |

### 5.2.3	MBS User Service Description metadata unit

The MBS User Service Description metadata unit carries information about a single MBS User Service corresponding to an MBS User Service Announcement as defined in clause 4.5.7 of TS 26.502 [6]. Table 5.2.3-1 provides the detailed semantics for the UserServiceDescription object that encodes this metadata unit .

Table 5.2.3-1: Semantics of UserServiceDescription object

| Attribute name |  |  | Use | Data Type | Description |
| --- | --- | --- | --- | --- | --- |
| User‌Service‌Description |  |  |  | object | MBS User Service Description metadata unit. |
|  | serviceId |  | 1 | array(Uri) | The set of External service identifiers as defined in table 4.5.7-1 of TS 26.502 [6]. Each value is expressed as a URI. |
|  | class |  | 1 | Uri | The Service class as defined in table 4.5.7-1 of TS 26.502 [6]. The value is expressed as a URI. |
|  | names |  | 0..N | array | The Service names as defined in table 4.5.7-1 of TS 26.502 [6]. |
|  |  | name | 1 | string | One of the Service names. |
|  |  | lang | 1 | string | Language of the service name encoded using ISO Language code according to ISO 639-2 [17]. |
|  | descriptions |  | 1 | string | The Service descriptions as defined in table 4.5.7-1 of TS 26.502 [6]. |
|  |  | description | 1 | string | One of the Service descriptions. |
|  |  | lang | 1 | string | Language of the description encoded using ISO Language code according to ISO 639-2 [17]. |
|  | serviceLanguage |  | 0..1 | string | The Main service language as defined in table 4.5.7-1 of TS 26.502 [6], using ISO Language code according to ISO 639-2 [17]. |
|  | start |  | 0..1 | DateTime | The Start date-time as defined in table 4.5.7-1 of TS 26.502 [6]. |
|  | end |  | 0..1 | DateTime | The End date-time as defined in table 4.5.7-1 of TS 26.502 [6]. |
|  | distribution‌Session‌Descriptions |  | 1..1 | array(Distribution‌Session‌Description) | One or more MBS Distribution Session Description metadata units (see clause 5.2.4). |
|  | schedule‌Description |  | 0..1 | Schedule‌Description | A schedule description metadata unit (see clause 5.2.7). |
|  | appService‌Description |  | 0..1 | App‌Service‌Description | the MBS Application Service Description metadata unit (see clause 5.2.6) |
|  | availability‌Info |  | 0..1 | Availability‌Information | Provides additional information pertaining to the availability of the MBS Distribution Session within the MBS System. For details, refer to clause 5.2.4. |

### 5.2.4	MBS Distribution Session Description metadata unit

---

The Distribution Sessions Description metadata unit describes one MBS Distribution Session associated with an MBS User Service and carries the MBS Distribution Session Parameters as defined in clause 4.5.8 TS 26.502 [6]. Table 5.2.4-1 provides the detailed semantics for the DistributionSessionDescription object that encodes this metadata unit.

Table 5.2.4-1: Semantics of DistributionSessionDescription object

| Name |  | Use | Type | Description |
| --- | --- | --- | --- | --- |
| Distribution‌Session‌Description |  |  | object | MBS Distribution Session Description metadata unit. |
|  | conformance‌Profiles | 1 | string | A list of profiles indicating the set of features that the MBS Distribution Session conforms to and which the MBS Client needs to support in order to fully decode the MBS Distribution Session. The value of this attribute shall be a fully-qualified term identifier URI from the controlled vocabulary defined in annex C. The values in the controlled vocabulary shall conform to either the pro-simple or pro-fancy productions specified in section 4.5 of IETF RFC 6381 [17], without the enclosing DQUOTE characters, i.e. including only the unencodedv or encodedv elements respectively. The identifier of a profile shall not contain any comma. Profile identifiers defined in the present document are URNs and shall conform to IETF RFC 8141 [16]. Externally defined profiles may use profile identifiers that are URNs or URLs. When a URL is used, it should also contain a month-date in the form mmyyyy; the assignment of the URL must have been authorized by the owner of the domain name in that URL on or very close to that date, to avoid problems when domain names change ownership. |
|  | session‌Description‌URI | 1 | Uri | Provides a URL to a Session Description document carrying the Session Description parameters as defined in table 4.5.8-1 of TS 26.502 [6]. |
|  | object‌Repair‌Parameters | 0..1 | Associated‌Procedure‌Description | The parameters to be used by the MBSTF Client at reference point MBS-4-UC for the unicast object repair procedure, as defined in table 4.5.8-2 of TS 26.502 [6]. |
|  | data‌Network‌Name | 0..1 | string | Indicates a Data Network Name (DNN) as defined in TS 23.003 [10]. |
|  | mbs‌App‌Service | 0..1 | Application‌Service | At most one MBS Application Service Description (see clause 5.2.6 for details). |
|  | unicast‌App‌Services | 0..N | array(Application‌Service) | <needs reference> |

The userServiceDescription object may include an availabilityInfo object providing additional information pertaining to the availability of the MBS Distribution Session within the 5G Network. If present, the availabilityInfo element shall include one or more infoBinding child elements. The infoBinding element shall contain the child elements serviceArea and radiofrequency:

-	The serviceArea element declares the one or more service areas in which the MBS Session corresponding to this MBS Distribution Session is currently available.

-	In the case of a broadcast MBS Session corresponding to this MBS Distribution Session, the mbsFSAId element identifies a preconfigured area within which, and in proximity to, the cell(s) announce the MBS FSA ID and its associated frequency.

NOTE:	This is used to guide frequency selection by the UE for a broadcast MBS Session.

-	The radioFrequency element indicates the one or more radio frequencies in the NG-RAN downlink which transmit the MBS Session corresponding to this MBS Distribution Session in the service area(s) identified by the serviceArea element.

| Name | Use | Type | Description |
| --- | --- | --- | --- |
| Availability‌Information |  | array(Availability‌Information‌Binding) |  |

| Name |  | Use | Type | Description |
| --- | --- | --- | --- | --- |
| Availability‌InformationBinding |  |  | object |  |
|  | mbsService‌Area | 0 ..N | array(Mbs‌Service‌Area) | The Target service areas of this MBS Distribution Session, as defined in table 4.5.8-1 of TS 26.502 [6]. |
|  | mbs‌FSA‌Id | 0 .. 1 | MbsFsaId | The MBS Frequency Selection Area (FSA) Identifier of the MBS Distribution Session in the parent service area, as defined in table 4.5.8-1 of TS 26.502 [6]. |
|  | radio‌Frequency | 0 .. N | array(integer) | The transmission frequency associated with the MBS Frequency Selection Area (FSA) Identifier in the parent service area. |

### 5.2.5	Session Description metadata unit

The sessionDescriptionURI property of the MBS User Service Bundle Description references a Session Description metadata unit. Each Session Description metadata unit shall describe one MBS Distribution Session. The Session Description metadata unit is conveyed in a Session Description document that shall be formatted according to RFC 8866 [8]. The Session Description document may be packaged in the same MBS User Service Bundle Description metadata unit.

-	The session description for the MBS Object Distribution Method is specified in clause 6.2.3

-	The session description for the MBS Packet Distribution Method is specified in clause 7.2.3.

### 5.2.6	MBS Application Service Description metadata unit

In order to support application services in MBS, the MBS User Service Bundle Description metadata unit shall include an appServiceDescription property referencing an Application Service Entry Point document which contains the descriptive information of the resources delivered via MBS and/or unicast distribution.

Table 5.2.6-1 provides the detailed semantics for the MBS Application Service Description metadata unit.

Table 5.2.6-1: Semantics of AppServiceDescription object

| Name |  | Use | Type | Description |
| --- | --- | --- | --- | --- |
| AppServiceDescription |  |  | object | MBS Application Service Description metadata unit. |
|  | media‌Manifest‌Description‌URI | 1 | Uri | Reference to an Application Service Entry Point document, for example to a DASH MPD. |
|  | mimeType | 0 .. 1 | string | Format of the Application Service Entry Point document, for example application/dash+xml. |
|  | identicalContents | 0, 2..N | Application‌Service | Each object expresses two or more identical contents, as indicated by the basePattern values as defined in table 5.2.6-2, for the same resources. For detailed semantics on identical content, refer to clause 7.6.3.1 of TS 26.346 [X]. |
|  | alternativeContents | 0 .. N | Application‌Service | each object expresses two or more alternative contents, as indicated by the basePattern values as defined in table 5.2.6-2. For detailed semantics on alternatove content, refer to clause 7.6.3.2 of TS 26.346 [X]. |

Table 5.2.6-2: Semantics of ApplicationService object

| Name |  | Use | Type | Description |
| --- | --- | --- | --- | --- |
| ApplicationService |  |  | object | Application service |
|  | basePattern | 1 | string | provides a |

If the MBS User Service Description contains a reference to an Application Service Entry Point document, then:

1)	At least one MBS Distribution Session Description of type Object Distribution Method shall be present, i.e. the MBS User Service Description shall include at least one DistributionSessionDescription object in the distributionSessionDescriptions referencing a Session Description Document that describes an Object Distribution Method as defined in clause 7.

2)	When multiple MBS Distribution Session Descriptions of type Object Distribution Method are present, the appServiceDescription property shall define a mapping between the Application Service Entry Point document and the associated MBS Distribution Session.

3)	The MBS Distribution Session described by the Session Description document shall deliver objects that are directly or indirectly referenced by the Application Service Entry Point document.

4)	When the Application Service Entry Point document is a DASH MPD, then all of the following shall hold:

a)	The MBS Distribution Session shall deliver the objects such that the last packet of the delivered object is available to the MBS Client by no later than its availability time as announced in the DASH MPD.

b)	The Content-Location element in the FLUTE File Delivery Table for the delivered object shall match the URL in the DASH MPD.

Editor’s Note: Bullets 4 and 5 should be moved to Clause 7.

4)	If an update to the Application Service Entry Point document is delivered as a FLUTE transmission object then the Content-Location element in the FLUTE File Delivery Table for the delivered object shall match the URL of the referenced Application Service Entry Point document.

In the case of 3GP-DASH formatted content, the appServiceDescription property may refer to a unified media manifest document which describes Representations available for both MBS reception and unicast retrieval, and this shall be used by MBS Clients compliant with this specification. In practical deployments, different subsets of the Representations described by the unified manifest document and referenced by such appServiceDescription may be specified for:

-	Availability via MBS delivery only,

-	Availability via both unicast and MBS delivery,

-	Availability via unicast only, and the Representation is redundant in MBS area coverage, i.e. the usage of these resources does not provide an improved user experience. As an example, this may be a lower bitrate Representation of a media component for which a higher bitrate is available over MBS distribution, and

-	Availability always via unicast, and the Representation is supplementary in MBS area coverage, i.e. even in MBS area coverage these resources provide an improved user experience. As an example, this may be a secondary language that is only accessible over unicast.

All resources that are directly or indirectly referenced in the Application Service Entry Point document of this metadata unit that are expected to be retrieved by HTTP GET shall be delivered by at least one of the MBS Distribution Sessions associated with the MBS User Service Description.

### 5.2.7	MBS Schedule Description metadata unit

Availability of the Schedule Description metadata unit is indicated by the presence of the scheduleDescription property in the MBS User Service Bundle Description metadata unit.

Table 5.2.7-1 provides the detailed semantics for the ScheduleDescription object that encodes the MBS Schedule Description metadata unit.

Table 5.2.7-1: Semantics of ScheduleDescription object

| Name | Use | Type | Description |
| --- | --- | --- | --- |
| ScheduleDescription | 0..N | array(Service‌Schedule) | An array of service schedules with semantics defined in table 5.2.7-2. |

Table 5.2.7-2: Semantics of ServiceSchedule object

| Name |  | Use | Type | Description |
| --- | --- | --- | --- | --- |
| ServiceSchedule |  |  | object |  |
|  | sessionSchedule | 1 | Session‌Schedule | Specifies a time after which MBS Client shall seek to update its schedule information by acquiring the latest available Schedule Description instance document. For details refer to table 5.2.7-3. |
|  | sessionOverrideSchedule | 0..1 | Session‌Schedule‌Override | Indicates either the cancellation of the session occurrence, or schedule override. For details refer to table 5.2.7-5. |
|  | objectSchedule | 0..1 | Object‌Schedule | The Object distribution schedule of the MBS Distribution as defined in table 4.5.8-2 of TS 26.502 [6]. For details refer to table 5.2.7-4. |
|  | serviceId | 1 | string | the value defines the service identifier serviceID of the MBS User Service as defined in table 5.2.3.1 to which this schedule applies |
|  | serviceClass | 1 | Uri | the value defines the service class class of the MBS User Service to which this schedule applies, as defined in table 5.2.3.1. |

A Schedule Description instance document describes the distribution schedule of the MBS User Service and the availability of content via unicast delivery in terms of:

-	Start/stop lists,

-	Recurrence information,

-	The service ID or service class to which the schedule may apply,

An MBS User Service containing multiple content components may be carried on a single MBS Distribution Session, or on multiple MBS Distribution Sessions. The MBS Client can expect to receive MBS data during the described time period(s) when at least one of the MBS Distribution Sessions for the MBS User Service is active.

A Schedule Description instance document may also include a schedule of when individual objects are intended to be transmitted as part of an MBS Distribution Session using the Object Distribution Method. The object schedule information is defined in terms of:

The Schedule Description metadata unit is delivered to the MBS Client

prior to the MBS Distribution Session as part of the MBS User Service Announcement along with the Session Description metadata unit and may be updated subsequently along with the parent MBS User Service Description metadata unit.

.

Table 5.2.7-3 provides the detailed semantics for the SessionSchedule object.

Table 5.2.7-3: Semantics of SessionSchedule object

| Name |  | Use | Type | Description |
| --- | --- | --- | --- | --- |
| SessionSchedule |  |  | object |  |
|  | start | 1 | DateTime | The start time of this Session schedule. |
|  | stop | 1 | DateTime | The stop time of this Session schedule. |
|  | reoccurencePattern | 0..1 | string |  |
|  | numberOfTimes | 0..1 | integer |  |
|  | index | 0..1 | integer | Starting index of the first session occurrence with the index value increased by one for each session reoccurrence. |
|  | reoccurenceStopTime | 0..1 | string |  |
|  | fDTInstanceURI | 0..1 | Uri |  |

Table 5.2.7-4 provides the detailed semantics for the ObjectSchedule object.

Table 5.2.7-4: Semantics of ObjectSchedule object

| Name |  |  | Use | Type | Description |
| --- | --- | --- | --- | --- | --- |
| ObjectSchedule |  |  | 0..1 | array | Object schedule. |
|  | objectURI |  | 0..1 | Uri |  |
|  | sessionId |  | 0..1 | string | Identifies the MBS Distribution Session for each object. If not present, an MBS Client shall instead determine the MBS Distribution Session by examining the Session Description metadata unit for the MBS Distribution Session. |
|  | objectEtag |  | 0..1 | string | Represents the object version identifier. If present, the purpose of this entity tag is to enable an MBS Client to determine if an object has changed since a prior reception without having to download the object. |
|  | unicastOnly |  | 0..1 | boolean |  |
|  | deliveryInfo |  | 0..N | object |  |
|  |  | start | 0..1 | DateTime | The start time of this Object schedule. |
|  |  | stop | 0..1 | DateTime | The stop time of this Object schedule. |

The session schedule and object transmission schedule are described in the Schedule Description instance document respectively by the sessionSchedule and objectSchedule objects.

-	The start and stop time of a single sessionSchedule is specified by the start and stop properties.

-	The start and stop time of a single objectSchedule is specified by the start and stop properties as included in the deliveryInfo object.

In both cases the time is specified as the absolute date and UTC time. The duration may be determined by subtracting the start time from the stop time.

The MBS Distribution Session shall be available to the MBS Client during the time interval(s) announced by the session schedule (i.e. scheduleDescription/serviceSchedule/sessionSchedule element of the Schedule Description instance document), for either unicast or MBS reception. In particular, for unicast reception, the Schedule Description is indicative of the time availability for unicast access of an MBS User Service while the TMGI for the MBS Distribution Session is not activated, as well as for unicast fallback reception when the MBS Client is not located in the MBS coverage area for the service.

The MBS Client may activate reception of that MBS Distribution Session only within the sessionSchedule (and the objectSchedule if present) time window.

When an objectSchedule object is present in a serviceSchedule object, then:

-	The MBS Client should not expect that an object described by an objectSchedule will be updated during a time window instance, defined by start and stop properties, within a deliveryInfo object of that objectSchedule.

-	There shall be only one object version (as defined in the File-ETag attribute in the FLUTE File Delivery Table) transmitted in a time window defined by the start and stop keys within a deliveryInfo element for a given objectSchedule element.

-	If objectETag property is not present, the objects transmitted in the time windows from different deliveryInfo elements in an objectSchedule should not be expected to be the same object version.

-	If the objectETag property is present, there shall be only one object version transmitted in all of the time windows delimited by the start and end attributes of each of the one or more deliveryInfo elements.

-	In-band Schedule Description instance document updates can be used to provide a dynamic schedule update to override the existing delivery schedule, such as using the cancellation mechanism specified in this clause.

-	A sessionSchedule element in the same serviceSchedule element shall be present, and its start and stop properties shall specify a time window that completely overlaps the time windows specified in each of the objectSchedule elements of the same serviceSchedule.

When a sessionSchedule object is present and there are no objectSchedule objects in a serviceSchedule object, then the MBS Client should download each new object, independently of whether the MBS Distribution Session uses the MBS Object Distribution Method or the MBS Packet Distribution Method.

The objectSchedule element specifies details about the objects to be delivered during an MBS Distribution Session. The sessionId property, if present, identifies the MBS Distribution Session for each object. If not present, an MBS Client shall instead determine the MBS Distribution Session by examining the Session Description metadata unit for the MBS Distribution Session. The objectETag property of the objectSchedule element is the version identifier of the object. If present, the purpose of this entity tag is to enable an MBS Client to determine if an object has changed since a prior reception without having to download the object.

The scheduleUpdate object specifies a time after which MBS Client shall seek to update its schedule information by acquiring the latest available Schedule Description instance document.

An index property is included in the sessionSchedule object. If the sessionSchedule does not describe any session reoccurrence, then the index corresponds to the single session occurrence. If the sessionSchedule describes one or more reoccurrences the index is the starting index of the first session occurrence with the index value increased by one for each session reoccurrence.

A cancelled key is defined as a child of the objectSchedule/objectURI element.

-	If the cancelled property is set to "true" or "1", then the transmission of the object identified by the objectURI element is cancelled, and the MBS Client shall cancel any applicable repair and/or reception reporting procedures for that object.

If this object schedule-level cancellation indication in the updated schedule description is received after the associated object has already been delivered, then any related repair or reception reporting for that object (associated with its parent service), either in progress or yet to occur, shall be aborted.

-	If the cancelled key is set to "false" or "0" or is absent, then normal object transmission and associated delivery procedures, if applicable, shall occur.

Table 5.2.7-5 provides the detailed semantics for the SessionScheduleOverride object.

Table 5.2.7-5: Semantics of SessionScheduleOverride object

| Name |  | Use | Type | Description |
| --- | --- | --- | --- | --- |
| SessionScheduleOverride |  |  | object |  |
|  | start | 1 | DateTime |  |
|  | stop | 1 | DateTime |  |
|  | index | 0..1 | integer |  |
|  | cancelled | 0..1 | boolean | The default value if omitted is false. |
|  | sessionDescriptionURI | 0..1 | Uri |  |

A sessionScheduleOverride object is defined as part of the serviceSchedule object. If present, the sessionScheduleOverride object indicates either the cancellation of the session occurrence, or schedule override, as follows:

-	If the cancelled property (a child of sessionScheduleOverride object) is set to "true" or "1", then the transmission of the MBS Distribution Session identified by the index attribute (a child of sessionScheduleOverride object) is cancelled, and the MBS Client shall cancel any applicable repair and/or reception reporting for all objects belonging to that MBS Distribution Session.

If this session schedule-level cancellation indication in the updated schedule description is received after any of the associated objects have already been delivered, then any related repair or reception reporting for those objects (associated with their parent service(s)), either in progress or yet to occur, shall be aborted.

-	If the cancelled property (a child of sessionScheduleOverride object) is set to "false" or "0" or is absent, then the start and stop time elements (children of sessionScheduleOverride object) shall override the nominal start and stop time of the transmission schedule of the session as identified by the index property (a child of sessionScheduleOverride object).

The value of the index property in the sessionScheduleOverride object corresponds to any of the value of the index key in the reoccurenceStartStopType in the sessionSchedule object.

Schedule information received in the Schedule Description metadata unit shall take precedence over timing information that may have been received in the Session Description metadata unit (t and/or r lines in the SDP).

### 5.2.8	MBS Object Repair Parameters metadata unit

An Object Repair Parameters document for the object repair procedures may be delivered to MBS Clients:

-	Prior to the MBS Distribution Session becoming active, along with the MBS Distribution Session Description metadata unit (out of band of that session); or

-	in band within an MBS Distribution Session.

The most recently delivered Object Repair Parameters document shall take priority, such that configuration parameters received prior to – and out-of-band of – the MBS Distribution Session they apply to are regarded as "initial defaults", and configuration parameters received during – and in band with – the MBS Distribution Session, override the earlier received parameters. Thus, a method to update parameters dynamically on a short timescale is provided but, as would be desirable where dynamics are minimal, is not mandatory.

During the User Service Discovery/Announcement Procedure, the Object Repair Parameters document is clearly identified using a URI, to enable UE cross-referencing by the MBS Client of instance documents delivered in band and out of band.

Table 5.2.8-1: Semantics of AssociatedProcedureDescription object

| Name |  | Use | Type | Description |
| --- | --- | --- | --- | --- |
| Associated‌Procedure‌Description |  |  | object |  |
|  | postObjectRepair | 0..1 | Post‌Object‌Repair |  |
|  | mbsObjectRepair | 0..1 | Mbs‌Object‌Repair |  |

Table 5.2.8-1: Semantics of PostObjectRepair object

| Name |  | Use | Type | Description |
| --- | --- | --- | --- | --- |
| Post‌Object‌Repair |  |  | object |  |
|  | serviceURIs | 0..N | array(Uri) |  |
|  | offsetTime | 0..1 | DurationSec |  |
|  | randomTimePeriod | 0..1 | DurationSec |  |

| Name |  | Use | Type | Description |
| --- | --- | --- | --- | --- |
| MbsObjectRepair |  |  | object |  |
|  | sessionDescriptionURI | 0 ..N | Uri |  |

=====  CHANGE  =====

### 6.2.1	General

If FLUTE [12] is used to realise the Object Distribution Method, the MBS Distribution Session shall conform to the MBMS Download Profile as defined in clause L.4 of TS 26.346 [7] with the additional requirements in clause 6.2 of the present document.

The usage of this distribution method is identified in the MBS Session Description metadata unit as defined in clause 6.2.3, in particular by the indication of the protocol FLUTE/UDP in combination with the MBS service type.

The MBSTF shall use the Profiled FDT Schema according to clause L.6 of TS 26.346 [7] to describe the object list currently being transmitted in the MBS Distribution Session.

Generally, the end of transmission of an object is the expiry time for the latest FDT instance describing the object. Objects shall be described in an FDT Instance with the Expires attribute. Depending on the operating mode (clause 6.2.4), different settings of the expiry time and different numbers of objects per FDT Instance are recommended.

Inclusion of the @Content-MD5 and @File-ETag FDT Instance attributes is optional.

The @File-ETag represents the value of the HTTP entity tag as defined in section 3.11 of RFC 2616 [13] which may also serve as the version identifier of the File object described by the FDT Instance.

In order to fetch missing portions of an object, the MBS Client may use the Object Repair services. The Object Repair service is realized as a Byte-Range based File Repair, as specified in clause 9.3.6.2 of TS 26.346 [7].

NOTE:	The use of Alternate-Content-Location-1 and Alternate-Content-Location-2 as defined in TS 26.346 [7] is not supported in MBS User Services.

=====  CHANGE  =====

#### 6.2.2.2	MBS service type of MBS Session

A new MBS service type declaration attribute mbs-servicetype is defined which results in, e.g.:

-	a=mbs-servicetype:broadcast 123869108302929

or:

-	a=mbs-servicetype:multicast 123869108302929

The MBS service type declaration attribute shall be used in Session Description metadata to indicate the type of the corresponding MBS Distribution Session as defined in table 6.2.2.2-1.

Table 6.2.2.2-1: Assignment of mbs-servicetype attribute value

| Attribute value | Meaning |
| --- | --- |
| multicast | The MBS Distribution Session is delivered using a Multicast MBS Session. |
| broadcast | The MBS Distribution Session is delivered using a Broadcast MBS Session. |

The MBS service type attribute shall be declared at session level in the Session Description metadata unit. The session level attribute applies to all media entries without a media-level occurrence of the mbs-servicetype attribute. The Session Description metadata unit shall include only a single instance of MBS service type declaration attribute.

Definition:

-	mbs-service-type-declaration-line = "a=mbs-servicetype:" ("broadcast"/"multicast" SP tmgi) CRLF

-	tmgi = 1*15DIGIT

EXAMPLE:

UK MCC = 234 (MCC Digit 1 = 2; MCC Digit 2 = 3 and MCC Digit 3 = 4)

Vodafone UK MNC = 15

and, with padding, Vodafone UK MNC = 15F (MNC Digit 1 = 1; MNC Digit 2 = 5 and MNC Digit 3 = F)

MBS Service ID = 70A886

Therefore, TMGI = 70A886 32F451 (Hex) or 123869108302929 (Decimal)

The Temporary Mobile Group Identity (tmgi) information element is defined in TS 24.008 [11] including the coding of the fields. Octets 3 to 8 (MBS Service ID, MCC and MNC) shall be placed in the tmgi attribute of the MBS service type declaration line, and are encoded as a decimal number. Octet 3 is the most significant octet. Because this is encoded as a decimal number, leading zeros of the MBS Service ID field may be omitted.

=====  CHANGE  =====

#### 6.2.3.3	Object collection operating mode

Object collection operating mode (OBJECT_COLLECTION) refers to the case in which multiple objects are distributed via the Object Distribution Method.

In this operating mode, each FDT Instance delivered in the session should describe all objects that are part of the current collection.

=====  CHANGE  =====

### 7.2.1	General

The Packet Distribution Method combines three different delivery methods of TS 26.346 [7] (namely the MBMS Streaming Delivery Method, Group Communication Delivery Method and Transparent Delivery Method) into a single distribution method, with a set of modifications.

For the Packet Distribution Method, the MBSTF may handle the ingested content on two different protocol layers according to the operating mode provisioned for the MBS Distribution Session:

-	Proxy mode: The MBSTF handles UDP packet payloads and forwards UDP packet payloads from ingest into the MBS Distribution Session. The MBSTF may use different UDP ports for the MBS Distribution Session. The MBSTF re-uses the Proxy Mode of the Transparent Delivery Method as defined in clause 8B of [7].

-	Forward-only mode: The MBS receives complete IP packets and forwards the ingested packets as MBS PDUs. The MBSTF re-uses the Group Communication Delivery Method as defined in clause 8A of [7] and the Forward-Only Mode of the Transparent Delivery Method as defined in clause 8B of [7].

NOTE:	A specific treatment of RTP sessions, for example as provided by the MBMS Streaming Delivery Method, is not provided by MBS User Services. However, RTP sessions may be delivered in proxy or forward-only mode.

=====  CHANGE  =====

# A.1	XML-based representation

Void.

-

-

=====  CHANGE  =====

# A.2	JSON-based representation

## A.2.1	MBS User Service Announcement schema

Below is the schema specifying the format of MBS User Service Description instance documents using an XML-based representation. Documents following this schema shall be identified with the MIME type application/mbs-user-service-description+json as registered in clause D.5. The schema filename is TS26517_MBSUserServiceAnnouncement.yaml.

openapi: 3.0.0 info: title: 'MBS User Service Announcement Element units’ definition' version: 1.1.0 description: ¦ MBS User Service Announcement Element units. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. externalDocs: description: 3GPP TS 26.517 V17.1.0; 5G System; 5G MBSF; Stage 3. url: http://www.3gpp.org/ftp/Specs/archive/26_series/26.517/ paths: {} components: schemas: BundleDescription: type: array items: $ref: '#/components/schemas/UserServiceDescription' minItems: 1 UserServiceDescription: type: object properties: name: type: array items: type: string serviceLanguage: type: array items: type: string serviceId: type: string distributionSessionDescription: $ref: '#/components/schemas/DistributionSessionDescription' appServiceDescription: $ref: '#/components/schemas/AppServiceDescription' scheduleDescription: $ref: '#/components/schemas/ScheduleDescription' availabilityInfo: $ref: '#/components/schemas/AvailabilityInformation' required: - distributionMethod - serviceId DistributionSessionDescription: type: object properties: conformanceProfile: type: string sessionDescriptionURI: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' objectRepairParameters: $ref: '#/components/schemas/AssociatedProcedureDescription' dataNetworkName: type: string mbsAppService: type: array items: $ref: '#/components/schemas/ApplicationService' unicastAppServices: type: array items: type: object properties: unicastAppService: type: array items: $ref: '#/components/schemas/ApplicationService' required: - sessionDescriptionURI AppServiceDescription: type: object properties: mediaManifestDescriptionURI: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' mimeType: type: string identicalContents: type: array items: type: object properties: unicastAppService: type: array items: $ref: '#/components/schemas/ApplicationService' minItems: 2 alternativeContents: type: array items: type: array items: $ref: '#/components/schemas/ApplicationService' ApplicationService: type: object properties: basePattern: type: string required: - basePattern AvailabilityInformation: type: array items: $ref: '#/components/schemas/AvailabilityInformationBinding' AvailabilityInformationBinding: type: object properties: mbsServiceArea: type: array items: $ref: 'TS29571_CommonData.yaml#/components/schemas/MbsServiceArea' mbsFSAId: $ref: 'TS29571_CommonData.yaml#/components/schemas/MbsFsaId' radioFrequency: type: array items: type: integer minimum: 0 AssociatedProcedureDescription: type: object properties: postObjectRepair: $ref: '#/components/schemas/PostObjectRepair' mbsObjectRepair: $ref: '#/components/schemas/MbsObjectRepair' PostObjectRepair: type: object properties: serviceURIs: type: array items: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' offsetTime: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' randomTimePeriod: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' MbsObjectRepair: type: object properties: sessionDescriptionURI: type: string ScheduleDescription: type: array items: $ref: '#/components/schemas/ServiceSchedule' ServiceSchedule: type: object properties: sessionSchedule: $ref: '#/components/schemas/SessionSchedule' sessionScheduleOverride: $ref: '#/components/schemas/SessionScheduleOverride' objectSchedule: $ref: '#/components/schemas/ObjectSchedule' serviceId: type: string serviceClass: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' required: - serviceId - serviceClass - serviceSchedule SessionSchedule: type: array items: type: object properties: start: $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime' stop: $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime' reoccurencePattern: type: string numberOfTimes: type: integer minimum: 1 reoccurenceStopTime: type: string index: type: integer FDTInstanceURI: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' required: - start - stop SessionScheduleOverride: type: array items: type: object properties: start: $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime' stop: $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime' index: type: integer cancelled: type: boolean sessionDescriptionURI: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' ObjectSchedule: type: array items: type: object properties: objectURI: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' sessionId: type: string objectEtag: type: string unicastOnly: type: boolean deliveryInfo: type: array items: type: object properties: start: $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime' stop: $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

=====  CHANGE  =====

# B.1	XML-based representation

Void.

=====  CHANGE  =====

Annex D (normative):
IANA registration

# D.1	General

This annex provides the formal registrations of MIME media types for different resources specified in the present document. It is referenced from the IANA registry at http://www.iana.org/.

# D.2	Registration of MIME media type "application/mbs-user-service-description+json"

## D.2.1	General

The MIME media type application/mbs-user-service-description+json denotes that the message body is an MBS User Service Description instance document compliant with the YAML schema specified in clause A.2.1.

Table D.2.1-1 provides the MIME media type registration for application/mbs-user-service-description+json.

Table D.2.1-1: MIME media type registration for application/mbs-user-service-description+json

| Parameter | Value |
| --- | --- |
| MIME media type name | application |
| MIME subtype name | mbs-user-service-description+json |
| Required parameters | None |
| Optional parameters | The 'profiles' parameter as specified in clause D.2.2. |
| Encoding considerations | This is a JSON document and the encoding considerations are the same as for media type application/json defined in IETF RFC 8259. |
| Security considerations | This media format is used to configure the receiver on how to participate in a service. This format is highly susceptible to manipulation or spoofing for attacks desiring to mislead a receiver about a session. Both integrity protection and source authentication are recommended to prevent misleading of the receiver. |
| Interoperability considerations | The specification defines a platform-independent expression of an entry point document, and it is intended that wide interoperability can be achieved. |
| Published specification | 3GPP TS 26.517 |
| Applications which use this media type | 3GPP MBS-based applications and services |
| Additional information | File extension(s): json Intended usage: COMMON |
| Other information/general comment | None |
| Person & email address to contact for further information | Thomas Stockhammer (tsto@qti.qualcomm.com) 3GPP TSG SA WG4 |
| Restrictions on usage | None |
| Author/Change controller | 3GPP TSG SA WG4 |

## D.5.2	Profiles parameter

Table D.5.2-1 provides the definition of the profiles parameter to be used with the MBS User Service Description instance document as defined in clause D.5.1.

Table D.5.2-1: Definition of profiles parameter

| Parameter | Value |
| --- | --- |
| Parameter name | profiles |
| Parameter value | Optional attribute indicating one or more profiles to which the resource representation claims conformance. The contents of this attribute shall conform to either the pro-simple or pro-fancy productions specified in section 4.5 of IETF RFC 6381:2011. The set of profile identifiers indicated in this parameter should match the set indicated in the profiles attribute of the corresponding MPEG-DASH MPD (see clause Error! Reference source not found.). |

EXAMPLE:

application/mbs-user-service-description+json;profiles="1,2"