3GPP TSG-WG SA4 Meeting #124-e	S4-230995

# Berlin, Germany, 22th – 26st May 2023											Revision of S4-230835

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.506 | CR |  | rev |  | Current version: | 1.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [GA4RTAR] pCR on 5GS centric architecture for RTC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | GA4RTAR |  |  |  |  |  | Date: |  |  | 16 May 2023 |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This input proposes 5GS centric architecture for RTC as similar as 26.501 for better understanding to the reader. In addition, as raised in opening plenary, the needs of aligning with other SA4 specs (e.g., 5GMS, SR_MSE, MeCAR, etc) are resolved by updating the terminologies and the related texts for further clarifications. Finally, all other structure diagrams are updated similarly. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

Based on the discussion in S4-230834, it is proposed to change the terminologies in TS 26.506 as follows;

|  | As-is | To-be |
| --- | --- | --- |
| 1 | Media Session Handler (MSH) | Real-Time media Communication Media Session Handler (RTC MSH) |
| 2 | 5G-RTC | RTC |
| 3 | 5G-RTC AF | RTC AF |
| 4 | 5G-RTC AS | RTC AS |

Second Change

4	Real-time Media Communication Architecture

4.1	Overall architecture for Real-Time Media Communication (RTC)

Real-Time media Communication over 5G system (5G-RTC) in the context of this specification is defined as the delivery of delay-sensitive media from one peer to another with support of 5G network. AR conversational service described in TR 26.998 [2] is a typical use cases for 5G-RTC, which enables end-users to directly communicate real-time media including AR/MR media contents as specified in TS 26.119 [3]. As identified in clause 8.4 of TR 26.998, there may be different options to enable such AR conversational service, for example re-use of parts of MTSI such as the IMS data channel or 5G Media Streaming for managed services.

The overall 5G-RTC architecture is shown in Figure 4.1-1 as below.

Figure 4.1-1: Real-time media communication (RTC) in 5G System

NOTE:	The functions indicated by the yellow filled boxes are in scope of the present document for RTC. The functions indicated by the grey boxes are defined in 5G System specifications. The functions indicated by the blue boxes are neither in scope of 5G RTC nor 5G System specifications.

The media data is exchanged between two or more RTC endpoints over 5G System. The RTC endpoint is an endpoint configured by RTC architecture in the present document. It is typically a UE, but an edge computing server can also be the RTC endpoint. The Application Provider provides a RTC Aware-Application on the UE to make use of RTC endpoint and network functions using interfaces and APIs. RTC architecture provides the core functions and entities to support WebRTC-based service over 5G System, two main functions are defined in trusted DN.

-	RTC AF: An Application Function similar to that defined in TS 26.501 [6], dedicated to real-time media communication

-	RTC AS: An Application Server dedicated to real-time media communication

NOTE:	Both RTC AF and RTC AS in external DN are out of scope of the present specification.

The detailed RTC architecture mapping to the overall high-level architecture in Figure 4.1-1 is shown in Figure 4.1-2 below. Note that Figure 4.1-2 illustrates only the half portion of Figure 4.1-1 (the link from one RTC endpoint to RTC AF and RTC AS), as the rest of portion is symmetric.

Figure 4.1-2: RTC General Architecture

NOTE1:	Some of functions may not be required depending on the collaboration scenario. Descriptions of collaboration scenario and its architecture variant are specified in Annex A.

NOTE2:	The WebRTC framework is a WebRTC protocol stack and its implementation, defined in W3C and IETF. Media codecs and other media processing functions are specified in TS 26.119 [3].

NOTE3:	Red ovals indicate API provider functions.

The subfunctions inside RTC AF, RTC AS, and RTC endpoint are efined in clause 4.2 and the interfaces shown in Figure 4.1-2 are defined in clause 4.3.

Third Change

4.4.2	Extended 5G-RTC architecture for Edge Computing

4.4.2.1	General

The 5G-RTC architecture can be extended to add support for media processing in the edge. The extended architecture is an integration of the 5G-RTC architecture defined in TS 26.506 with the architecture for enabling Edge Applications defined in TS 23.558 and TS 26.501 [6].

The extended 5G-RTC architecture supports both client-driven as well as Application Function-driven management of the edge processing session.

The 5G-RTC Application Provider may request the deployment of edge resources as part of the Provisioning Session.

-	The 5G-RTC Application Provider provisions the edge provisioning through RTC-1, a similar fashion as defined in TS 26.512 clause 7.10, enabling client-driven and/or Application Function driven edge configuration.

-	In the client-driven approach, the WebRTC Application becomes aware of the support of edge processing in the network and takes steps, such as using the EDGE-5 APIs, to discover and locate a suitable 5G-RTC AS instance in the Edge DN, similar to the process defined in TS 26.501 clause 8.1.

-	In the Application Function driven approach, the 5G-RTC Application Provider requests 5G-RTC AF to deploy edge processing for the media sessions of the corresponding Provisioning Session, similar to the process defined in TS 26.501 clause 8.2. The WebRTC Application may get aware of the deployed EAS through the Application Service Provider through RTC-8 or through the Media Session Handler through RTC-5 (and possibly RTC-6). The EAS is provided together such that the associated can be made by UE between two set of data. Additionally, the EAS may also be discovered through other means, such as DNS resolution with support from the DNS server (e.g., EASDF/DNS resolver) as specified in 3GPP TS 23.548 [1].

When the WebRTC application is a web application, the implementation of the EDGE-5 interface to discover the 5G-RTC AS/EAS location by accessing the EEC is difficult as the Web browser providers may not implement interfaces necessary for supporting edge enabled 5G-RTC applications/services. Also, in the Application Function-driven approach the Application Client (AC) and EEC are not used to discover the 5G-RTC AS/EAS location.

To resolve the above EAS discovery issue in the Application Function-driven approach and when the WebRTC application is a web application, the EAS information can be shared with the Media Session Handler by the 5G-RTC AF using RTC-5 interface.

NOTE:	Other methods that can be used for sharing EAS information (e.g., sharing EAS hostname to the WebRTC application by RTC-8 or by other means and then using DNS resolution) are FFS.

Figure 4.4.2-1: Edge-enabled 5G-RTC architecture

NOTE:	This architecture diagram is an example for CS-2 scenario.

Fourth Change

Annex A (normative):
Architecture variants for collaboration scenarios

A.1	General

This clause addresses the derivative architecture for each of the collaboration scenarios. The four collaboration scenarios are summarized below and further details is specified in Annex A.

The four collaboration scenarios are specified based on the location of required functional entities in trusted domain as defined as follows.

-	5G support for OTT WebRTC: in this scenario the WebRTC session runs completely over the top. However, the MNO may offer support in form of QoS allocation, bitrate recommendations, and QoE report collection based on request by the UE.

-	MNO-provided trusted WebRTC functions: in this scenario the MNO offers trusted support functions such as ICE servers to the WebRTC application on the UE.

-	MNO-facilitated WebRTC services: the MNO hosts and facilitates WebRTC sessions by providing a trusted WebRTC signalling function, which may also offer 5G network assistance.

-	Inter-operable WebRTC services: collaboration scenario 3 is extended with functions to support MNO to MNO inter-operability.

NOTE:	Collaboration scenario 4 is in the scope of this specification. Some of its details, which are not specified in the current version of the document, will be specified, after the relevant works are finished.

The list of key functional entities in trusted domain differs from collaboration scenarios as described in Table A.1-1.

Table A.1-1: Mapping of key functions to each collaboration scenarios

| Functions | Collaboration scenario 1 | Collaboration scenario 2 | Collaboration scenario 3 | Collaboration scenario 4 |
| --- | --- | --- | --- | --- |
| Provisioning function | Optional | Optional | Optional | Optional |
| Configuration function | Optional | Required | Optional (may be fulfilled by WebRTC signalling function) | Optional (may be fulfilled by WebRTC signalling function) |
| MSH | Required | Optional | Optional | Optional |
| Network support function | Required | Required | Optional (may be fulfilled by WebRTC signalling function) | Optional (may be fulfilled by WebRTC signalling function) |
| Trusted ICE function | N/A | Required | Optional | Optional |
| Trusted WebRTC signalling function | N/A | N/A | Required | Required |
| Trusted media function | N/A | Optional | Optional | Optional |

NOTE:	The collaboration scenario 3 may further split depending on the role of MNO, as addressed in TR 26.930.

A.2	Collaboration scenario 1:

Figure A.2-1 shows the architecture variant for the collaboration scenario 1 when the WebRTC session is completely running over the top. For this case, many of WebRTC-related entities are not the scope of this specification. However, Network Support Function is present in the trusted domain to support QoS allocation, bitrate recommendations, and QoE report collection.

Figure A.2-1: Derivative 5G-RTC architecture for collaboration scenario 1

A.3	Collaboration scenario 2:

Figure A.3-1 shows the architecture variant for the collaboration scenario 2 when MNO provides the trusted WebRTC functions such as ICE function. It also contains the configuration function to support the network-assisted WebRTC sessions over 5G system.

Figure A.3-1: Derivative 5G-RTC architecture for collaboration scenario 2

NOTE:	RTC-4m interface is present only when the ICE function contains the TURN server in this scenario.

A.4	Collaboration scenario 3:

Figure A.4-1 shows the architecture variant for the collaboration scenario 3 when MNO hosts the WebRTC sessions by providing the trusted WebRTC signalling function in 5G-RTC AS. In addition, trusted media server is present in 5G-RTC AS to support SFU and MCU functionality.

Figure A.4-1: Derivative 5G-RTC architecture for collaboration scenario 3