# Source: 	China Mobile Com. Corporation

# Title: 	[FS_AI4Media] New network-based AI use-cases

# Agenda item: 	9.7

# Document for:	Discussion and Agreement

- 1 Introduction

- The SID FS_AI4Media defines some media AI related use cases, but none of them are real-time communication. This contribution introduces a new network-based AI use cases for real-time communication.,

  - NLP on Speech

- 2 Discussion

2.1 NLP on Speech in real-time communication

The Clause 4.4 of FS_AI4Media PD defines NLP on speech use case. The main scenario is that UE downloads some training models from the network, and performs AI training locally, then shares the training results with the network to implement distributed learning.

AI-based NLP on speech is especially useful in business communication between different countries/languages, which can be introduced to real-time communication as a new use case to improve call experience. Currently, voice-to-text conversion of WeChat and WhatsApp are completed on the server rather than on the mobile phone. Therefore, we propose to study a use case of network-based NLP on speech in real-time voice/video call. This network-based NLP on speech for video call can be a supplemental use case for NLP on speech use cases.

- 3 Proposed changes

1st Change

4.4 NLP on Speech

4.4.1 NLP on Speech in real-time communication

NLP on speech in real-time communication can be done on both UE and network, or fully on the network. A use case which is fully completed on network is described as below.

UE-B has subscribed intelligent translation service. UE-A initiates an audio/video call and establishes a connection between UE-A and UE-B through IMS network. When detecting that UE-B has intelligent translation service, the IMS serving for UE-B decodes the audio stream from UE-A, recognizes and translates into text of required language based on the AI recognition, then sends the text to UE-B through a new unidirectional channel.

Figure XX :Workflow for NLP on Speech

- 3 Proposal

We propose to add the text of clause 4.4.1 of this contribution to the Permanent Document as new to update the original use cases.