3GPP TSG SA WG4#124	 																			S4-231077

Berlin, De, 22nd – 26th May 2023

Source: 				 Interdigital Finland Oy

Title: 	[MeCAR] PD QoE Metrics

Agenda item: 	9.5

Document for:	Discussion and Agreement

# Discussion

The contribution proposes to populate the section 5.2 of the metrics PD material including:

- Client reference architecture and observation points

- New figure started from 4.3.1-1 including observation points of Figure 6.2-1 from TR 26.812

- New observation points between the scene manager and the presentation engine.

- Add metrics &collection box to the media session handler

- Metrics definition

- Typical latencies in AR Services and delay measurements

- Timestamps and observation points

-  Delay measurements

# Proposed changes

--------------------------------------------- Begin change ------------------------------------------------------------------------

## 5.2	Metrics Observation Points and KPIs

### 5.2.1	Client reference architecture

The XR client reference architecture, shown below in Figure XXX-1 is based on the XR Baseline Client architecture in Figure 4.3.1-1. It also contains a number of observation points where specific metric-related information can be made available to the application. The application can use and combine information from the different observation points to calculate more complex metrics.

Figure XXX-1 – Observation points in client reference architecture

### 5.2.2	Metrics Observation Points

#### 5.2.2.1	Observation Point 1

Observation point 1 (OP-1) is derived from the XR Runtime API which exchanges information between the XR Runtime and the XR Source Management/Presentation Engine. The OP-1 which corresponds to the IF-1, is implemented as an API-1 that exposes functions provided by the XR Runtime. An example of this API is the Khronos OpenXR API.

#### 5.2.2.2	Observation Point 2

Observation point 2 (OP-2) collects information at the input of the Scene Manager. The OP-2 corresponds to the IF-10 for information coming from the application and the IF-9 for data received from the Media Access Function.

#### 5.2.2.3	Observation Point 3

Observation point 3 (OP-3) is derived from the API which exchanges information between the Media Access Function and the 5G System. It corresponds to the IF-4 interface.

#### 5.2.2.4	Observation Point 4

Observation point 4 (OP-4) is derived from the API which exchanges information between the XR Source Management and the Media Access Functions. It corresponds to the IF-3 interface.

#### 5.2.2.5	Observation Point 5

Observation point 5 (OP-5) collects information between the Scene Manager and the Presentation Engine.

Note: the definition of the format on the interface between the Scene Manager and the Presentation Engine is FFS.

### 5.2.3	Timestamps and observation points

The timestamps collected at the observation points are listed in Table YYY-1.

Table YYY-1 Timestamps definition and collection.

| Timestamp | Definition | Observation Point / Interface |
| --- | --- | --- |
| pose prediction time (T1) | The time when the viewer pose prediction is made. It corresponds to the time when the predicted viewer pose is collected using the XR runtime API-1 by the application or the XR Source Manager. | OP-1 / IF-1a |
| lastChangeTime | The time when the user action is made. It corresponds to the lastChangeTime field in the action information defined as the timestamp of the last change to the state of the action. | OP-1 / IF-1a |
| sceneUpdateTime (T6) | The time when the Scene Manager starts to update the 3D scene graph according to the viewer pose and the user actions. | OP-2 / IF-10 |
| startRenderTime (T3) | The time when the renderer starts to render the scene according to the viewer pose. | OP-5 / IF Not Defined |
| predicted displayTime (T2.actual) | It is the predicted display time of the rendered frame in the swapchain. The display time is available through the XR runtime. With OpenXR, the frame’s predicted display time is reported by xrWaitFrame in XrFrameState.predictedDisplayTime. | OP-1 / IF-1c |

### 5.2.4	Delay Measurements

Using all the timestamps collected at the Observation Points, the application may compute the delays and latencies listed in the Table YYY-2.

Table YYY-2 Delays and latencies computation

| Name | Definition | Computation |
| --- | --- | --- |
| pose-to-render-to-photon | The time used to provide the pose information from the XR runtime to the renderer and the renderer using this pose to generate the displayed media. The final pose correction to the latest pose may always be done in the XR runtime. | T2.actual – T1 |
| roundtrip-interaction-delay | The time duration between the moment at which a user action is initiated and the time such an action is presented to the user. This delay is the sum of the age-of-content and the user-interaction-delay. | T2.actual – lastChangeTime |
| render-to-photon-delay | The time duration between the start of the rendering and the display of the rendered frame to the user. | T2.actual – T3 |
| user-interaction-delay | The time duration between the moment at which a user action is initiated and the time such an action is taken into account by the content creation engine in the scene manager. | T6 – lastChangeTime |
| age-of-content | The time duration between the moment the content is created in the scene by the Scene Manager and the time it is presented to the user. | T2.actual – T6 |
| scene-update-delay | the time spent by the Scene Manager to update the scene graph. | T3 – T6 |

### 5.2.5	Relevant KPIs for XR and AR according to TR 26.998

In TR 26.928, some high-level statements on experience KPIs for AR are provided. To achieve Presence in Extended and Augmented Reality, seamless integration of virtual content and physical environment is required. Like in VR, the virtual content has to align with user's expectations. For truly immersive AR and in particular MR, it is expected that users cannot discern virtual objects from real objects.

Also relevant for VR and AR, but in particular AR, is not only the awareness for the user for the environment. This includes, safe zone discovery, dynamic obstacle warning, geometric and semantic environment parsing, environmental lighting and world mapping.

Table 5.2.2-1 provides the KPIs from TR 26.998, Table 4.5.2-1 with  focus on AR and in particular glasses. For some background and additional details refer for example to [10], [11], [49], [50], and [51].

[10]	Daniel Wagner, Louahab Noui, Adrian Stannard, "Why is making good AR displays so hard?", LinkedIn Blog, August 7, 2019, https://www.linkedin.com/pulse/why-making-good-ar-displays-so-hard-daniel-wagner/

[11]	Daniel Wagner, "MOTION TO PHOTON LATENCY IN MOBILE AR AND VR", Medium Blog, August 20, 2018, https://medium.com/@DAQRI/motion-to-photon-latency-in-mobile-ar-and-vr-99f82c480926

[49]	Oscar Falmer: “AR Headsets Landscape”, 10th September 2021, https://docs.google.com/spreadsheets/d/1aUO8nuXWCnL1xYnpe9tvSgCphifhMRLrFj1enayv0X8/edit#gid=0

[50]	Oscar Falmer: “Mobile AR Features Landscape”, 20th September 2021, https://docs.google.com/spreadsheets/d/1S1qEyDRCqH_UkcSS4xVQLgcMSEpIu_mPtfHjsN02GNw/edit#gid=0

[51]	Microsoft, Coordinate Systems, https://docs.microsoft.com/en-us/windows/mixed-reality/design/coordinate-systems

Table 4.5.2-1 AR KPIs from TR 26.998 with relevance for MeCAR

| Feature | KPIs for AR glasses | Relevance for MeCAR |
| --- | --- | --- |
| Tracking |  |  |
| Freedom Tracking | 6DoF | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. |
| Translational Tracking Accuracy | Sub-centimeter accuracy - tracking accuracy of less than a centimeter | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. |
| Rotational Tracking Accuracy | Quarter-degree-accurate rotation tracking is desired | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. |
| AR tracking space | In AR, the tracking space is theoretically unlimited. However, when moving, tracking accuracy may not be assured beyond a certain level of space or trajectory distance. SLAM based methods quickly introduce a large drift in large scale mapping. To correct the scaling issues, a loop closure technique [12] needs to be applied in order to continuously harmonize the local coordinate systems with global ones. | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. |
| World-scale experience | World-scale experiences that let users wander beyond - orientation-only or seated-scale experiences - standing-scale or room-scale experiences To build a world-scale experience, techniques beyond those used for room-scale experiences, namely creating an absolute room-scale coordinate system that is continuously registered with the world coordinate system, typically requiring dynamic sensor-driven understanding of the world, continuously adjusting its knowledge over time of the user's surroundings. | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. |
| Tracking frequency | At least 1000 Hz | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. |
| Latency (for more details refer to clause 4.5.3) |  |  |
| motion-to-photon latency | Less than 20 ms, and preferably even sub 10ms for AR as you may observe movement against the real world. | This latency is run-time internal. It is a device feature and an assumption/requirement for the device, but not a metric to be computed. |
| pose-to-render-to-photon latency | 50-60ms for render to photon is desired in order to avoid wrongly rendered content with late warping applied. | This is a relevant metric and measuring this should be in scope of MeCAR. For details refer to clause 5.2.3. |
| Video Rendering and Display |  |  |
| Persistence – Duty time | Turn pixels on and off every 2 - 4 ms to avoid smearing / motion blur | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. |
| Display refresh rate | 60 Hz minimum 90 Hz acceptable 120 Hz and beyond desired 240 Hz would allow always on display at 4ms | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. |
| Colour | RGB colours Accurate colours independent of viewpoint. | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. |
| Spatial Resolution per eye | for 30 x 20 degrees - 1.5K by 1K per eye is required - 1.8K by 1.2K per eye is desired for 40 x 40 degrees - 2K by 2K required - 2.5 K by 2.5 K desired ultimate goal for display resolution is reaching or going slightly beyond the human vision limit of roughly one arcmin (1/60°) | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. However, the resolution of the content is relevant and is a metric that should be computed. |
| Content frame rates | Preferably matching the display refresh rate for lowest latency Lower frame rates for example 60 fps or 90 fps may be used but add to overall end to end delay. | This is a feature that can be handled together with latency. For more details refer to clause 5.2.3. |
| Brightness | 200-500 nits for indoor Up to 2K for state-of-the-art devices in 2021 [49] 10K to 100K nits for full outdoor experiences | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. |
| Optics |  |  |
| Field of View | Augmentable FoV - typically, 30 by 20 degrees FoV acceptable - 40 by 40 degrees desired maximize the non-obscured field of view | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. |
| Eye Relief | the minimum and maximum eye-lens distance wherein a comfortable image can be viewed through the lenses. at least 10mm, ideally rather 20mm | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. |
| Calibration | correction for distortion and chromatic aberration that exactly matches the lens characteristics | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. |
| Depth Perception | Avoid vergence and accommodation conflict (VAC) for accommodation being different for the real and virtual object | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. However, it relates also to the content and may be checked for metrics. |
| Physics |  |  |
| Maximum Available Power | AR Glass: below 1 W, typically 500mW For less design-oriented devices, additional power may be available. | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. |
| Maximum Weight | AR Glass: around 70g. However, if the weight is well distributed, several hundred grams may be acceptable. | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. |
| Audio Related metrics |  |  |
| Tbd | Tbd | tbd |

### 5.2.6	Typical Latencies in networked AR Services and their Measurements

Building on top of the architectures introduced in TR 26.998, in this document as well as the latency considerations in TR 26.928, Figure 4.5.3-1 provides a summary of different latencies involved networked AR services. Based on TR 26.928 as well as Table 5.2.3-1, two relevant latency requirements for adequate user experience matter:

-	motion-to-photon latency being less 20ms, but preferably even single digit latency below 10ms.

-	pose-to-render-to-photon latency: as small as 50-60ms

It is important to note that the motion-to-photon latency is primarily a function of the device implementation as it is basically covered within the AR runtime. What matters and is relevant is the time used to provide the pose information from the AR runtime to the renderer and the renderer using this pose to generate the displayed media. Final pose correction to the latest pose may always be done in the AR runtime.

Figure 5.2.3-1 (based on Figure 4.5.3-1 from TR 26.998) provides different latency critical uplink and downlink operations, depending on where the rendering is done, locally, in the edge or in the cloud. If done in the edge or cloud, rendered data needs to be delivered in low-latency and high-quality over the network. The typical operations in this case include:

-	pose detection in the UE

-	sending the pose through a 5G uplink network to the edge of cloud.

-	rendering the scene in the edge or cloud

-	compressing and encrypting the rendered scene and delivering to the UE

-	decrypting and decompressing the rendered scene

-	composition of the scene

-	applying the latest pose in the pose correction and display the immersive media.

Note that Figure 5.2.3-1 also adds buffers that are typically handled by the AR Run time, namely eye and depth as well as sound buffers.

Figure 5.2.3-1: Typical Latencies in networked AR services with OPs

It is ultimately relevant that in case of networking the rendering loop, the processes in the loop are executed such that the end-to-end latency requirements for the pose-to-render-to-photon latency are ensured. Clearly the “closer” the rendering happens at the AR UE, the easier it is to meet latency requirements. However, with proper support of 5G system and media functionalities, these networked AR challenges are solved. This is subject of the remaining discussion of this report.

With reference to TR 26.928 , other types of latencies impact the user experience, for example when used for cloud gaming, interactive scenes or in case of real-time network-based processing of sensor data. These aspects are not specific to AR but are also relevant.

Figure 5.2.3-1 also adds the OPs.

Editor’s Note: the above diagram is expected to be further updated and refined based on the agreements of the XR baseline client.

Based on the OPs, a subset of the latencies can be measured.

In order to collect the information related to latencies, the rendering loop of an XR Runtime needs to be understood.

Once a session is running and is in focused state, the rendering loop as shown in Figure 19 can be used:

1)	Before an application can begin writing to a swapchain image, it first waits on the image to avoid writing to it before the Compositor has finished reading from it. Then an XR application synchronizes its rendering loop to the runtime. In the common case that an XR application has pipelined frame submissions, the application is expected to compute the appropriate target display time using both the predicted display time and predicted display interval.

2) 	Once the wait time completes, the application initiates the rendering process. In order to support the application in rendering different views the XR Runtime provides access to the viewer pose and projection parameters that are needed to render the different views. The view and projection info is provided for a particular display time within a specified XR space. Typically, the target/predicted display time for a given frame.

3)	the application then performs its rendering work. Rendering work may be very simple, for example just directly copying data from the application into the swap chain or may be complex, for example iterating over the scene graph nodes and rendering complex objects. Once all views/layers are rendered, the application sends them to the XR Runtime for final compositing including the expected display time as well as the associated render pose.

4)	The XR application offloads the composition of the final image to an XR Runtime-supplied compositor. After the compositor has blended and flattened all layers, it then presents this image to the system’s display at a specific display time.

Core aspect in metrics computation are:

- The time difference when the view and projection info is sampled from the run time and the time when the corresponding rendered information is provided to the run time for display with the associated render pose.

- The difference of the associated render pose and the actual pose at the time when the data is rendered.

Figure 19 - Rendering loop for visual data

Detailed algorithms are ffs.

--------------------------------------------- End change ------------------------------------------------------------------------

# Proposal

We propose to update clause 5.2 of MeCAR PD v0.7.0 with the proposed changes.