3GPP TSG-WG SA4 Meeting #124-e	S4-230997

# Berlin, Germany, 22th – 26st May 2023											Revision of S4-230836

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.506 | CR |  | rev |  | Current version: | 1.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [GA4RTAR] pCR on Editorial update to call flow |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | GA4RTAR |  |  |  |  |  | Date: |  |  | 16 May 2023 |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Update CS1 and CS2 figure to keep the consistancy with CS3 |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

# 5	Procedures for basic RTC architecture

## 5.1	General

The RTC procedures that are defined in this clause are classified based on the collaboration scenarios that are described in Annex A. Depending on the scenario, only a subset of the functions that are defined in clause 4.2 may be involved.

In general, the 5G-RTC call flow may consist of the following procedures.

-	Provisioning

-	Configuration

-	ICE candidates discovery

-	Session establishment

-	QoS request (either client-driven or WebRTC signalling function/server-driven)

-	WebRTC traffic delivery

-	QoS updates

-	Session termination

## 5.2	Provisioning Procedure

An application provider may use the RTC-1 interface to provision network assistance and other resources for its RTC sessions.

This procedure is common to the different collaboration scenarios.

Figure 5.2-1: Provisioning procedure

## 5.3	Call flow for Over-the-top (OTT) RTC sessions (CS#1)

The RTC session is established between two endpoints using external signalling mechanisms. Each endpoint of the connection that is using the 5G system may benefit from 5G network support for the network path within that 5G network.

The following call flow applies to this scenario.

Figure 5.3-1: Call flow for Over-the-top (OTT) RTC sessions (collaboration scenario 1)

The working assumptions are:

-	The application on UE1 and the remote endpoint (e.g., UE2 or server for edge computing) use an external WebRTC signalling server to establish the WebRTC session.

0.	A provisioning session may have been created by the AP with the MNO.

Network assistance for the RTC session is achieved through the following steps:

1.	The application on UE1 uses application-specific signalling functions to establish a WebRTC session with remote endpoint.

2.	The application informs the MSH about the new RTC session and shares information about the media streams and their associated 5-Tuples.

3.	The MSH requests network assistance for the RTC session and provides the transport and bandwidth information to the Network Support AF.

4.	The Network Support AF uses the N5 or N33 interface to request QoS allocation. It may request differential charging based on pre-existing provisioning for these sessions.

5.	Confirmation of QoS allocation is notified to the Network Support AF and the MSH.

6.	The Network Support AF will also subscribe to events related to the QoS flows of the RTC session with the PCF and SMF.

7.	The Network Support AF receives notifications about any changes to the QoS flows of the RTC session from the PCF or the SMF.

8.	The Network Support AF sends notifications to the MSH about changes to the session. This information may contain for example be bitrate recommendations.

9.	The MSH forwards the bitrate recommendation to the RTC application.

10.	The application may act on the bitrate recommendation, e.g. by reducing the uplink media bitrate.

11.	The application may request remote endpoint to adjust the bitrate of the downlink media.

## 5.4	Call flow for Network-supported RTC sessions (CS#2)

The MNO offers access to trusted ICE functionality to UEs that wish to participate in RTC sessions. The session establishment takes into account the configured trusted ICE functions.

The call flow is as follows.

Figure 5.4-1: Call flow for Network-supported RTC sessions (collaboration scenario 2)

The working assumptions are:

-	The application on UE1 and remote endpoint use an external WebRTC signalling server to establish the WebRTC session.

0.	A provisioning session may have been created by the AP with the MNO.

Call flow using network-supported RTC session is achieved through the following steps:

1.	The AF uses the RTC-5 interface to provide the MSH with a list of trusted STUN/TURN servers that the UE may use for establishing RTC sessions.

2.	The application queries the MSH for the list of trusted ICE servers.

3.	The UE discovers and tests the ICE candidates to validate that they are suitable for the connection.

4.	The application on UE1 and the remote remote endpoint use an external RTC signalling server to exchange information about ICE candidates and to exchange the SDP offer/answer.

Then, the WebRTC session is established using the most suitable ICE candidate.

5.	The STUN or TURN server in ICE function, upon reception of the allocation request by the application (or WebRTC framework) may extract the 5-Tuple information for each of the media sessions and convey the information to the Network Support AF in 5G-RTC AF for requesting QoS assistance.

6.	The Network Support AF uses the N5 interface to request QoS allocation. It may request differential charging based on pre-existing provisioning for these sessions.

7.	Confirmation of QoS allocation is notified to the Network Support AF and the MSH.

8.	The Network Support AF will also subscribe to events related to the QoS flows of the WebRTC session with the PCF and SMF.

9.	The Network Support AF receives notifications about any changes to the QoS flows of the WebRTC session from the PCF or the SMF. Then, the Network Support AF sends notifications to the ICE function (STUN/TURN server).

10.	The STUN/TURN server may forward the bitrate recommendation to the application, if the allocation session is still active.

11.	The application may act on the bitrate recommendation, e.g. by reducing the uplink media bitrate.

12.	Media traffic is delivered to remote endpoint. If TURN server is present in the configuration, RTC-4m interface is involved.

## 5.5	Call flow for MNO-Facilitated RTC sessions (CS#3)

In collaboration scenario 3, MNO hosts the WebRTC sessions by providing a trusted WebRTC signalling server in the 5G-RTC AS. In addition, a trusted media server is also present in 5G-RTC AS to support SFU and MCU functionality. Note that, when the WebRTC application is a web-based application, the MSH function is not supported.

The call flows for this scenario when MSH is involved are as shown in Figure 5.5-1.

Figure 5.5-1: Call flows for MNO facilitated 5G-RTC sessions (collaboration scenario 3)

The 5G-RTC Application Provider may create a Provisioning Session with the 5G-RTC AF and starts provisioning the usage of the RTC Streaming session between two endpoints. During the establishment phase, the used features such as content consumption measurement, logging, collection and reporting; QoE metrics measurement, logging, collection and reporting; dynamic policy; network assistance; are negotiated and detailed configurations are exchanged.

The 5G-RTC Application Provider Provisioning Session phase is optionally performed prior to the establishment of any related WebRTC sessions by the 5G-RTC Application Provider. Detailed procedure is addressed in clause 5.2.

The ICE candidate discovery phase is performed with the following steps in an MNO-facilitated 5G-RTC system:

1.	Configuration information: The 5G-RTC AF uses the RTC-5 interface to provide the MSH with a list of trusted STUN/TURN servers, trusted WebRTC signalling and data channel servers and their capabilities. The UE may use this configuration information for establishing RTC sessions.

2.	ICE Servers request: The application queries the MSH for the list of trusted ICE servers.

3.	ICE candidate validation: The UE discovers and tests the ICE candidates to validate that they are suitable for the connection.

The WebRTC session establishment phase is performed with the following steps in an MNO-facilitated 5G-RTC system:

4.	Query configuration information: The WebRTC framework queries the MSH for the WebRTC signalling function information. In some cases where the signalling is not handled by WebRTC framework, the native WebRTC application queries the MSH for the WebRTC Signalling function information.

5.	Configuration information: MSH sends the WebRTC signalling and data channel servers and their capabilities information to WebRTC framework or in some cases with native WebRTC application.

In SDP exchange phase, two or more WebRTC endpoints exchange signalling information related to the WebRTC session such as ICE candidates, SDP offer/answer using the trusted WebRTC signalling function.

NOTE:	Figure 5.5-1 illustrates that SDP offer is generated by the WebRTC Framework or native WebRTC Application. However, in SFU/MCU mode, SDP offer is generated by Media Function in 5G-RTC AS.

6.	SDP offer: The WebRTC Framework or native WebRTC Application creates a request with SDP offer which includes the ICE candidates and sends it to the WebRTC signalling function.

7.	Determine remote endpoint location: The WebRTC signalling function uses the registration information to locate the remote endpoint

8.	SDP offer: The WebRTC signalling function forwards the request to remote endpoint.

9.	SDP answer: Upon accepting the offer, remote endpoint responds to signalling function with SDP answer.

10.	SDP answer: WebRTC signalling function sends the SDP answer to the UE1.

With this, a WebRTC session is established between WebRTC endpoints using the most suitable ICE candidate and the WebRTC signalling function.

The Dynamic policy phase is then performed to allocate QoS for the media streams of the RTC session with the following steps:

11.	QoS request: The WebRTC signalling function sends a request to 5G-RTC AF for the allocation of QoS for the session. The 5G-RTC AF sends a request to the PCF to allocate QoS for the media streams of the RTC session

12.	Confirmation: PCF or SMF confirms the successful allocation of network support or QoS allocation.

If the Network support function feature is supported in the 5G-RTC AF, then the Network Support Function AF (NS-AF) offers the bitrate recommendation request API based on existing policy templates, through the usage of either the Npcf_PolicyAuthorization API over N5 interface, or the Nnef_AFSessionWithQoS over N33 interface to the PCF. If no corresponding AF application session context already exists, the NS-AF uses the Npcf_PolicyAuthorization_Create method over N5 interface with the appropriate service information to create and provision an application session context. The Network assistance phase is performed with the following steps in an MNO-facilitated 5G-RTC system.

13.	Subscribe to QoS events: The NS-AF also subscribes to events related to the QoS flows of the WebRTC session with the PCF and SMF.

14.	QoS events: The NS-AF receives notifications about any changes to the QoS flows of the WebRTC session from the PCF or the SMF.

15.	QoS notifications/ Bitrate recommendations: The NS-AF may send notifications to the MSH about the changes in QoS flow. When the allocated session is active, the MSH forwards the bitrate recommendation to the application.

16.	Adjust media bitrate: The WebRTC application may act on adjusting the bitrate recommendation, e.g., by reducing the uplink media bitrate.

After successful creation of a WebRTC session and the bitrate negotiations, the actual WebRTC session over 5G may start:

17.	Media transfer:

a)	The WebRTC Application may connect to the selected TURN server and/or Media Function in the “5G-RTC AS” through the RTC-4m interface and real-time communication starts, and the media is delivered to the remote endpoint.

b)	In some cases, a peer-to-peer connection is also possible and the media is delivered directly to the remote endpoint.

18.	Method calls and notifications: Supporting information about the WebRTC session is passed from the WebRTC framework to the Media Session Handler.

19.	Reporting, network assistance, and dynamic policy: The Media Session Handler exchanges supporting information about the WebRTC session with the 5G-RTC AF.

20.	End session: The WebRTC Application informs the WebRTC framework that the RTC session has ended. It is also sent to the WebRTC Signalling Function to terminate the session.

21.	Session ending event: The WebRTC framework informs the Media Session Handler about the end of the RTC session.

22.	Final reporting: The Media Session Handler performs any final reporting to the 5G-RTC AF.