3GPP TSG-SA4 Meeting #124	S4-230989

# Berlin, Germany, 22nd May 2023 - 26th May 2023						revision of S4-230821

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.925 | CR | 0003 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_XRTraffic] Proposed Updates to TR 26.925 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Tech. Netherlands B.V |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_XRTraffic |  |  |  |  |  | Date: |  |  | 2023-05-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | XR Traffic characteristics are not complete |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds XR Traffic characteristics Provides updates and bug fixes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.6, 5.5, 5.6, 6.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TR 26.926 CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.234: "Transparent end-to-end Packet-switched Streaming Service (PSS); Protocols and codecs".

[3]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[4]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs ".

[5]	3GPP TS 26.116: "Television (TV) over 3GPP services; Video profiles".

[6]	3GPP TS 26.118: "3GPP Virtual reality profiles for streaming applications".

[7]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[8]	3GPP TS 26.223: "Telepresence using the IP Multimedia Subsystem (IMS); Media handling and interaction ".

[9]	Recommendation ITU-R BT.1872-3 (10-2019): " User requirements for broadcast auxiliary services including digital television outside broadcast, electronic/satellite news gathering and electronic field production".

[10]	3GPP TS 23.501: "System Architecture for the 5G System ".

[11]	3GPP TS 26.238: "Uplink streaming".

[12]	ST 2110-10:2017 - SMPTE Standard - Professional Media Over Managed IP Networks: System Timing and Definitions.

[13]	Video Services Forum (VSF) Technical Recommendation TR-05, Essential Formats	and Descriptions for Interoperability of	SMPTE ST 2110-20 Video Signals.

[14]	Recommendation ITU-T H.264 (04/2017): "Advanced video coding for generic audiovisual services" | ISO/IEC 14496-10:2014: "Information technology - Coding of audio-visual objects - Part 10: Advanced Video Coding".

[15]	Recommendation ITU-T H.265 (12/2016): "High efficiency video coding" | ISO/IEC 23008-2:2015: "High Efficiency Coding and Media Delivery in Heterogeneous Environments - Part 2: High Efficiency Video Coding".

[16]	3GPP TR 26.949: "Video formats for 3GPP services".

[17]	IETF RFC 793: "TCP"

[18]	https://www.ietf.org/id/draft-ietf-quic-transport-19.txt.

[19]	DVB BlueBook A176: "Adaptive media streaming over IP multicast - reference architecture".

[20]	CableLabs OC-TR-IP-MULTI-ARCH: "IP Multicast Adaptive Bit Rate Architecture Technical Report".

[21]	"How youtube led to Google's cloud-gaming service", spectrum.ieee.org | SEP 2019 | 09.

[22]	3GPP TR 22.842: "Study on Network Controlled Interactive Services (Release 17)".

[23]	"Cloud Gaming: Architecture and Performance", Ryan Shea and Jiangchuan Liu, Simon Fraser University; Edith C.-H. Ngai, Uppsala University; Yong Cui, Tsinghua University; IEEE Network-July/August 2013.

[24]	Jens-Rainer Ohm, Gary J. Sullivan, Heiko Schwarz, Thiow Keng Tan, and Thomas Wiegand, "Comparison of the Coding Efficiency of Video Coding Standards—Including High Efficiency Video Coding (HEVC)" IEEE TRANSACTIONS ON CIRCUITS AND SYSTEMS FOR VIDEO TECHNOLOGY, VOL. 22, NO. 12, DECEMBER 2012.

[25]	T.K. Tan, M. Mrak, R. Weerakkody, N. Ramzan, V. Baroncini, G.J. Sullivan, J.-R. Ohm, K.D. McCann, "HEVC subjective video quality test results", IBC2014 Conference, 2014.

[26]	Thiow Keng Tan ; Rajitha Weerakkody ; Marta Mrak ; Naeem Ramzan ; Vittorio Baroncini, Jens-Rainer Ohm, Gary J. Sullivan, "Video Quality Evaluation Methodology and Verification Testing of HEVC Compression Performance" IEEE Transactions on Circuits and Systems for Video Technology, Volume: 26, Issue: 1, Jan. 2016.

[27]	Minhua Zhou, Jianle Chen, Kiho Choi and Dmytro Rusanovskyy, "Tool comparison between VVC (VTM3.0) and NVC", ISO/IEC JTC1/SC29/WG11 MPEG2019/ m46554, Marrakech, Morocco, January 2019.

[28]	Frank Bossen, Xiang Li, Karsten Suehring, "AHG report: Test model software development (AHG3)", JVET-M0003, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 13th Meeting: Marrakech, MA, 9-18 Jan. 2019.

[29]	K. Choi et al., "Description of video coding technology proposal by Samsung, Huawei, and Qualcomm for New Video Coding Standard", MPEG-M46354, Marrakech, Morocco, January 209.

[30]	3GPP TR 26.928: "Extended Reality over 5G".

[31]	ST 297:2006 - SMPTE Standard - For Television — Serial Digital Fiber Transmission System for SMPTE 259M, SMPTE 344M, SMPTE 292 and SMPTE 424M Signals.

[32]	ST 2081-10:2018 - SMPTE Standard - 2160-line and 1080-line Source Image and Ancillary Data Mapping for 6G-SDI.

[33]	ST 2082-1:2015 - SMPTE Standard - 12 Gb/s Signal/Data Serial Interface — Electrical ST 2082-1:2015.

[34]	SMPTE ST-2083, 24G-SDI, In development.

[35]	3GPP TR 22.827: "Study on Audio-Visual Service Production".

[36]                      ST 2042-1:2017 - SMPTE Standard - VC-2 Video Compression

[37]                      ITU-T/T.802 | ISO/IEC 15444-3 "Information technology - JPEG 2000 image coding system - Part 3: Motion JPEG 2000

[38]                     ITU-T G.1032 "Influence factors on gaming quality of experience" (https://www.itu.int/rec/T-REC-G.1032-201710-I/en)

[39]                     ITU-T P.809 "Subjective evaluation methods for gaming quality" (https://www.itu.int/rec/T-REC-P.809/en)

[40]                     M. Mathis, J. Semke, J. Mahdavi, The macroscopic behavior of the TCP congestion avoidance algorithm. Comput. Commun. Rev. (ACM SIGCOMM) 27(3), 67–82 (1997)

[41]                     Yusuke Miki, Tsuyoshi Sakiyama, Kenichiro Ichikawa, Mayumi Abe, Seiji Mitsuhashi, Masayuki Miyazaki , "Ready for 8K UHDTV broadcasting in Japan", IBC 2015 Conference.

[42]	Hirokazu Kamoda, "NHK Launched World’s First 8K Broadcasting in Japan", NAB Pilot guest blog post (https://nabpilot.org/nhk-launch-worlds-first-8k-broadcasting/)

[43]	Thierry Fautier, Eric Mazieres, France Television Lab blog post "8K EXPERIMENT AT ROLAND GARROS 2019" (https://www.francetelevisions.fr/lab/projets/8K-Experiment-at-Roland-Garros-2019)

[44]	M. Aracena, T. Fautier, O. Oyman, "Live VR end-to-end workflows: real-life deployments and advances in VR and network technology", SMPTE 2020 Annual Technical Conference & Exhibition, November 2020, https://www.vr-if.org/wp-content/uploads/vrif2020.115.01.pdf.

[45]	3GPP TS 26.511, "5G Media Streaming (5GMS); Profiles, codecs and formats".

[46]	3GPP TR 26.955, "Video codec characteristics for 5G-based services and applications".

[47]	3GPP TR 26.926, "Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems".

=====  CHANGE =====

## 4.6	XR Media Services

For details on XR Services including AR and VR, refer to 3GPP TR 26.928 [30] and 3GPP TR 26.926 [47]. Identified scenarios are:

-	Viewport-independent 6DOF streaming, see TR 26.926 [47], clause 6.2.2.

-	Viewport-dependent 6DOF streaming, see TR 26.926 [47], clause 6.2.3.

-	Raster-based split rendering, see TR 26.926 [47], clause 6.2.5.

-	Generalized split rendering, see TR 26.926 [47], clause 6.2.6.

-	XR conversational, see TR 26.926 [47], clause 6.2.7.

=====  CHANGE =====

## 5.5	Typical Traffic Characteristics for Cloud gaming

For cloud gaming, the downlink streaming of 720p/1080p/4k @60fps encoded A/V typically consists of a 5-35Mbps bitstream. One instance of a cloud gaming service requires a minimum uplink bitrate of 1.5 Mbps [21].

In the future the cloud gaming is presumed to reach up to 8k resolutions and up to 120fps downlink bitstreams. No information on such currently deployed services are available to formulate typical bitrates. However, Clause 6 provides indication that allows estimation of bitrates. Annex A provides background information on deployed cloud gaming services.

Different game types result in different round-trip user interaction delay requirements (sometimes referred also as acceptable game latency). As discussed in TR 26.928 [30], clause 4.2, with regards to such requirements, games may be divided into the following 4 types: games requiring (i) at most 50 ms, (ii) at most 100 ms, (iii) at most 200ms, and (iv) games with no latency requirements. The game latency impacts the traffic model as well as the requirements on the delivery system. The shorter the latency requirements, the higher the expected bitrate.

Cloud gaming traffic characteristics are also discussed in 3GPP TR 26.928 [30], clause 6 as well was TR 26.926 [47].

=====  CHANGE =====

## 5.6	XR Traffic Characteristics

Initial typical bitrates and traffic characteristics for XR services are collected in 3GPP TR 26.928 [30], clause 6 as well was TR 26.926 [47]. A summary of expected bitrates and traffic characteristics are provided:

-	Viewport-independent 6DOF streaming

-	Downlink only

-	HTTP Streaming

-	Up to 100 Mbps to address high-quality 6DOF VR services to allow 2k per eye streaming at 90 fps (see TR 26.928, clause 4.2 and 6.2.2)

-	Viewport-dependent 6DOF streaming

-	Downlink only

-	HTTP streaming, HTTP/TCP level information and responses are exchanged every 100-200 ms in viewport-dependent streaming.

-	Up to 25 - 50 Mbps to address high-quality 6DOF VR services to allow 2k per eye streaming at 90 fps (see TR 26.928, clause 4.2 and 6.2.3)

-	Raster-based split rendering

-	Primarily downlink

-	for H.264/AVC the bitrates are in the order of up to 50 Mbps per eye buffer, i.e. up to 100 Mbit/s.

-	for H.265/HEVC the bitrates are in the order of 20 - 30 Mbps per eye buffer, i.e. 40 – 60 Mbit/s

- 	latency requirements are in the range of 15ms

-	uplink pose information  (see TR 26.926 [47], clause 5.8)

- 	typically every 10 -15ms constant packet size of up to 100 bytes

-	latency requirements are 10 – 15 ms

-	Generalized split rendering

-	details are FFS

-	XR conversational

-	details are FFS

=====  CHANGE =====

### 6.1.2	Compression Improvements

Due to the increasing consumption of video content with higher resolutions, the need for more efficient video compression techniques is growing. The first version of the High Efficiency Video Coding (HEVC) standard [15], jointly developed by the ITU-T VCEG and the ISO MPEG, was finalized in 2013. A wide range of products and services support HEVC [15] for video encoding/decoding, especially for Ultra High Definition (UHD) content, where HEVC [15] can provide around 50% bitrate savings for the same subjective quality as its predecessor H.264/AVC [14].

Both codecs are defined as part of the TV Video Profiles in TS 26.116 [5] and are also the foundation of the VR Video Profiles in TS 26.118 [6].

Work on video compression technologies beyond the capabilities of HEVC [15] are continued by the MPEG/ITU, with the creation of the Joint Video Exploration Team (JVET) on future video coding in October 2015. Many new coding tools have been proposed in the context of JVET, which eventually led to a Call for Proposals on video coding technologies with video compression capabilities beyond HEVC [15]. The reference software used in the exploration phase of JVET, called Joint Exploration Model (JEM), was leveraged as the base for the majority of responses to the call. Results included responses demonstrating compression efficiency gains of around 40 % or more with respect to HEVC [15]. This initiated the work by the Joint Video Experts Team (JVET) on the development of a new video coding standard, to be known as Versatile Video Coding (VVC).

MPEG has started working on a new video coding standard to be known as MPEG-5 Essential Video Coding (EVC) in January 2019. MPEG-5 EVC aims to provide a standardized video coding solution to address business needs in some use cases, such as video streaming, where existing ISO video coding standards have not been as widely adopted as might be expected from their purely technical characteristics. In addition, a main profile adds a small number of additional tools, each of which is individually capable of being either properly deactivated or switched to the corresponding basic tool. The target coding efficiency for the call for proposals was to be at least as efficient as HEVC. This target was exceeded by approximately 24 % in the responses to the call for proposals, which were evaluated at this meeting. The development of the MPEG-5 EVC standard is expected to be completed in 2020.

Figure 6.1.2-1 shows the typical improvements of video compression rates over time as well as the target for the VVC standard. It is also observed that compression technologies have enabled the reduction of bitrates by 50 % in a time frame of 7-10 years. Most of the gains come by the increase of encoding and decoding complexity, spurred according to Moore's law.

Figure 6.1.2-1: Video bitrate efficiency improvements and target for the final VVC standard [reproduced with appropriate permission from Fraunhofer]

Table 6.1.2-1 provides a summary of the expected compression efficiency of different codecs and expectations on target bitrates for different video technologies.

Table 6.1.2-1: Expected Video coding standards performance and bitrate target

| Codec | Coding performance (Random-Access) |  | Targeted bitrate (Random Access) |
| --- | --- | --- | --- |
|  | Objective | Subjective |  |
| HEVC | -40 % vs AVC [24][25][26] | -60 % vs AVC [24][25][26] | 4k: - Statmux: 10-13 Mbps - CBR: 18-25 Mbps 8k: - CBR: 40-56 Mbps - High quality: 80-90 Mbps [24][25][26] |
| EVC-Baseline | -30 % vs AVC [27][29] | n/a | Not expected for 4k or 8k |
| EVC-Main | -24 % vs HEVC [27][29] | n/a | Expected [27][29] 4k CBR: 15-19Mbps 8k CBR: 30-60 Mbps |
| VVC | -30 % vs HEVC [28][29] Best-CfP: -42 % vs HEVC Target: -50 % vs HEVC | n/a | Expected [28][29] 4k CBR: 10-15 Mbps 8k CBR: 25-35 Mbps |
| Note: (Average) with peaks up to 25Mbps thanks to STATMUX. |  |  |  |

Also noteworthy is the improvement of encoders over time even for existing standards which also leads to bitrate reductions at the same quality.

Based on this information it can be expected that within the time frame until 2025, video compression technology permit bitrate reductions by a factor of 50 % compared to what is today possible with HEVC [15].

However, not to forget according to Jevons Paradox, stating that the efficiency with which a resource is used tends to increase (rather than decrease) the rate of consumption of that resource. This may well mean that the compression efficiency gains spur even more traffic.

Additional updates to video chararacteristics and bitrates are available in 3GPP TR 26.955 [46].