3GPP SA4 #124

Berlin, Germany, 22nd – 26th of May 2023	S4-231026

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.522 | CR | pseudo | rev | - | Current version: | 0.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5G_RTP] pCR to TS 26.522 on PDU set HE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_RTP |  |  |  |  |  | Date: |  |  | 17/05/2023 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Adds definition of PDU set, clarifies SDP signalling and adds appropriate notes on the guidelines to be defined for PDU set HE. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 4.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## Change#1

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Data Burst: A data burst is a set of multiple PDUs generated and sent by the application such that there is an idle period between two data bursts. A Data Burst can be composed of one or multiple PDU Sets.

PDU Set: One or more PDUs carrying the payload of one unit of information generated at the application level (e.g. frame(s), video slice(s), metadata, etc.).

## Change#2

### 4.4.2	RTP Header Extension for PDU Set Marking

#### 4.4.2.1	General

The RTP Header Extension for PDU Set marking can be used by an AS (e.g., MRF) or a sender UE that sends media to a receiver UE over RTP.

The RTP Header Extension for PDU Set marking shall support both RTP Header Extension formats (i.e., the one-byte and the two-byte formats) according to RFC 8285.

If the RTP Header Extension for PDU Set marking is the only RTP header extension used, the endpoints shall use the 1-byte header format for maximum savings. If other 2-byte RTP header extension elements are used, then the 2-byte header may be used.

NOTE:	The headers are not shown with padding as this depends on other prospective extension elements in use, as per RFC 8285 alignment specifications.

#### 4.4.2.2	One-byte RTP Header Extension Format

The one-byte RTP Header Extension for the marking of PDU Sets and End of Bursts is defined as follows:

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0xBE    |    0xDE       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   | L=5   |E| EDB |  PSI  |      PSSN         |     PSN   |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                     PSSize                    |

+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+

#### 4.4.2.3	Two-byte RTP Header Extension Format

The two-byte RTP Header Extension for the marking of PDU Sets and End of Bursts is defined as follows:

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0x100         | appbits |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      ID       |      L=6      |E| EDB |  PSI  |      PSSN

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|    PSN    |                   PSSize                      |

+-+-+-+-+-+-+-+-+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+

#### 4.4.2.4	Semantics

The semantics of the fields of the RTP Header Extension for the marking of PDU Set and End of Bursts are defined as follows:

-	End PDU of the PDU Set [E] (1 bit): This field is a flag that shall be set to 1 for the last PDU of the PDU Set and set to 0 for all other PDUs of the PDU Set.

-	End of Data Burst [EDB] (3 bits): The EDB field is 3 bits in length and indicates the end of a Data Burst. The 3 bits encode the End of Data Burst indication as per the encoding and guidelines provided in Clause 4.4.2.6.1.

-	PDU Set Importance [PSI] (4 bits): The PDU Set Importance field indicates the importance of this PDU Set compared to other PDU Sets within the same QoS flow. Lower values shall indicate a higher importance PDU Set with the highest importance PDU Set indicated by 0 and the lowest importance PDU Set indicated by 15.

NOTE 1:	A complete set of guidelines for setting the PSI field for various audio/video codecs are provided in Clause 4.4.2.6.2

Editor’s Note: AS/UE are unaware of QoS flows and so the above text needs to be revised to remove the term. The PSI value needs to be set considering one or more RTP streams (depending on multiplexing and other aspects that are under discussion in reference to the guidelines).  Once the guidelines are finalized, the text above will be revised possibly with the introduction of a new term in place for QoS flow that consists of one or more RTP streams.

-	PDU Set Sequence Number [PSSN] (10 bits): The field encodes the sequence number of the PDU Set to which the current PDU belongs acting as a 10-bit numerical identifier for the PDU Set.

NOTE 2:	This value wraps around at 1023, however, using the RTP packet sequence number and PSSN pair a receiver may uniquely distinguish between any PDU Sets.

-	PDU Sequence Number within a PDU Set [PSN] (6 bits): The sequence number of the current PDU within the PDU Set. The PSN shall be set to 0 for the first PDU in the PDU Set and incremented monotonically for every PDU in the PDU set in order of transmission from the sender.

NOTE 3:	A receiver may use the RTP packet sequence number together with the PSN to distinguish between PDUs within a PDU Set that contains more than 64 PDUs.

-	PDU Set Size [PSSize] (24 bits): The PDU Set Size indicates the total size of all PDUs of the PDU Set to which this PDU belongs. This field is optional and subject to an SDP signaling offer/answer negotiation, where the Application Server may indicate whether it will be able to provide the size of the PDU Set for that RTP stream. If not enabled, the field should not be present. If enabled, but the Application Server is not able to determine the PDU Size for a particular PDU Set, it should set the value to 0 in all PDUs of that PDU Set. The PSSize shall indicate the size of [a PDU Set including RTP/UDP/IP header encapsulation overhead of its corresponding PDUs] / [sum of RTP payload sizes of all PDUs present in a PDU Set]. The PSSize is expressed in bytes.

NOTE 4:	This field may be optionally present given the signaling of the “pdu-set-size” extension attribute in the SDP offer/answer negotiation as per Clause 3.4.

NOTE 5:	Guidelines to set the PDU Set Size in bytes by an Application Server are provided in Clause 4.4.2.6.3.

#### 4.4.2.5	SDP Signaling

An AS or sender UE capable of sending PDU set marking HE shall use the SDP attribute extmap for PDU set marking HE in the media description of the RTP stream(s) carrying the PDU set HE. A receiver that does not support PDU set marking HE can ignore the RTP header when included. General rules of usage of one-byte and two-byte header apply as defined in RFC 8285.

The semantics of the fields of the RTP Header Extension for the marking of PDU Set and End of Bursts are defined as follows:

The URN for the PDU Set marking shall be set to "urn:3gpp:pdu-set-marking:rel-18". 3GPP should register this header extension identifier with IANA as maintained in Real-Time Transport Protocol (RTP) Parameters (iana.org).

The ABNF syntax for the extmap attribute for the signaling of PDU Set Information and End of Burst marking is defined as follows:

extmap-attr="a=extmap:" 1*5DIGIT ["/" direction] SP "urn:3gpp:pdu-set-marking:rel-18" SP extensionattributes

extensionattributes = *3(format / "pdu-set-size")

format = "short" / "long"

#### 4.4.2.6	Detailed Guidelines for PDU Set Marking Semantics

##### 4.4.2.6.1	End of Data Burst Field

NOTE: These detailed guidelines are FFS.

##### 4.4.2.6.2	PDU Set Importance Field

NOTE: These detailed guidelines are FFS.

NOTE: The following aspects need to be further defined:

- default value for importance when the sender cannot define importance

- Codec level aspect:

  - video: importance when PDU set is i) slice, ii) frame iii) parameter sets iv) tile set v) other?

  - audio: when and if to use PDU set marking HE in an audio frame.

  - Text/metadata: when and if to use PDU set marking HE in text/metadata

  - Image: A frame is a pdu set and the importance for all frames is i) same ii) set based on application aspects.

- Importance across bitstreams

  - Multiplexed streams: importance marking when a 5-tuple corresponds to more than one bitstream

  - Importance marking considerations for non-multiplexed bitstreams

##### 4.4.2.6.3	PDU Set Size Field

NOTE: These detailed guidelines are FFS.

##### 4.4.2.6.3	Guidelines for AS

This clause describes guidelines for an AS that is on the media path between two or more UEs, e.g., an MRF, MCU etc. Such an AS may receive media over RTP with PDU set marking HE added by the sender UE.

NOTE: These detailed guidelines are FFS.

## #End of changes