Title:		          Correction in TR 26.933 v0.2.0 (S4-231496)

Source:                       Panasonic Holdings Corporation

Document for:           Agreement

Agenda item:             7.8

# Introduction

This document proposes a correction in clause 6.1.4.3.3 of TR 26.933 v0.2.0 (S4-231496)

# Proposed Correction

Following change is proposed.

First change

6.1.4.3.3  M/S

M/S (mid-side) stereo microphone using a microphone (usually a cardioid) pointed forward, and a bidirectional (figure-eight) microphone oriented perpendicular to the directional microphone. The figure-eight microphone captures side signal, and the cardioid microphone capture mid signal. Therefore, we can obtain the left and right channel signal through the simple addition and subtraction.

Left=mid+side

Right=mid-side

In addition, controlling the ratio of the two signals, different angles can be obtained.

Figure  6.1.3.3-1  The configuration of M/S stereo microphone

End of changes

# Conclusion

Source would like to request to review the proposed correction and update the clause 6.1.4.3.3 accordingly.

# Reference

S4-231496, TR 26.933 v0.2.0, SA4#125, Goteborg, Sweden, 21-25 August 2023.